3GPP TSG-RAN WG3 Meeting #119	R3-230360

# Athens, Greece, 27th February - 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0980 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Burst Arrival Time semantics description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Burst Arrival Time IE reuses the same encoding as the RRC ReferenceTime IE, which is a sequence of four INTEGER values having decreasing time granularity as shown below: ReferenceTime-r16 ::= SEQUENCE { refDays-r16 INTEGER (0..72999), refSeconds-r16 INTEGER (0..86399), refMilliSeconds-r16 INTEGER (0..999), refTenNanoSeconds-r16 INTEGER (0..99999) } It is unclear what “truncated to 1 us granularity” means in the context of the above encoding structure (since e.g. the value of the IE cannot be shortened, nor can the 10ns granularity of the IE be changed). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The semantics description is reworded to: “provided with 1us accuracy”. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol point of view. The impact can be considered isolated because the change affects only the Burst Arrival Time IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The semantics of the Burst Arrival Time IE remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.117 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.117	Burst Arrival Time

This IE indicates the Burst Arrival Time of the TSC QoS flow as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time | M |  | OCTET STRING | Encoded in the same format as the ReferenceTime IE as defined in TS 38.331 [10]. The value is provided with 1 us accuracy. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>