3GPP TSG-RAN WG3 Meeting #119	R3-230407

Athens, GR, 27 Feb – 03 Mar, 2023

Title:	[Draft] Reply LS on time synchronization status reporting to UE(s)

Response to:	S2-2301463/R3-230037

Release:	Rel-18

Work Item:	FS_5TRS_URLLC

Source:	Huawei [will be RAN3]

To:	SA2

Cc:	RAN2

Contact person:	Feng Han

hanfeng3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 would like to thank SA2 for their LS on Proposed method for Time Synchronization Status reporting to UE(s).

Regarding the questions raised by SA2, RAN3 provides the feedback as follows:

SA2 question: SA2 would like to kindly request RAN2 and RAN3 to provide feedback whether both scopes (group of cells per gNB, group of cells across gNBs) can be beneficial and supported.

RAN3 Answer: The scope of the reference report ID shall be group of cells per gNB, while the scope of cells across gNBs is not expected.

SA2 question: SA2 would like to kindly request RAN3 to provide feedback whether the following attributes are available in RAN: time source, traceability to UTC or GNSS, synchronization state, clock accuracy, clock frequency stability, PTP clockClass.

RAN3 Answer: For the inquired attributes, RAN3 considers the clock frequency stability attribute is not clear. It would be available at the NG-RAN node if it means the offsetScaledLogVariance as defined in TS 23.501. And in case the PTP clockClass is deemed necessary for TSS, it can be mapped from SyncE or GNSS time source by NG-RAN’s implementation. The rest attributes are available in RAN.

Besides, RAN3 understands that the detailed clock attributes are vendor-specific, i.e. the clock information listed above should be optional if finally specified in the specification.

# 2	Actions

To SA2

ACTION: 	RAN3 respectfully asks SA2 to take the above information into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#119bis-e		2023-04-17 - 2023-04-26		e-meeting

RAN3#120		2023-05-22 - 2023-05-26		Incheon, KR