3GPP TSG-RAN WG3 Meeting #119	R3-230417

Athens, GR, 27 Feb – 03 Mar, 2023

Title: 	Correction of RRC references

Source: 	Huawei

Agenda item:	9.3

Document Type:	discussion

# 1. Introduction

In RAN3#118 it was agreed that the rapporteurs would provide Rel-17 CRs to correct the use of references to the RRC specification. In this paper, the changes proposed for F1AP are discussed.

# 2. Background

The guidelines were agreed in R3-226777 in R3#118. After that meeting the, the rapporteurs were tasked to update the specs accordingly.

# 3. Discussion

In the following section, we outline proposed changes to F1AP. The changes are split into two sections:

- one for changes requiring dedicated CRs, where the changes are not just to align to the guidelines but where actual errors require dedicated CRs for traceability, and where traceability is needed.

- one part with non-controversial changes to reflect the new guide lines

One important thing to note is that since the general level of detail for the semantics descriptions in F1AP was quite low (compared to e.g. Xn) and the proposals in this paper therefore deviate slightly from the guidelines to avoid introducing unintentional consequences during this correction. More specifically, instead of using “Corresponds to information provided in...” the more general sentence “Corresponds to ...” was used

## 3.1 Proposed changes in dedicated CRs

### 3.1.1	Changes related to SL-relay

9.3.1.263 Path Switch Configuration

Corresponding to SL- targetRelayUEIdentity-r17, defined in TS 38.331 [8]

This reference does not exist. The closest one is the targetRelayUE-Identity field in CellGroupConfig IE. The suffix is not needed to provide uniqueness. Proposed correction:

- Corresponds to the targetRelayUE-Identity contained in the CellGroupConfig IE defined in TS 38.331 [8]

9.3.1.26 DU to CU RRC Information

SL-PHY-MAC-RLC-Config as defined in TS 38.331 [8].

This points to a type definition with a suffix (r16) where a type with different suffix exist (v17) and where both are sent in parallel within the SL-ConfigDedicatedNR IE. The proposal is instead to refer to the specific type, i.e. add a suffix:

- Includes the SL-PHY-MAC-RLC-Config-r16 IE, as defined in TS 38.331 [8].

### 3.1.2	DRX Cycle

9.3.1.24 DRX Cycle

This IE is defined in TS 38.331 [8]

Three different references in this section. The current reference is not clearly pointing to a specific IE but it seems appropriate to refer to the following fields in the UEAssistanceInformation message:

- Corresponds to the preferredDRX-LongCycle contained in the UEAssistanceInformation message defined in TS 38.331 [8]

- Corresponds to the preferredDRX-ShortCycle contained in the UEAssistanceInformation message defined in TS 38.331 [8]

- Corresponds to the preferredDRX-ShortCycleTimer contained in the UEAssistanceInformation message defined in TS 38.331 [8]

### 3.1.3	Burst Arrival Time

The Burst Arrival Time IE reuses the same encoding as the RRC ReferenceTime IE, which is a sequence of four INTEGER values having decreasing time granularity as shown below:

ReferenceTime-r16 ::=           SEQUENCE {

refDays-r16                         INTEGER (0..72999),

refSeconds-r16                      INTEGER (0..86399),

refMilliSeconds-r16                 INTEGER (0..999),

refTenNanoSeconds-r16               INTEGER (0..99999)

}

It is unclear what “truncated to 1 us granularity” means in the context of the above encoding structure (since e.g. the value of the IE cannot be shortened, nor can the 10ns granularity of the IE be changed). Hence it is proposed to revise the semantics to “provided with 1us accuracy”.

## 3.2 Proposed changes in the common NR CR

This includes changes that unlike the above is more of an editorial nature to align with the agreed principles.

### 3.2.1	References to NR RRC

#### 3.2.1.2	Changes were the suffix is removed

9.2.9.3 GNB-DU RESOURCE CONFIGURATION (2 places)

Corresponds to the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [8].

This reference is a field in the BWP-UplinkCommon IE in 38.331. The suffix is not needed to provide uniqueness and can be removed. New preamble applied:

- Includes the IAB-specific rach-ConfigCommonIAB contained in the BWP-UplinkCommon IE as defined in subclause 6.3.2 of TS 38.331 [8].

9.2.10.1 ACCESS AND MOBILITY INDICATION

nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8].

This reference is a field in the RLF-Report field inside the UEInformationResponse message in 38.331. The suffix is not needed to provide uniqueness and can be removed.

- Includes the nr-RLF-Report contained in the UEInformationResponse message defined in TS 38.331 [8].

9.2.10.1 ACCESS AND MOBILITY INDICATION

RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8].

This reference is a type definition in the UEInformationResponse message in 38.331. The suffix is not needed to provide uniqueness and can be removed. New preamble applied:

- Includes the RA-ReportList IE as defined in subclause 6.2.2 in TS 38.331 [8].

9.2.10.1 ACCESS AND MOBILITY INDICATION

SuccessHO-Report-r17 IE as defined in subclause 6.2.2 in TS 38.331 [8].

This reference is a type definition in the UEInformationResponse message in 38.331. The suffix is not needed to provide uniqueness and can be removed. New preamble and removal of suffix:

- Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8].

9.2.12.14 POSITIONING INFORMATION RESPONSE

SRS-PosRRC-InactiveConfig-r17 as defined in TS 38.331 [8]

The current reference is a type definition inside RRCRelease message. The suffix is not needed to provide uniqueness and can be removed The proposal is to keep the current reference, remove the suffix and add preamble:

- Includes the SRS-PosRRC-InactiveConfig IE as defined in TS 38.331 [8]

9.2.12.31 MEASUREMENT PRECONFIGURATION CONFIRM

PosMeasGapPreConfigToAddModList-r17 as defined in TS 38.331 [8]

This reference is a type definition in the MeasGapConfig IE in 38.331. The suffix is not needed to provide uniqueness and can be removed. The proposal is to removes suffix and add preamble

- Includes the PosMeasGapPreConfigToAddModList IE as defined in TS 38.331 [8]

9.2.12.31 MEASUREMENT PRECONFIGURATION CONFIRM

PosMeasGapPreConfigToReleaseList-r17 as defined in TS 38.331 [8]

This reference is a type definition in the MeasGapConfig IE in 38.331. The suffix is not needed to provide uniqueness and can be removed. The proposal is to remove suffix and add preamble:

- Includes the PosMeasGapPreConfigToReleaseList IE as defined in TS 38.331 [8]

9.3.1.26 DU to CU RRC Information

ul-GapFR2-Config-r17, as specified in TS 38.331 [8].

This points to the field used in RRCReconfiguration message. The suffix is not needed to provide uniqueness and can be removed. Proposed update is as follows:

- Includes the ul-GapFR2-Config contained in the RRCReconfiguration message as specified in TS 38.331 [8].

9.3.1.111 BAP Address

Corresponds to the bap-Address-r16, defined in subclause 6.2.2 or subclause 6.3.2 of TS 38.331 [8], or the iab-donor-DU-BAP-Address-r16 defined in subclause 6.2.2 of TS 38.331[8].

These points to a field in either RRCReconfiguration message (6.2.2) or BAP-RoutingID IE (6.3.2) or CellGroupConfig IE (6.3.2). The suffix is not needed for uniqueness. Proposal is to refer to the individual IEs/messages instead of sections number and removing the suffix .

- Corresponds to the bap-Address contained in the RRCReconfiguration message or contained in the BAP-RoutingID IE or contained in the CellGroupConfig IE, or the iab-donor-DU-BAP-Address contained in the RRCReconfiguration message defined in TS 38.331[8].

9.3.1.112 BAP Path ID

Corresponds to the bap-Pathid-r16 defined in subclause 6.3.2 of TS 38.331 [8].

This points to a field in BAP-RoutingID IE. The suffix is not needed for uniqueness. Proposal is to removing the suffix and align the rest:

- Corresponds to the bap-Pathid contained in the BAP-RoutingID IE defined in TS 38.331 [8].

9.3.1.263 Path Switch Configuration

Corresponding to T420-r17, defined in TS 38.331 [8]

This points to a field in the CellGroupConfig IE. The suffix is not required for uniqueness. Proposed adjustment:

- Corresponds to the t420 contained in the CellGroupConfig IE defined in TS 38.331 [8]

#### 3.2.1.3	Editorial changes to reflect the guidelines

9.2.2.1 UE CONTEXT SETUP REQUEST (2 places), and 9.3.1.277 SDT Bearer Configuration Info

RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8]

This reference is an IE in 38.331. New preamble/formatting applied:

- Includes the RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8]

9.2.2.1 UE CONTEXT SETUP REQUEST, 9.2.2.7 UE CONTEXT MODIFICATION REQUEST, and 9.2.2.11 UE CONTEXT MODIFICATION CONFIRM

Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU.

This reference is a message and not an IE in 38.331. Proposed correction:

- Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU.

9.2.2.5 UE CONTEXT RELEASE COMMAND and 9.2.3.2 DL RRC MESSAGE TRANSFER

Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8].

This reference is a message and not an IE in 38.331. Proposed correction (2 places):

- Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8].

9.2.3.1 INITIAL UL RRC MESSAGE TRANSFER

Includes the UL-CCCH-Message IE or UL-CCCH1-Message IE as defined in subclause 6.2 of TS 38.331 [8].

This reference is a message and not an IE in 38.331. Proposed correction:

- Includes the UL-CCCH-Message message or UL-CCCH1-Message message as defined in subclause 6.2 of TS 38.331 [8].

9.2.3.1 INITIAL UL RRC MESSAGE TRANSFER

CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331 [8].

This reference is an IE in 38.331. New preamble applied:

- Includes the CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331 [8].

9.2.3.1 INITIAL UL RRC MESSAGE TRANSFER

Includes the UL-DCCH-Message IE including the RRCSetupComplete message, as defined in subclause 6.2 of TS 38.331 [8].

This reference is a message and not an IE in 38.331. Proposed correction:

- Includes the UL-DCCH-Message message including the RRCSetupComplete message, as defined in subclause 6.2 of TS 38.331 [8].

9.2.3.2 DL RRC MESSAGE TRANSFER

Includes the UL-CCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8].

This reference is a message and not an IE in 38.331. Proposed correction:

- Includes the UL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8].

9.2.3.3 UL RRC MESSAGE TRANSFER

Includes the UL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. In case of CG-SDT, may include the UL-CCCH-Message IE or UL-CCCH1-Message IE as defined in subclause 6.2 of TS 38.331 [8].

This reference is a message and not an IE in 38.331. Proposed correction:

- Includes the UL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. In case of CG-SDT, may include the UL-CCCH-Message message or UL-CCCH1-Message message as defined in subclause 6.2 of TS 38.331 [8].

9.2.9.3 GNB-DU RESOURCE CONFIGURATION (2 places)

Corresponds to the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [8].

This current reference is a field in BWP-UplinkCommon IE. The field name is kept and a reference to the IE where it occurs is added and the new preamble/formatting is applied:

- Includes the rach-ConfigCommon contained in the BWP-UplinkCommon IE as defined in subclause 6.3.2 of TS 38.331 [8].

9.2.9.3 GNB-DU RESOURCE CONFIGURATION (2 places)

Corresponds to the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8].

This reference is an IE in 38.331. New preamble/formatting applied:

- Includes the NZP-CSI-RS-Resource IE as defined in subclause 6.3.2 of TS 38.331 [8].

9.2.9.3 GNB-DU RESOURCE CONFIGURATION (2 places)

Corresponds to the SchedulingRequestResourceConfig as defined in subclause 6.3.2 of TS 38.331 [8].

This reference is an IE in 38.331. New preamble/formatting applied:

- Includes the SchedulingRequestResourceConfig IE as defined in subclause 6.3.2 of TS 38.331 [8].

9.2.9.3 GNB-DU RESOURCE CONFIGURATION (2 places)

Corresponds to the PDCCH-ConfigSIB1 as defined in subclause 6.3.2 of TS 38.331 [8].

This reference is an IE in 38.331. New preamble/formatting applied:

- Includes the PDCCH-ConfigSIB1 IE as defined in subclause 6.3.2 of TS 38.331 [8].

9.2.9.3 GNB-DU RESOURCE CONFIGURATION (2 places)

Corresponds to the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [8].

This reference is a field in the MIB message in 38.331. New preamble applied:

- Includes the subCarrierSpacingCommon contained in the MIB message as defined in subclause 6.2.2 of TS 38.331 [8].

9.3.1.10 Served Cell Information 9.3.1.216 NR Mode Info Rel16

The tdd-UL-DL-ConfigurationCommon as defined in TS 38.331 [8]

This reference is a field is a field contained in ServingCellConfigCommon IE. Proposed modification:add reference to the IE, add preamble.

- Includes the tdd-UL-DL-ConfigurationCommon contained in the ServingCellConfigCommon IE as defined in TS 38.331 [8]

9.3.1.10 Served Cell Information 9.3.1.25 CU to DU RRC Information

Contains the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8].

This reference is a message. The current text also specifies that this is an inter node message which is not specifically included in guidelines but no reason to change this. We could however align the preamble

- Includes the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8].

9.3.1.10 Served Cell Information

This IE corresponds to the PLMN-IdentityInfoList IE and the NPN-IdentityInfoList IE (if available) in SIB1 as specified in TS 38.331 [8].

This is not directly referring to the content about a specific IE and can therefore be kept as is

9.3.1.10 Served Cell Information

The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8].

This is not directly referring to the content about a specific IE but the formatting can be improved

- The presence of this IE indicates that the intraFreqReselectionRedCap is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8].

9.3.1.18 gNB-DU System Information

MIB message, as defined in subclause 6.2.2 in TS 38.331 [8].

Refers to the MIB message. Proposal to add preamble and format:

- Includes the MIB message, as defined in subclause 6.2.2 in TS 38.331 [8].

9.3.1.18 gNB-DU System Information

SIB1 message, as defined in subclause 6.2.2 in TS 38.331 [8].

Refers to the SIB1 message. Proposal to add preamble and format:

- Includes the SIB1 message, as defined in subclause 6.2.2 in TS 38.331 [8].

9.3.1.18 gNB-DU System Information

SIB12, as defined in subclause 6.3.1 in TS 38.331 [8].

Refers to the SIB12 IE. Proposal to add preamble, IE and format:

- Includes the SIB12 IE, as defined in subclause 6.3.1 in TS 38.331 [8].

9.3.1.18 gNB-DU System Information

SIB13, as defined in subclause 6.3.1 in TS 38.331 [8].

Refers to the SIB13 IE. Proposal to add preamble, IE and format:

- Includes the SIB13 IE, as defined in subclause 6.3.1 in TS 38.331 [8].

9.3.1.18 gNB-DU System Information

SIB14, as defined in subclause 6.3.1 in TS 38.331 [8].

Refers to the SIB13 IE. Proposal to add preamble, IE and format:

- Includes the SIB13 IE, as defined in subclause 6.3.1 in TS 38.331 [8].

9.3.1.18 gNB-DU System Information

SIB10, as defined in subclause 6.3.1 in TS 38.331 [8].

Refers to the SIB10 IE. Proposal to add preamble, IE and format:

- Includes the SIB10 IE, as defined in subclause 6.3.1 in TS 38.331 [8].

9.3.1.18 gNB-DU System Information

SIB17, as defined in subclause 6.3.1 in TS 38.331 [8]

Refers to the SIB17 IE. Proposal to add preamble, IE and format:

- Includes the SIB17 IE, as defined in subclause 6.3.1 in TS 38.331 [8].

9.3.1.18 gNB-DU System Information

SIB20, as defined in subclause 6.3.1 in TS 38.331 [8]

Refers to the SIB20 IE. Proposal to add preamble, IE and format:

- Includes the SIB20 IE, as defined in subclause 6.3.1 in TS 38.331 [8].

9.3.1.18 gNB-DU System Information

SIB15, as defined in subclause 6.3.1 in TS 38.331 [8].

Refers to the SIB15 IE. Proposal to add preamble, IE and format:

- Includes the SIB15 IE, as defined in subclause 6.3.1 in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

CG-ConfigInfo, as defined in TS 38.331 [8].

Refers to a message in 38.331. Proposal is to add preamble, message and formatting:

- Includes the CG-ConfigInfo message, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

This IE is used in the NG-RAN and it consists of the UE-CapabilityRAT-ContainerList, as defined in TS 38.331 [8].

The reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- This IE is used in the NG-RAN and it includes the UE-CapabilityRAT-ContainerList IE, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

MeasConfig, as defined in TS 38.331 [8] (without MeasGapConfig).

The reference is pointing to an IE. The end of the text also refers to another IE. Proposal is to add preamble, IE and formatting:

- Includes the MeasConfig IE, as defined in TS 38.331 [8] (without the MeasGapConfig IE).

9.3.1.25 CU to DU RRC Information

HandoverPreparationInformation, as defined in TS 38.331 [8].

The reference is pointing to a message. Proposal is to add preamble, message and formatting:

- Includes the HandoverPreparationInformation message, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information, and 9.3.1.26 DU to CU RRC Information

CellGroupConfig, as defined in TS 38.331 [8].

The reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- Includes the CellGroupConfig IE, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

UEAssistanceInformation, as defined in TS 38.331 [8].

The reference is pointing to a message. Proposal is to add preamble, message and formatting:

- Includes the UEAssistanceInformation message, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

CG-Config, as defined in TS 38.331 [8].

The reference is pointing to a message. Proposal is to add preamble, message and formatting:

- Includes the CG-Config message, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

LocationMeasurementInfo, as defined in TS 38.331[8]

The reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- Includes the LocationMeasurementInfo IE, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

MUSIM-GapConfig as defined in TS 38.331 [8].

The reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- Includes the MUSIM-GapConfig IE, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information, and 9.3.1.26 DU to CU RRC Information

SDT-MAC-PHY-CG-Config, as defined in TS 38.331 [8].

The current reference is is a type definition inside the RRCRelease message. The proposal is to add preamble:

- Includes the SDT-MAC-PHY-CG-Config IE as defined in TS 38.331 [8]

9.3.1.25 CU to DU RRC Information

MBSInterestIndication as defined in TS 38.331 [8].

The reference is pointing to a message. Proposal is to add preamble, message and formatting:

- Includes the MBSInterestIndication message, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

NeedForGapsInfoNR, as defined in TS 38.331 [8].

The reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- Includes the NeedForGapsInfoNR IE, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

NeedForGapNCSG-InfoNR, as defined in TS 38.331 [8].

The reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- Includes the NeedForGapNCSG-InfoNR IE, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

NeedForGapNCSG-InfoEUTRA, as defined in TS 38.331 [8].

The reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- Includes the NeedForGapNCSG-InfoEUTRA IE, as defined in TS 38.331 [8].

9.3.1.25 CU to DU RRC Information

ConfigRestrictInfoDAPS-r16 as defined in TS 38.331 [8]. This IE is used at the source node if DAPS HO is configured.

This refers to a type definition. There exist another name with a different suffix. It is therefore proposed keep the suffix and add a reference to the message name:

- Includes the ConfigRestrictInfoDAPS-r16 IE as defined in TS 38.331 [8]. This IE is used at the source node if DAPS HO is configured.

9.3.1.26 DU to CU RRC Information

MeasGapConfig as defined in TS 38.331 [8].

The reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- Includes the MeasGapConfig IE, as defined in TS 38.331 [8].

9.3.1.26 DU to CU RRC Information

requestedP-MaxFR1, as defined in TS 38.331 [8].

This reference is pointing to a field in the CG-Config message. Proposal is to include reference to the message, add preamble and formatting.

- Includes the requestedP-MaxFR1 contained in the CG-Config message, as defined in TS 38.331 [8].

9.3.1.26 DU to CU RRC Information

Identical to the value of the drx-LongCycleStartOffset IE within the DRX-Config as defined in TS 38.331 [8].

This is referring a field in the DRX-Config IE. Proposal is to align the preamble, reference and the formatting:

- Corresponds to the drx-LongCycleStartOffset contained in the DRX-Config IE as defined in TS 38.331 [8].

9.3.1.26 DU to CU RRC Information (2 places)

BandCombinationIndex, as defined in TS 38.331 [8].

This is referred in two different ways in F1AP:

- BandCombinationIndex, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination; if this IE is included, the gNB-CU uses this information to deduce the selected band.

- BandCombinationIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Band Combination.

This reference is pointing to a type definition in CG-Config message. The proposal is to change add preamble and formatting:

- Includes the BandCombinationIndex IE as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination; if this IE is included, the gNB-CU uses this information to deduce the selected band.

- Includes the BandCombinationIndex IE, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Band Combination.

9.3.1.26 DU to CU RRC Information (2 places)

FeatureSetEntryIndex, as defined in TS 38.331 [8].

This is referred in two different ways in F1AP:

- FeatureSetEntryIndex, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected FeatureSet.

- FeatureSetEntryIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Feature Set.

This reference is pointing to a type definition in CG-Config message. The proposal is to change add preamble and formatting:

- Includes the FeatureSetEntryIndex IE, as defined in TS 38.331 [8] For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected FeatureSet.

- Includes the FeatureSetEntryIndex IE, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Feature Set.

9.3.1.26 DU to CU RRC Information

PH-TypeListSCG, as defined in TS 38.331 [8].For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in SN.

This reference is pointing to a type definition in CG-Config message. The proposed solution is to add preamble:

- Includes the PH-TypeListSCG IE, as defined in TS 38.331 [8]. For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in SN.

9.3.1.26 DU to CU RRC Information

DRX-Config, as defined in TS 38.331 [8].

This reference is pointing to an IE. Proposal is to add preamble, IE and formatting:

- Includes the DRX-Config IE, as defined in TS 38.331 [8].

9.3.1.26 DU to CU RRC Information

pdcch-BlindDetectionSCG, as defined in TS 38.331 [8]. This IE is used between the MgNB-DU and the MgNB-CU.

This reference points to an existing field in the CG-ConfigInfo message. Proposal is to add preamble and reference to the message:

- Includes the pdcch-BlindDetectionSCG contained in the CG-ConfigInfo message, as defined in TS 38.331 [8]. This IE is used between the MgNB-DU and the MgNB-CU.

9.3.1.26 DU to CU RRC Information

requestedPDCCH-BlindDetectionSCG, as defined in TS 38.331 [8]. This IE is used between the SgNB-DU and the SgNB-CU.

This reference points to an existing field in the CG-Config message. Proposal is to add preamble and reference to the message:

- Includes the requestedPDCCH-BlindDetectionSCG contained in the CG-Config message, as defined in TS 38.331 [8]. This IE is used between the SgNB-DU and the SgNB-CU.

9.3.1.26 DU to CU RRC Information

PH-TypeListMCG, as defined in TS 38.331 [8]. For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in MCG.

This reference is pointing to a type definition in CG-ConfigInfo message. The proposed solution is to add preamble and formatting:

- Includes the PH-TypeListMCG IE, as defined in TS 38.331 [8]. For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in MCG.

9.3.1.26 DU to CU RRC Information

MeasGapSharingConfig as defined in TS 38.331 [8].

The reference point to an IE. Proposal is to add preamble, IE and formatting:

- Includes the MeasGapSharingConfig IE as defined in TS 38.331 [8].

9.3.1.26 DU to CU RRC Information

SL-ConfigDedicatedEUTRA-Info as defined in TS 38.331 [8].

This points to a type definition in RRCReconfiguration message. Proposal is to add preamble, IE and formatting:

- Includes the SL-ConfigDedicatedEUTRA-Info IE, as defined in TS 38.331 [8].

9.3.1.26 DU to CU RRC Information

RequestedP-MaxFR2, as defined in TS 38.331 [8].

This point to a field in the CG-Config message (although written here with an initial capital letter). The proposal is to add preamble, add message and formatting:

- Includes the requestedP-MaxFR2 contained in the CG-Config message, as defined in TS 38.331 [8].

9.3.1.26 DU to CU RRC Information

MUSIM-GapConfig as defined in TS 38.331 [8].

This points to an existing IE. Proposal is to add preamble, IE and formatting:

- Includes the MUSIM-GapConfig IE as defined in TS 38.331 [8].

9.3.1.26 DU to CU RRC Information

sl-RLC-ChannelToAddModList-r17, as defined in TS 38.331 [8]

This point to an existing field in SL-ConfigDedicatedNR IE. Proposal is to add preamble, IE and formatting:

- Includes the sl-RLC-ChannelToAddModList-r17 contained in the SL-ConfigDedicatedNR IE, as defined in TS 38.331 [8]

9.3.1.26 DU to CU RRC Information

Identical to the value of the interFrequencyConfig-NoGap-r16 IE, as defined in TS 38.331 [8].

This reference a field in the MeasConfig IE. The suffix id included in the description of the field so the suffix can be kept in RAN3 spec. Proposal is to add IE and formatting:

- Corresponds to the interFrequencyConfig-NoGap-r16 IE contained in the MeasConfig IE, as defined in TS 38.331 [8].

9.3.1.67 Uplink TxDirectCurrentList Information

UplinkTxDirectCurrentList as defined in TS 38.331 [8].

This points to an IE. Proposal is to add preamble and IE:

- Includes the UplinkTxDirectCurrentList IE as defined in TS 38.331 [8].

9.3.1.120 SL DRB ID

Corresponds to the SLRB-Uu-ConfigIndex defined in TS 38.331 [8].

This points to an IE. Proposal is to do some minor adjustments:

- Corresponds to the SLRB-Uu-ConfigIndex IE defined in TS 38.331 [8].

9.3.1.121 PC5 QoS Flow Identifier

Corresponds to the SL-QoS-FlowIdentity defined in TS 38.331 [8].

This points to an IE. Proposal is to do some minor adjustments:

- Corresponds to the SL-QoS-FlowIdentity IE defined in TS 38.331 [8].

9.3.1.225 MBS CU to DU RRC Information

mtch-neighbourCell, as defined in TS 38.331[8]

This points to a field in the MBS-SessionInfoList IE. Proposed update:

- Includes the mtch-neighbourCell contained in the MBS-SessionInfoList IE, as defined in TS 38.331[8]

9.3.1.225 MBS CU to DU RRC Information

MRB-PDCP-ConfigBroadcast, as defined in TS 38.331[8]

This points to a type definition defined in MBS-SessionInfoList IE. The suffix is not needed for uniqueness. Proposed update:

- Includes the MRB-PDCP-ConfigBroadcast IE, as defined in TS 38.331[8]

9.3.1.226 MBS Broadcast Neighbour Cell List

MBS-NeighbourCellList, as defined in TS 38.331[8]

This points to an IE. Proposed update:

- Includes the MBS-NeighbourCellList IE, as defined in TS 38.331[8]

9.3.1.267 Remote UE Local ID

Corresponds to the sl-LocalIdentity defined in TS 38.331 [8].

This points to a field in the SL-SRAP-Config IE. Proposed update:

- Corresponds to the sl-LocalIdentity contained in the SL-SRAP-Config IE defined in TS 38.331 [8]

9.3.1.270 UE Paging Capability

Corresponds to the inactiveStatePO-Determination IE defined in TS 38.331 [8].

This points to a field in the UERadioPagingInformation IE. Proposed update:

- Corresponds to the inactiveStatePO-Determination contained in the UERadioPagingInformation IE defined in TS 38.331 [8]

9.3.1.283 Uplink TxDirectCurrentTwoCarrierList Information

Includes the UplinkTxDirectCurrentTwoCarrierList IE as defined in TS 38.331 [8].

No update needed

### 3.2.1 References to LTE RRC

9.3.1.25 CU to DU RRC Information

UEAssistanceInformation, as defined in TS 36.331 [41].

This points to a message. Proposed update:

Includes the UEAssistanceInformation message, as defined in TS 36.331 [41].

# 4. Conclusion

- In this paper the suggested changes required to align references to RRC is presented. The actual changes are provided in associated CRs. A set of dedicated CRs are also included for changes that are not pure alignment to the guidelines.