3GPP TSG-RAN WG3 Meeting #118	R3-230512

# Online Meeting, 14th – 18th November 2022

Agenda Item:	24.2

Source:	Ericsson

Title: 	Introduction of Network Energy Saving

Document for:	Discussion and Approval

# 1	Background

The new Work Item NES has been approved, refer to [1].

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

Objective 2 and object 6 have explicit RAN3 objectives, which we should work on the stage 3 specification.

# 2	Discussion

## 2.1	Impact on XnAP TS 38.423

The below existing procedure could be used to implement inter-node information exchange on cell DTX/DRX:

### 8.4.2 NG-RAN node Configuration Update #### 8.4.2.1 General The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface. NOTE: Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified. The procedure uses non UE-associated signalling. #### 8.4.2.2 Successful Operation Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The cell DTRX information can be carried at cell level.

Proposal 1: RAN3 to discuss and agree that NG-RAN node1 uses “NG-RAN node configuration Update” procedure to update the cell DTRX information to NG-RAN node 2 over XnAP.

In [2], we have further discussed an enhancement on the inter-node coordination, that is the Cell DTRX can be coordinated among the neighbouring nodes:

The “Cell DTRX information Report” in the above is the NG-RAN node configuration Update, and the “Cell DTRX information coordination” is the NG-RAN node configuration Update Acknowledge.

Proposal 2: RAN3 to discuss and agree that to coordinate the cell DTRX information among the NG-RAN nodes over XnAP;

Proposal 3: RAN3 to discuss and agree that NG-RAN node x uses “NG-RAN node configuration Update Acknowledge” procedure to send the coordinated value to NG-RAN node 1 over XnAP.

There is an existing procedure to perform Cell Activation, see below:

### 8.4.3 Cell Activation #### 8.4.3.1 General The purpose of the Cell Activation procedure is to enable an NG-RAN node to request a neighbouring NG-RAN node to switch on one or more cells, previously reported as inactive due to energy saving. The procedure uses non UE-associated signalling. #### 8.4.3.2 Successful Operation Figure 8.4.3.2-1: Cell Activation, successful operation

The existing procedure can be used, the beam aspect can be added. Further in the response, the NG-RAN node2 could send feedback related to if the beam is activated, and if other beams are activated instead.

Proposal 4: It is proposed that RAN3 to discuss and agree to include beam activation information in the Cell Activation request, and include beam activation result, including if successful, if other beams are activated in the response over XnAP.

## 2.2	Impact on F1AP TS 38.473

There are two existing procedures over F1AP for gNB-CU and DU to update the configuration.

### 8.2.4 gNB-DU Configuration Update #### 8.2.4.1 General The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling. NOTE: Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified. #### 8.2.4.2 Successful Operation Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

Proposal 5: RAN3 to discuss and agree to include the Cell DTRX information in F1 Setup and gNB-DU over F1AP.

### 8.2.5 gNB-CU Configuration Update #### 8.2.5.1 General The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling. #### 8.2.5.2 Successful Operation Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

Proposal 6: RAN3 to discuss and agree to include the Beam Activation Information in gNB-CU Configuration Update and the feedback in the response message over F1AP.

There is an existing procedure to perform Paging in gNB-DU. See below:

### 8.7.1 Paging #### 8.7.1.1 General The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling. #### 8.7.1.2 Successful Operation Figure 8.7.1.2-1: Paging procedure. Successful operation.

A restricted area could be SSBs that the UE last used. This is particularly useful for stationary UEs. When gNB-CU pages UE.

Proposal 7: RAN3 to discuss and agree to include the last used SSB in the paging over F1AP.

# 3	Proposals

Proposal 1: RAN3 to discuss and agree that NG-RAN node1 uses “NG-RAN node configuration Update” procedure to update the cell DTRX information to NG-RAN node 2 over XnAP

Proposal 2: RAN3 to discuss and agree that to coordinate the cell DTRX information among the NG-RAN nodes over XnAP

Proposal 3: RAN3 to discuss and agree that NG-RAN node x uses “NG-RAN node configuration Update Acknowledge” procedure to send the coordinated value to NG-RAN node 1 over XnAP

Proposal 4: It is proposed that RAN3 to discuss and agree to include beam aspect in the Cell Activation request, and include beam activation result, including if successful, if other beams are activated in the response over XnAP

Proposal 5: RAN3 to discuss and agree to include the Cell DTRX information in F1 Setup and gNB-DU over F1AP.

Proposal 6: RAN3 to discuss and agree to include the Beam Activation Information in gNB-CU Configuration Update and the feedback in the response message over F1AP.

Proposal 7: RAN3 to discuss and agree to include the last used SSB in the paging over F1AP.

The F1AP CR is submitted to implement Proposal 5 to 7, [3].

# 4	Reference

[1]	RP-221443	Study on network energy savings for NR

[2]	R3-226457	Further discussion on SI Network Energy Saving, Ericsson

[3]	R3-230512	Introduction of Network Energy Saving, Ericsson