3GPP TSG-RAN WG3 Meeting #119	R3-230559

Athens, GR, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0948 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE_ID based subgrouping for UE power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specified in TS 38.304, when the UE determines the UE-ID based subgrouping ID, it shall use 15bit if eDRX is applied, or 13bit of 5G-S-TMSI, UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 For inactive UE supporting the UE-ID based subgrouping, currently the Core Network Assistance Information for RRC INACTIVE IE (for NGAP), RAN paging message (for XnAP) have the following UE identity index values: - 10-bit length UE Identity Index Value refering to TS 38.304 and TS 36.304; - 16-bit length Extended UE Identity Index Value referring to TS 36.304, introduced during the eMTC WI (see R3-210199). In order to support the UE-ID based subgrouping, the simplest way is to update the Extended UE Identity Index Value IE applicable to NR as well, i.e. referring to TS 38.304. While for F1AP, only the 10-bit length UE Identity Index value is included in the Paging message. A longer bit size is needed to calculate the UE-ID based subgrouping ID for inactive UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Extended UE Identity Index Value IE by referring to TS 38.304 to support the UE-ID based subgrouping. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated functional impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the Core Network Assistance Information for RRC INACTIVE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Paging with UE_ID based subgrouping is not supported for inactive UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3.52 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423 CR1987 TS38.473 CR1130 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

<Below is provided just for reference >

#### 9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| UE Specific DRX | O |  | Paging DRX 9.3.1.90 |  | - |  |
| Periodic Registration Update Timer | M |  | 9.3.3.24 |  | - |  |
| MICO Mode Indication | O |  | 9.3.1.23 |  | - |  |
| TAI List for RRC Inactive |  | 1 |  |  | - |  |
| >TAI List for RRC Inactive Item |  | 1..<maxnoofTAIforInactive> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Expected UE Behaviour | O |  | 9.3.1.93 |  | - |  |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.3.52 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| MICO All PLMN | O |  | 9.3.1.194 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause Indication for Voice Service | O |  | ENUMERATED (supported, …) | This IE indicates whether the UE supports the feature of indication of paging cause for voice service. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforInactive | Maximum no. of TAIs for RRC Inactive. Value is 16. |

<Unchanged Text Omitted>

#### 9.3.3.52	Extended UE Identity Index Value

This IE is used by the NG-RAN node to calculate the Paging Frame as specified in TS 36.304 [29], or the UE_ID based subgrouping ID as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended UE Identity Index Value | M |  | BIT STRING (SIZE(16)) |  |

Change Ends