3GPP TSG-RAN WG3 #119                                                    R3-230594

Athens, 27th – 3rd March, 2023

Agenda Item:	8.1

Source:	CATT

Title:	Discussion on RAN impact for NPN enhancement in Rel-18

Document for:	Discussion

Introduction

A LS is received from SA2 in [1] about RAN impact for NPN enhancement in R18.

SA2 has finalized the study FS_eNPN_Ph2. RAN impacts are identified for KI#1 and KI#2, which correspond to the following objectives in the approved WID (SP-221340) at SA#98e.

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection, including:

2.	Support for non-3GPP access for SNPN.

The SA2 normative work for KI#1 and KI#2 has been progressed with specification updates to Rel-18 versions of the TS 23.501 and TS 23.502 containing the NG-RAN impacts.

The NG-RAN impacts identified are impacts to NGAP to support equivalent SNPNs, and impacts on cell (re-)selection for objective 1, and NGAP impacts to support selected SNPN (NID) for objective 2.

In this paper, we further analysis the detail impact in RAN3 for KI#1 and KI#2.

Discussion

2.1 KI#1: Enabling support for idle and connected mode mobility between SNPNs without new network selection.

If the UE moves its 3GPP access between SNPN and PLMN, the network selection is performed as specified in TS 23.122 [17] and UE performs initial registration as specified in clause 4.2.2.2.2 of TS 23.502 [3].

If the UE moves its 3GPP access between SNPNs, the network selection is performed as specified in TS 23.122 [17], then the UE performs initial or mobility registration as specified in clause 4.2.2.2.2 of TS 23.502 [3].

If the UE and network supports equivalent SNPNs, the AMF may provide list of equivalent SNPNs to the UE and NG-RAN. The UE may move its 3GPP access to the SNPN in the list of equivalent SNPNs without performing network selection. A UE supporting equivalent SNPNs gets a new registered SNPN ID during the Registration procedure if serving SNPN is changed.

It can be found that a list of equivalent SNPNs should be provided from AMF to NG-RAN via NGAP to support mobility between SNPNs. The list of equivalent SNPNs can be added in NPN mobility information in Mobility restriction list in both XnAP and NGAP.

38.413 and 38.423

NPN Mobility Information

This IE indicates the access restrictions related to an NPN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NPN Mobility Information | M |  |  |  |
| >SNPN Mobility Information |  |  |  |  |
| >>Serving NID | M |  | NID 9.3.3.42 |  |
| >>Equivalent SNPNs |  | 0..<maxnoofESNPNs> |  | Allowed SNPNs in addition to Serving SNPN. This list corresponds to the list of "equivalent SNPNs" as defined in TS 24.501 [26]. This list is part of the roaming restriction information. Roaming restrictions apply to SNPNs other than the Serving SNPN and Equivalent SNPNs. |
| >>>NID | M |  | 9.3.3.42 |  |
| >PNI-NPN Mobility Information |  |  |  |  |
| >>Allowed PNI-NPN List | M |  | 9.3.3.45 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofESNPNs | Maximum no. of Equivalent SNPNs. Value is 15. |

Proposal 1: Introduce a list of equivalent SNPNs in NPN mobility information in Mobility restriction list in Xn/NG.

AMF needs to be awarded of the target NID when supporting connected mode mobility between SNPNs. The Target ID in Handover required message can be enhanced to convey NID as below for NG based handover.

38.413

9.3.1.25	Target ID

This IE identifies the target for the handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Target ID | M |  |  |  |
| >NG-RAN |  |  |  |  |
| >>Global RAN Node ID | M |  | 9.3.1.5 |  |
| >>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >>Target NID | O |  | NID 9.3.3.42 |  |
| >E-UTRAN |  |  |  |  |
| >>Global eNB ID | M |  | Global ng-eNB ID 9.3.1.8 |  |
| >>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| >Target RNC-ID |  |  |  |  |
| >>LAI | M |  | 9.3.3.30 |  |
| >>RNC-ID | M |  | 9.3.1.123 | This IE is ignored if the Extended RNC-ID IE is included in the Target ID IE. |
| >>Extended RNC-ID | O |  | 9.3.1.124 | The Extended RNC-ID IE is used if the RNC identity has a value larger than 4095. |

Proposal 2: Introduce Target NID in Target ID IE in Handover required message.

2.2 KI#2: Support of Non-3GPP access for SNPN

In 23.502, the chapter of 4.12.2.2 Registration procedure for untrusted non-3GPP access including the following description. It requires that NG-RAN provide selected PLMN ID and optionally the Selected NID and the Establishment cause to AMF in N2 message i.e., Initial UE message.

6.	The N3IWF shall select an AMF based on the received AN parameters and local policy, as specified in clause 6.3.5 of TS 23.501 [2]. The N3IWF shall then forward the Registration Request received from the UE to the selected AMF within an N2 message. This message contains N2 parameters that include the Selected PLMN ID and optionally the Selected NID and the Establishment cause.

NOTE 3:	The Selected NID is present when the UE connects to an SNPN via Untrusted non-3GPP access.

Also, in the chapter of 4.12a.2.2 Registration procedure for trusted non-3GPP access, it has the similar description about N2 message i.e., Initial UE message.

-	In the N2 message sent in step 6b, the TNGF includes a UE Location Information (ULI) that contains a "null" IP address (e.g. 0.0.0.0) because the UE is not yet assigned an IP address. After the UE is assigned an IP address, the TNGF includes this address in subsequent N2 messages. This N2 message also includes the Selected PLMN ID and optionally the Selected NID, and the Establishment cause.

NOTE 3:	The Selected NID is present when the UE connects to an SNPN via Trusted non-3GPP access.

The selected NID should be introduced in Initial UE message as an optional IE.

9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| Source to Target AMF Information Reroute | O |  | 9.3.3.27 |  | YES | ignore |
| Selected PLMN Identity | O |  | PLMN Identity 9.3.3.5 | Indicates the selected PLMN id for the non-3GPP access. | YES | ignore |
| Selected NID | O |  | NID 9.3.3.42 | Indicates the selected NID for the non-3GPP access. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) | Indication of an IAB node | YES | reject |
| CE-mode-B Support Indicator | O |  | 9.3.1.156 |  | YES | reject |
| LTE-M Indication | O |  | 9.3.1.157 |  | YES | ignore |
| EDT Session | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Authenticated Indication | O |  | ENUMERATED (true, …) | Indicates the FN-RG has been authenticated by the access network. | YES | ignore |
| NPN Access Information | O |  | 9.3.3.46 |  | YES | reject |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |

Proposal 3: Introduce selected NID in Initial UE message as an optional IE in NG.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Proposal 1: Introduce a list of equivalent SNPNs in NPN mobility information in Mobility restriction list in Xn/NG.

Proposal 2: Introduce Target NID in Target ID IE in Handover required message.

Proposal 3: Introduce selected NID in Initial UE message as an optional IE in NG.

# 4. Reference

[1] R3-230036, LS on RAN impact for NPN enhancement in Rel-18, SA2.