3GPP TSG-RAN Meeting #119	R3-230629

# 27th Feb – 3rd March 2023

# Athens, Greece

Agenda item:	8.1

Source: 	ZTE

Title: 	Discussion on Tracking IANA assignment requests

Document for:	Discussion and Agreement

Introduction

In the RAN#98-e meeting, the LS on Tacking IANA assignment requests has been treated[1], and this LS has been also sent to RAN3 before this meeting.

In this paper, we provide some discussion on the IANA port allocation issue for W1 interface  in RAN3.

Discussion

In RAN3#112-e meeting, RAN3 has replied the LS related to the IANA port allocation issue in R3-212800 [2]. To be more specific, RAN3 prefers the Solution 1 and Solution 2 in TR 29.835 (corresponding to solution#6 and solution#7 in TR 29.941), i.e., 3GPP allocated port number and OAM allocated port number respectively.

RAN3 would like to thank CT4 for their LS on Information on the port number allocation solutions.RAN3 had discussions the different solutions in the TR, and reached the following understandings: From RAN3 perspective: - Both Solutions 1 and 2 are feasible. - RAN3 also noticed that Solution 11 is a once-and-for-all solution that can be considered, though its adoption is not entirely under 3GPP control. It requires IETF endorsement. - The rest of the solutions are not desirable.

Currently, for W1 interface, the SCTP Destination Port number value (i.e. 37472) has been assigned by IANA for W1AP. And SCTP port number values for other RAN3 interfaces, e.g. X2, Xn and F1, have also been assigned by IANA. The details have been given in the following table[3].

Table C.5-1: Service Name/port number assigned to 3GPP since 2009

| Service Name | Port Number | Transport Protocol | Description | Registration Date | Intra/Inter |
| --- | --- | --- | --- | --- | --- |
| sgsap | 29118 | sctp | SGsAP | 11/06/2009 | Intra (MME/MSC) |
| sbcap | 29168 | sctp | SBcAP | 11/06/2009 | Intra (MME/CBC) |
| s102 | 23272 | udp | S102 application | 26/08/2009 | Intra (1xCS IWS/MME) |
| s1-control | 36412 | sctp | S1-Control Plane | 01/09/2009 | Intra (MME/eNB) |
| x2-control | 36422 | sctp | X2-Control Plane | 01/09/2009 | Intra (eNB/eNB) |
| iuhsctpassoc | 29169 | sctp | HNBAP and RUA Common Association | 08/09/2009 | Inter (HNB/HNB-GW) |
| 3gpp-cbsp | 48049 | tcp | Cell Broadcast Service Protocol | 07/12/2009 | Intra (BSC/CBC) |
| lcs-ap | 9082 | sctp | LCS Application Protocol | 04/06/2010 | Intra (MME/E-SMLC) |
| wlcp | 36411 | udp | Wireless LAN Control plane Protocol (WLCP) | 14/11/2014 | Intra (UE/TWAG) |
| slmap | 36423 | sctp | SLm Interface Application Protocol | 18/06/2015 | Intra (E-SMLC/LMU) |
| nq-ap | 36424 | sctp | Nq/Nq' Application Protocol | 18/06/2015 | Intra (the RCAF/MME or SGSN) |
| xw-control | 36462 | sctp | Xw-Control Plane | 13/11/2015 | Intra (eNB/WT) |
| pfcp | 8805 | udp | Destination Port number for PFCP | 08/05/2017 | Intra (CU/UP) |
| ng-control | 38412 | sctp | NG Control Plane | 18/05/2017 | Intra (gNB/ng-eNB-AMF) |
| xn-control | 38422 | sctp | Xn Control Plane | 18/05/2017 | Intra (gNB-gNB/ng-eNB) |
| f1-control | 38472 | sctp | F1 Control Plane | 23/06/2017 | Intra (gNBCU/gNBDU) |
| e1-interface | 38462 | sctp | E1 signalling transport | 06/11/2018 | Intra (gNB-CU-CP/gNB-CU-UP) |
| 3gpp-monp | 8809 | udp | MCPTT Off-Network Protocol (MONP) | 15/04/2019 | Intra (MCPTT client/MCPTT client) |
| 3gpp-w1ap | 37472 | sctp | W1 signalling transport | 16/07/2020 | Intra (ng-eNB-DU/ng-eNB-CU |

However, for the UDP/TCP port assignment, the corresponding port number of W1 is still missing. To be more specific, according to the TS 29.281, the F1-U, Xn and N3 interfaces have been captured in the scope of user plane of GTP in the 5G System, while the W1-U has not been included[4].

The present document defines the user plane of GTP used on: - the Gn and Gp interfaces of the General Packet Radio Service (GPRS); - the Iu, Gn and Gp interfaces of the UMTS system; - the S1-U, S11-U, S2a, S2b, X2, S4, S5, S8, S12, M1 and Sn interfaces of the Evolved Packet System (EPS); - the F1-U, Xn, N3, N9, N19, N3mb and N19mb interfaces of the 5G System (5GS). This definition ensures full backwards compatibility with RNC, SGSN and GGSN implementations according to release 7 of 3GPP TS 29.060 [6]. NOTE: Releases previous to Release-8 have used 3GPP TS 29.060 [6] as normative definition of the user plane of GTP. This shall be considered when essential corrections are included in the present document or in pre-release-8 version of 3GPP TS 29.060 [6]. Fallback from GTPv1-U to GTPv0-U shall not be supported. Therefore, 3GPP Rel-8 and onwards GTPv1-U entity should not listen to the well-known GTPv0 port 3386. If GTPv1 entity listens to the GTPv0 port, the entity shall silently discard any received GTPv0-U message.

However, based on the TS37.470 [5], the W1-U shall apply the GTP-U.

- 7.2 W1 User Plane Protocol (W1-U) Figure 7.2-1 shows the protocol structure for W1-U. The TNL is based on IP transport, comprising the UDP and GTP-U on top of IP. Figure 7.2-1: Interface protocol structure for W1-U

Therefore, the W1-U should also be captured in the user plane of GTP, as defined in TS29.281. Since CT4 is the WG who is responsible for TS29.281, RAN3 should inform CT4 of this issue.

Proposal: Send the reply LS to CT Chair and CT4 to inform that the W1-U has not been captured in the scope of user plane of GTP in the 5G system.

The draft reply LS has been given in [6].

Conclusion

Proposal: Send the reply LS to CT Chair and CT4 to inform that the W1-U has not been captured in the scope of user plane of GTP in the 5G system.

References

- RP-223487/R3-230006, LS on Tracking IANA assignment requests, CT Chair

- R3-212800, Reply LS to CT4 on Information on the port number allocation solutions, RAN3

- TR29.835 v17.1.0

- TS29.281 v17.4.0

- TS37.470 v17.0.0

- R3-230330, Reply LS on Tracking IANA assignment requests