3GPP TSG-RAN Meeting #119	R3-230631

# 27th Feb – 3rd Mar 2023

# Athens, Greece

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.470 | CR | 0003 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up on W1 in TS37.470 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are some descriptions to be updated in Release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Correct the number of the technical specifications for W1. -Add the definition of W1-U. -Remove the redundant description. -Correct the typo issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some descriptions on the W1 interface are not correct. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects some stage 2 description of W1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1; 4.1; 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

1	Scope

The present document is an introduction to the 3GPP TS 37.47x series of technical specifications that define the W1 interface. The W1 interface provides means for interconnecting an ng-eNB-CU and an ng-eNB-DU of an ng-eNB within an NG-RAN.

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

3.1	Terms

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

W1: interface between an ng-eNB-CU and an ng- eNB-DU, providing an interconnection point between the ng-eNB-CU and the ng-eNB-DU.

W1-C: Reference point for the control plane protocol between ng-eNB-CU and ng-eNB-DU.

W1-U: Reference point for the user plane protocol between ng-eNB-CU and ng-eNB-DU.

ng-eNB-CU: a logical node hosting RRC, SDAP and PDCP protocols of the ng-eNB that controls the operation of one or more ng-eNB-DUs. The ng-eNB-CU terminates the W1 interface connected with the ng-eNB-DU.

ng-eNB-DU: a logical node hosting RLC, MAC and PHY layers of the ng-eNB, and its operation is partly controlled by ng-eNB-CU. One ng-eNB-DU supports one or multiple cells. One cell is supported by only one ng-eNB-DU. The ng-eNB-DU terminates the W1 interface connected with the ng-eNB-CU.

ng-eNB: as defined in TS 38.300 [3].

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

4.1	W1 interface general principles

The general principles for the specification of the W1 interface are as follows:

-	An ng-eNB may consist of an ng-eNB-CU and ng-eNB-DUs. An ng-eNB-CU and an ng-eNB-DU is connected via W1 logical interface.

-	One ng-eNB-CU controls one or more ng-eNB-DUs.

-	One ng-eNB-DU supports one or multiple cells. One cell is supported by only one ng-eNB-DU.

-	ng-eNB-DU ID is used to identify ng-eNB-DU only over W1AP procedures, ng-eNB-DU ID is not connected to cell identifier.

-	The ng-eNB-CU terminates W1 interface connected with the ng-eNB-DU.

-	The ng-eNB-DU terminates W1 interface connected with the ng-eNB-CU.

-	The W1 interface shall separate Radio Network Layer and Transport Network Layer.

-	The W1 interface shall enable exchange of UE associated information and non-UE associated information.

-	The W1 interface is open;

-	From a logical standpoint, the W1 is a point-to-point interface between an ng-eNB-CU and an ng-eNB-DU.

NOTE:	A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	The W1 interface supports control plane and user plane separation;

-	The standard should not prevent to separated CP and UP.

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

8.3	E-UTRAN and NG-RAN; W1 application protocol (W1AP) (3GPP TS 37.473)

3GPP TS 37.473 [6] specifies the W1AP protocol for radio network control plane signalling over the W1 interface.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>