3GPP TSG-RAN WG3 Meeting #119	R3-230648

Athens, GR, 27th February –3rd March, 2023

Agenda Item:	10.2.1

Source:	CMCC

Title:	SHR for intra-system inter-RAT handover

Document for:	Discussion and Decision

# 1 Introduction

At last RAN3 meeting, there was a lot of discussion on the support of SHR for intra-system inter-RAT handover, and some good progress were achieved, including the following agreements and working assumption.

Inter-RAT SHR

WA: Support inter-RAT SHR from LTE to NR at least for T304 if no impact on LTE.

WA: In R18, UE context retrieval in source Node is supported for inter-RAT SHR, the details are FFS.

This WA does not limit to the forwarding mechanism.

The forwarding mechanism is one of the key issues for inter-RAT SHR from NR to LTE, but there is still no conclusion achieved because the decision depends on RAN2 progress on whether T304 trigger is supported for inter-RAT SHR from NR to LTE.

In this contribution, we would provide some proposals on this open issue according to the latest agreements in RAN2.

# 2 Discussion

RAN3 discussed three options as shown below for the forwarding mechanism at network side for inter-RAT SHR from NR to LTE if T304 trigger is supported.

- Option 1: The receiving node forwards the inter-RAT SHR to source NR node, then source NR node forwards the inter-RAT SHR to target LTE node

- Option 2: The receiving node forwards the inter-RAT SHR to target LTE node, then target LTE node forwards the inter-RAT SHR to source NR node

- Option 3: The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR

But, the decision in RAN3 on forwarding mechanisms still depends on the discussion in RAN2 on whether T304 trigger is supported for inter-RAT SHR from NR to LTE.

According to the latest progress in RAN2#120 [1], we know that RAN2 has achieved the conclusion that T304 trigger is not supported for inter-RAT SHR from NR to LTE.

Agreements:

- T304 trigger for inter-RAT SHR from NR to LTE is not supported.

Combined with the previous agreements in RAN3#117bis-e, it means that only T310 and T312 related triggers are supported for inter-RAT SHR from NR to LTE.

Observation: Only T310 and T312 related triggers are to be considered for inter-RAT SHR from NR to LTE.

In this case, only source NR node need to perform the root cause analysis, and there is nothing to send to the LTE node. There is no need for the receiving node to forward the inter-RAT SHR to target LTE node and then forward to the source NR node.

Proposal 1: The receiving node forwards the inter-RAT SHR to source NR node for root cause analysis.

In this scenario, UE should encode the inter-RAT SHR in NR format and it is sufficient for the UE to report the inter-RAT SHR once UE is back to NR, i.e., the third node is a NR node. The presence of inter-RAT SHR availability indicator and SHR reporting is only upon UE coming back to NR node.

Proposal 2: For inter-RAT SHR from NR to LTE, the SHR is encoded in NR format and is retrieved from UE by NR node.

Based on the above discussion, for inter-RAT SHR from NR to LTE, the receiving node is also NR node just like the source NR node and can understand the NR format of inter-RAT SHR. Similar to intra-NR SHR, the receiving node can get the source NR cell ID from sourceCellInfo included in SuccessHO-Report, and there is no need to report the source cell ID outside of inter-RAT SHR from NR to LTE.

Proposal 3: Similar to intra-NR SHR, the parameter of the source NR cell ID is included in sourceCellInfo IE in inter-RAT SHR from NR to LTE.

# 3 Conclusion

In this paper, we discuss SHR for Intra-system inter-RAT handover, and we have the following proposals:

Observation: Only T310 and T312 related triggers are to be considered for inter-RAT SHR from NR to LTE.

Proposal 1: The receiving node forwards the inter-RAT SHR to source NR node for root cause analysis.

Proposal 2: For inter-RAT SHR from NR to LTE, the SHR is encoded in NR format and is retrieved from UE by NR node.

Proposal 3: Similar to intra-NR SHR, the parameter of the source NR cell ID is included in sourceCellInfo IE in inter-RAT SHR from NR to LTE.

# Reference

- R2-2213006, Report from SON/MDT session, 3GPP TSG-RAN WG2 Meeting #120, November 2022