3GPP TSG-RAN WG3 Meeting #119          	          R3-230659

Athens, Greece, 27th Feb – 3rd Mar 2023

Agenda item:	14.2

Source:	CMCC

Title:	(TP to TS 38.401) L1/L2 based Inter-Cell Mobility

Document for:	Approval

1	Introduction

This contribution provides the TP to TS 38.401 for L1/L2 based Inter-Cell Mobility based on the discussion paper.

- Text Proposal to TS 38.401 BLCR

--------------------------------------------------------------Start of the change---------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU L1/L2 based inter-cell Mobility

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 based inter-cell mobility. Figure 8.2.1.x-y shows the intra-gNB-DU L1/L2 based inter-cell mobility procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 based inter-cell Mobility

- The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the target candidate cell(s).

- If the request for configuring L1/L2 inter-cell mobility is accepted, the gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations of the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes a generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration, which may contain a configuration per target candidate (e.g. including the lower layer RRC configurations of the candidate cells for L1/L2 inter-cell mobility. Details are FFS).

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 inter-cell mobility.

- The gNB-DU sends the L1/L2 inter-cell mobility command to the UE.

- The gNB-DU indicates the gNB-CU that gNB-DU already triggers UE mobility to a target candidate cell by L1/L2 Mobility Notification message including the target cell ID.

- FFS: How the gNB-DU detects the UE access to the target cell is up to RAN2.

- The gNB-DU indicates the gNB-CU that the UE successful accesses to the target cell by Access Success message including the target cell ID.

- The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with the UE CONTEXT MODIFICATION RESPONSEmessage.

--------------------------------------------------------------Next change -----------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for L1/L2 Triggered Mobility. Figure 8.2.1.Y-1 shows the inter-gNB-DU L1/L2 Triggered Mobility procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cell(s).

- If the candidate gNB-DU decides to accept the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends lower layer measurement result to the source gNB-DU.

- The source gNB-DU sends L1/L2 Triggered Mobility Cell Switch command to the UE.

- The source gNB-DU indicates the gNB-CU that source gNB-DU already triggers UE mobility to a target candidate cell by L1/L2 Mobility Notification message including the target cell ID.

- FFS: How the target gNB-DU detects the UE access to the target cell is up to RAN2.

- The target gNB-DU indicates the gNB-CU that the UE successful accesses to the target cell by Access Success message including the target cell ID.

- The gNB-CU sends the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of cells.

- The gNB-DU responds with the UE CONTEXT RELEASE COMPLETE message.

--------------------------------------------------------------End of the change----------------------------------------------------------