3GPP TSG-RAN WG3 #119	R3-230671

27th Feb – 3rd Mar 2023

Athens, Greece

Agenda item:		14.2

Source:	LG Electronics Inc.

Title:	(TP for NR_Mob_enh2 BL CR for TS 38.401) Discussion on inter-gNB-DU LTM related impacts

Document for:		Other

# 1. Text Proposal for TS 38.401

Beginning of Text Proposal to TS 38.401

#### 8.2.1.Y	Inter-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for L1/L2 Triggered Mobility. Figure 8.2.1.Y-1 shows the inter-gNB-DU L1/L2 Triggered Mobility procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells.

- If the candidate gNB-DU decides to accept the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration.

FFS: whether it is DL RRC MESSAGE TRANSFER message or UE Context Modification Request message.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS: whether it is UL RRC MESSAGE TRANSFER message or UE Context Modification Response message.

- The UE sends lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

- The source gNB-DU sends a UE CONTEXT MODIFICATION REQUIRED message to the gNB-CU to notify the LTM cell switch decision.

- The gNB-CU responds to the source gNB-DU with a UE CONTEXT MODIFICATION CONFIRM message to indicate that LTM execution is confirmed.

- The source gNB-DU sends L1/L2 Triggered Mobility Cell Switch command to the UE.

Editor’s note: The cell switch command message needs to be updated according to RAN2’s discussion.

FFS:  how the target gNB-DU detects the UE access and whether there is an F1 impact.

FFS: For inter-DU L1/L2 Triggered Mobility, whether and how to release the source cell/prepared cells’ resources in the source gNB-DU is FFS.

End of Text Proposal to TS 38.401