3GPP TSG-RAN WG3 #119                                            R3-230678

27th Feb – 3rd Mar 2023

Athens, Greece

Agenda item:	8.1

Source: 	ZTE

Title: 	Consideration on NPN enhancement in Rel-18

Document for:	Discussion and Approval

Introduction

The contribution provides our view on NG-RAN support for phase2 enhancement of NPN in Rel-18.

Discussion

SA2 is working on eNPN_Ph2 WID in SP-220805 approved at TSG SA#97e. The overall plan for eNPN phase 2 SID/WID is shown below, this WID in SA2 is targeted to be completed in SA#99, March 2023.

During the SA#98e, the WID is revised into SP-221340. According to the revised WID, SA2 focuses on the following enhancement:

- Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection.

- 	Support for non-3GPP access for SNPN.

- 	Support for providing access to Localized Services.

Inter-SNPN mobility has impact on both RAN2 and RAN3 specs, UE/NG-RAN should take equivalent SNPN list into consideration, for supporting relevant functions, e.g. idle/connected mode mobility. The support of Non-3GPP access for SNPN seems only to have few impact on RAN3 specs, e.g, N3IWF and TNGF being able to include the "selected NID" in the [NGAP] INITIAL UE MESSAGE. In order to support providing access to localized services, It seems there are different options in TR23.700-08, e.g SIB option and RRC option in solution 44. (SIB option: Broadcasting in SIB the Hosting Network search assistance information; RRC option: receive from AMF indication that UE is enabled for Hosting Network search assistance, and provide Hosting Network search assistance information for such UE via RRC signalling).

In this meeting, RAN3 received LS in R3-230036 from SA2 for the progress of Non-Public Networks phase 2, seen below:

-------------------------------------------------------------------------------------------------------------------------------------------

1. Overall Description:

SA2 has finalized the study FS_eNPN_Ph2. RAN impacts are identified for KI#1 and KI#2, which correspond to the following objectives in the approved WID (SP-221340) at SA#98e.

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection, including:

2.	Support for non-3GPP access for SNPN.

The SA2 normative work for KI#1 and KI#2 has been progressed with specification updates to Rel-18 versions of the TS 23.501 and TS 23.502 containing the NG-RAN impacts.

The NG-RAN impacts identified are impacts to NGAP to support equivalent SNPNs, and impacts on cell (re-)selection for objective 1, and NGAP impacts to support selected SNPN (NID) for objective 2.

2. Actions:

To RAN2, RAN3

ACTION: 	SA2 kindly asks RAN2, RAN3 to take the above into account and consider RAN related work for completing the functionality for KI#1 and KI#2.

-------------------------------------------------------------------------------------------------------------------------------------------

It can be seen, the progress of key issue on providing access to Localized Services is not included in this LS, However, considering that there are some candidate solutions captured into TR23.700-08, we suggest to wait SA2's progress of key issue on providing access to Localized Services, and then start RAN3 work for Non-Public Networks phase 2.

Based on the above, we propose as below.

Proposal: RAN3 waits SA2 to complete FS_eNPN_Ph2 WID (assumes it will be completed in SA#99, March 2023, based on SA2's plan), and then starts RAN3 work for Non-Public Networks phase 2.

Conclusion

We propose the following observations and proposals:

Proposal: RAN3 waits SA2 to complete FS_eNPN_Ph2 WID (assumes it will be completed in SA#99, March 2023, based on SA2's plan), and then starts RAN3 work for Non-Public Networks phase 2.