3GPP TSG-RAN WG3 #119	R3-230831

Athens, Greece, EU, 27 Feb – 3 Mar 2023

Agenda Item:	9.2.4

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on coordination of MN- and SN-initiated conditional reconfiguration (CB #8)

Document for:	Approval

# Introduction

CB: # 8_SCGReconfig

- Check the issues/comments raised

- Provide CRs if agreeable

# For the Chairman’s Notes

R3-230746 rev in R3-230979 – endorsed

R3-230122 rev in R3-230924 – agreed

R3-230123 rev in R3-230925 – agreed

# Discussion (1st round)

## Stage-3

In [0122] and [0123], CRs with correction and completion of a solution decided at #117-bis were proposed. After online and offline peer-to-peer discussions, following issues were identified:

- Removal of “using SRB3” was made in wrong point: it should concern the point about ‘executed-deleted’ instead of the one on ‘executed’ (the reason is that the intra-SN CPC execution may happen over SRB1 – but it is covered with ‘executed-deleted’).

- It is proposed to add “with sync” to the point about ‘executed-deleted’. It is not critical, but can help clarify the scenario.

- The last sentence in the point on ‘executed-deleted’ is removed, because it duplicates with a statement few lines above.

- Grammatical correction of the point on ‘deleted’.

- ZTE and Google added as co-signing companies.

Updated versions of the CRs are uploaded. The changes are marked with ‘Nokia-rev1’ name.

Question 1: Please, comment if any other corrections are needed in stage-3.

| Company | Comment |
| --- | --- |
| Intel | Overall OK. The last thing to discuss is whether to include the case when “SN-initiated CPC” has been prepared – which we think is it better to cover SN-initiated CPC as well, as we discussed via offline emails before the meeting. |

## Stage-2

In [0746], a draft CR with a stage-2 description of the coordination has been proposed. It was commented in [0782] that the relation to the stage-2 description of reconfiguration with or without MN involvement should be clarified.

Question 3: If you consider that the proposed stage-2 CR should be corrected, please, propose the needed changes.

| Company | Comment |
| --- | --- |
| Intel | From the proposed stage-2 text: In case that a MN initiated conditional reconfiguration (e.g., CHO or MN inter-SN initiated CPC) is prepared, and if any execution of a prepared SN initiated intra-SN CPC procedure or reconfiguration of the SCG will impact the prepared MN initiated conditional reconfiguration, the SN notifies to the MN via the SN Modification Required message. Regarding the first highlight, the same comment as in Q1. “SN-initiated CPC” is better to be covered. Regarding the second highlight, we propose to simply remove the highlighted. “will” seems not proper for stage-2 and, as we commented during our discussions in AI 14.3, S-SN shall not judge on its own whether its intra-S-SN RRC reconfiguration activity would impact the configuration generated from the other node or not. |
| Lenovo | First highlight is fixed. Second highlight I think it’s better to keep as it is. Since it is the logic based on which SN will decide to indicate executed/deleted to MN. |

# Conclusion, Recommendations [if needed]

If needed

# References

- [0122]	R3-230122, RAN3 #119

- [0123]	R3-230123, RAN3 #119

- [0746]	R3-230746, RAN3 #119

- [0782]	R3-220782, RAN3 #119