3GPP TSG-RAN WG3 Meeting #119	R3-230848

# Athens, Greece, 27 February-3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.455 | CR | 0118 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LPPA corrections of references to RRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the agreed guidelines in RAN3#118 meeting, the references to RRC specifications need to be corrected in LPPA. LPP references can also be corrected similarly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the references to RRC specification according to the agreed rules. Propose to correct references to LPP specification in a similar way. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The CR has no impact from an ASN.1 point of view and does not introduce functional changes for implementations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains in LPPA about RRC and LPP references. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.14, 9.2.20, 9.2.21, 9.2.22, 9.2.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: correcting WI code, output of CB # 11_RapporteurUpdates |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 9.2.14	MBSFN subframe Configuration

The MBSFN subframe Configuration IE describes the MBSFN subframe configuration for the concerned cell/TP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MBSFN subframe Configuration Value |  | 1 .. < maxMBSFN-Allocations > |  |  |
| >Radio Frame Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32) | Corresponds to information provided in radioFrameAllocationPeriod contained in the MBSFN-SubframeConfig IE as defined in TS 36.331 [10] |
| >Radio Frame Allocation Offset | M |  | INTEGER (0..7) | Corresponds to information provided in radioFrameAllocationOffset contained in the MBSFN-SubframeConfig IE as defined in TS 36.331 [10] |
| > CHOICE Subframe Allocation | M |  |  | Corresponds to information provided in subframeAllocation contained in the MBSFN-SubframeConfig IE as defined in TS 36.331 [10] |
| >>oneFrame | M |  | BIT STRING (SIZE(6)) |  |
| >>fourFrames | M |  | BIT STRING (SIZE(24)) |  |

| Range bound | Explanation |
| --- | --- |
| maxMBSFN-Allocations | Maximum number of MBSFN frame allocations with different offset as defined in TS 36.331 [10]. Value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.20	Assistance Information

This IE contains the assistance information.

Table 9.2.20-1: Assistance Information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information | M |  |  |  |
| >System Information |  | 1..<maxNrOfPosSImessage> |  | Corresponds to the number of SI messages with posSIBs to be scheduled |
| >>Broadcast Periodicity | M |  | ENUMERATED (ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ...) | Corresponds to information provided in posSI-Periodicity contained in the SystemInformationBlockType1 IE as defined in TS 36.331 [10] |
| >>Pos SIBs |  | 1..<maxNrOfPosSIBs> |  | Number of posSIBs in the System Information. |
| >>>PosSIB-Type | M |  | 9.2.23 |  |
| >>>PosSIB Segments | M |  | 9.2.21 |  |
| >>>Assistance Information Meta Data | O |  | 9.2.22 |  |
| >>>Broadcast Priority | O |  | INTEGER (1..16, ...) | The priority of the assistance Information where 1 represents the highest priority and 16 the lowest priority |

| Range bound | Explanation |
| --- | --- |
| maxNrOfPosSImessage | Maximum number of positioning system information messages. Value is 32. |
| maxNrOfPosSIBs | Maximum number of positioning system information blocks included in the message. Value is 32. |

### 9.2.21	PosSIB Segments

This IE provides one posSIB or two or more posSIB segments which must be scheduled in series in consecutive transmissions of the same SI message.

Table 9.2.21-1: PosSIB Segments

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PosSIB Segments |  | 1..<maxNrOfSegments> |  |  |
| >Assistance Data SIB Element | M |  | OCTET STRING | Includes the assistanceDataSIBElement IE as defined in TS 36.355 [13] |

| Range bound | Explanation |
| --- | --- |
| maxNrOfSegments | Maximum number of positioning SIB segments (in case of Assistance Information Element contains segmented data according to TS 36.355 [13]). Value is 64. |

### 9.2.22	Assistance Information Meta Data

This parameter contains meta data for an assistance information element.

Table 9.2.22-1: Assistance Information Meta Data

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Encrypted | O |  | ENUMERATED (true, …) | Corresponds to information provided in encrypted contained in the SystemInformationBlockType1 IE as defined in TS 36.331 [10] |
| GNSS ID | O |  | ENUMERATED (gps, sbas, qzss, galileo, glonass, bds, ..., navic) | Corresponds to information provided in gnss-id contained in the SystemInformationBlockType1IE as defined in TS 36.331 [10] |
| SBAS ID | O |  | ENUMERATED (waas, egnos, msas, gagan, ...) | Corresponds to to information provided in sbas-id contained in the SystemInformationBlockType1IE as defined in TS 36.331 [10] |

### 9.2.23	Positioning SIB Type

This parameter defines a specific positioning SIB, as defined in TS 36.331 [10].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning SIB Type | M |  | ENUMERATED ( posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType3-1, ..., posSibType4-1, posSibType5-1, posSibType2-24, posSibType2-25) |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>