3GPP TSG-RAN WG3 Meeting #119	R3-230930

Athens, Greece, 27 February – 3 March, 2023

Agenda item:	10.2.3

Source:	Nokia, Nokia Shanghai Bell, Ericsson, ZTE

Title:	(TP for SON BL CR to TS 38.473) TP for RACH optimisation

Document for:	Text Proposal

# 1	Introduction

This is TP for RACH optimisation following discussions at RAN3#119.

# 2	References

[1]		R3-226911, (TP for SON BL CR for TS 38.473) RACH Optimization enhancement

# TP for BL CR to TS 38.473

This TP is based on [1]

<<< start of changes >>>

## 8.2	Interface Management procedures

### 8.2.x		RACH Indication

#### 8.2.x.1		General

This procedure is initiated by the gNB-DU to inform the gNB-CU about the occurrences of successful random access procedures in the gNB-DU.

The procedure uses non-UE associated signalling.

#### 8.2.x.2		Successful Operation

Figure 8.2.x.2-1: RACH Indication procedure.

The gNB-DU initiates the procedure by sending the RACH INDICATION message to the gNB-CU.

The RACH INDICATION message contains information concerning one or more successful random access procedures occurring in the gNB-DU.

Upon reception of the RACH INDICATION message, the gNB-CU may trigger retrieval of RACH Reports from the UE.

#### 8.2.x.3 	Abnormal Conditions

Not applicable.

<<< next change >>>

#### 9.2.1.x	RACH INDICATION

This message is sent by the gNB-DU to inform the gNB-CU about one or more random access procedures performed at the gNB-DU.

Direction: gNB-DU ® gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RACH Indication List |  | 1..<maxnoofRACHIndications> |  |  | YES | ignore |
| >RACH Report List Item |  | 1 |  |  |  |  |
| >> gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | ignore |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |  |  |
| >>Random accessIndication | O |  | ENUMARATED (true, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHIndications | Maximum number of RACH Indications. Value is FFS. |

<<< end of changes >>>