3GPP TSG-RAN WG3 Meeting #119	R3-230965

Athens, GR, 27 Feb – 03 Mar, 2023

Title: 	(TP for SON BLCRs for TS 38.300): RACH report retrieval

Source: 	Huawei, Orange, China Telecom, CMCC, Ericsson

Agenda item:	10.2.3

Document Type:	Other

1. Introduction

The whole RACH report retrieval machnisam in CU DU split and MR-DC were agreed at last meeting. The following agreements are captured in chair minutes:

RAN3 supports a network-based solution for RACH report retrieval over F1AP based on an indication from the gNB-DU to the gNB-CU of successful RACH procedures which are not known to the gNB-CU (e.g., when RACH is triggered due to beam failure recovery, no PUCCH resource available, UL sync issue).

Define a new class-2 F1AP message (e.g., RACH INDICATION) to indicate certain RACH occurrence(s) from gNB-DU to gNB-CU.

FFS whether the new F1AP message is UE-associated or non-UE associated.

SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN

Define a new class-2 message (e.g., RACH INDICATION) over Xn so that the S-NG-RAN can inform M-NG-RAN that one or more RACH reports are available at the UE.

The new Xn message should be non-UE associated.

It is proposed to agree on the stage 2 TP in Annex to reflect the agreements mentioned above.

Annex – TP for SON BLCR for TS 38.300

15.5.3	Support for RACH Optimization

RACH optimization is supported by UE reported information made available at the NG RAN node as specified in TS 38.331 [12] and by PRACH parameters exchange between NG RAN nodes.

The contents of the RACH information report comprise of the following:

-	Contention detection indication per RACH attempt;

-	Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts;

-	Indication whether the selected SSB is above or below the configured RSRP threshold per RACH attempt;

-	2-step RACH information as specified in clause 5.7.10.4 of TS 38.331 [12].

The UE operating in NR-DC, may also support rSgNB RACH information report.

RA Report retrieval in case of dual connectivity:

In MR-DC, when UE performs a successful random-access procedure in the SN, the SN may inform the potential availability of RA Report in the UE to the MN via a RACH indication. The MN may then retrieve the RA Report from the UE based on the RACH indication received via XnAP signalling from SN.