3GPP TSG-RAN WG3 #119                                                                   R3-230998 was R3-230916

Athens, Greece, 27th Feb. – 3rd March, 2023

Title: 	(TP for MT-SDT BL CR to TS 38.300) Support of MT-SDT

Source: 	CATT, ZTE, Nokia, Nokia Shanghai Bell, Xiaomi, LG Electronics, Ericsson, Huawei, Lenovo, China Telecom, Intel Corporation

Agenda item:	20.2

Document for:	Discussion and Decision

- Introduction

Following the discussion of MT-SDT in RAN3#119 (SoD: R3-230834), this contribution provides a stage 2 TP for MT-SDT BL CR to TS 38.300, to make support of MT-SDT.

2. (TP for MT-SDT BL CR to TS 38.300) Support of MT-SDT

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

18.x	MT-SDT overall procedure

The overall procedure for MT-SDT is illustrated in the figure 18.x-1.

Figure 18.x-1: MT-SDT overall procedure

- DL SDT data and/or DL SDT signalling are received at the last serving gNB for the UE in RRC_INACTIVE state.

- RAN Paging is triggered, either only in the cells controlled by the last serving gNB or also by means of Xn RAN Paging in cells controlled by other gNBs within the RNA. The receiving gNB takes account the MT-SDT information received in RAN Paging from the last serving gNB to trigger the MT-SDT Paging towards the UE.

Editor’s Note: FFS on the details of MT-SDT Information in the Xn RAN Paging message.

- If the UE has been successfully reached, it initiates the MO-SDT procedure for as described in clause 18.2 or 18.3 with the following differences:

- Editor’s Note: FFS on the differences with MO-SDT and whether separate flow is needed or not.

- Editor’s Note: step 3 should follow RAN2 progress.

- Editor’s Note: FFS on whether to provide some MT-SDT specific info (e.g. MT-SDT resume cause or a specific indicator) in the RETRIEVE UE CONTEXT REQUEST.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>