3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231151

# Electronic meeting, 17 Apr – 26 Apr, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1123 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR) Support for NR Sidelink Relay Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, LGE, CATT, Ericsson, Nokia, Nokia Shanghai Bell, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2023-03-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduce the changes required to support Rel-18 Sidelink Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added transfer of UE authorization information for Multi-path ...To be completed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for SL relay enhancements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.268, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Updated the IE name Rev 2: resubmitted to RAN3#119b |  |  |  |  |  |  |  |  |

Start of change

#### 9.3.1.268	5G ProSe Authorized

This IE provides information on the authorization status of the UE for NR ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE |
| 5G ProSe Layer-2 Multi-path | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the 5G ProSe Layer-2 Remote UE is authorized for 5G ProSe multi-path transmission. |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-FiveG-ProSeLayer2Multipath,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

[snip]

FiveG-ProSeAuthorized ::= SEQUENCE {

fiveG-proSeDirectDiscovery					FiveG-ProSeDirectDiscovery									OPTIONAL,

fiveG-proSeDirectCommunication				FiveG-ProSeDirectCommunication								OPTIONAL,

fiveG-ProSeLayer2UEtoNetworkRelay			FiveG-ProSeLayer2UEtoNetworkRelay							OPTIONAL,

fiveG-ProSeLayer3UEtoNetworkRelay			FiveG-ProSeLayer3UEtoNetworkRelay							OPTIONAL,

fiveG-ProSeLayer2RemoteUE					FiveG-ProSeLayer2RemoteUE									OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { {FiveG-ProSeAuthorized-ExtIEs} }	OPTIONAL,

...

}

FiveG-ProSeAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-FiveG-ProSeLayer2Multipath	CRITICALITY ignore	EXTENSION FiveG-ProSeLayer2Multipath		PRESENCE optional		},

...

}

FiveG-ProSeAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FiveG-ProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer3UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2RemoteUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2Multipath ::= ENUMERATED {

authorized,

not-authorized,

...

}

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

[snip]

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-FiveG-ProSeLayer2Multipath						ProtocolIE-ID ::= 900 -- assigned by MCC

End of change