3GPP TSG-RAN WG3 Meeting #119-bis	R3-231192

Online, 17 – 26 Apr 2023

Agenda item:	14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP to TS38423, CHO with NRDC] Avoiding unnecessary CHO replace & data forwarding in case of CHO with multiple SCGs

Document for:	Endorsement

# 1	Introduction

After the December RAN plenary meeting, the scope of the work on Mobility Enhancements for NR-DC can be summarised in 3 points:

- Data forwarding for CHO with DC kept at the target

- Avoiding unnecessary CHO cancellation or replace

- CHO with multiple SCGs at the target

At the last RAN3 #119 meeting, the first topic focused most of the group’s attention and is considered separately at this meeting. However, there is nearly no progress on the two other topics. Therefore, in this paper, we address these two last topics. The paper is based on the one submitted to RAN3 #119 meeting.

# 2	Discussion

## 2.1	Avoiding unnecessary CHO cancellation or replace

At RAN #98, the WID was updated and an additional objective added:

to specify, if needed, a solution to avoid unnecessary signaling exchange between source MN and target SN.

This reflects an agreement from RAN3 #118:

RAN3 acknowledges unnecessary signaling exchange between MN and the target SN would cause inefficiency and extra latency for CHO + NR-DC, the solution is FFS.

One may observe that there is no direct signalling between the source MN and the target SN, so RAN3 must re-interpret the objective.

Observation 1-1: Since there is no direct signalling between the source MN and the target SN, RAN3 must interpret the objective as avoiding unnecessary signalling between the source MN and the target node, which is the target MN.

Already at RAN3 #118, it was discussed that there are, indeed, scenarios where an update of the SCG part of the UE config does not require updating or cancelling the CHO. For example, in case of a full configuration, the target configures the UE anew, so no changes at the source side affect the prepared CHO (possibly with SCG).

In case delta config is used, in theory, the source SCG config shall not be modified. However, it is still important to note is that some reconfigurations do not concern the target and the source may safely execute them without re-initiating CHO preparation. This applies in particular to any radio-layer-related changes (e.g. CSI-RS resource configuration or measurement gaps, which are specific to the source cell configuration).

A grey zone concerns configuration of the UE that do not concern the service itself and may be kept at the target after the HO, but does not have to. Examples of such configurations are SCell configuration or measurement gap config, but there may be more – the problem belongs to RAN2.

Obviously, in the current setup, the source does not know whether or not the target keeps such setting and consequently must re-initiate the CHO preparation – if it does not, the UE may end up in undefined state if the old HO command was issued with the intention that certain setting is to be kept. On the other hand, such re-initialisation is very costly.

Observation 1-2: There are scenarios where the modification of the SCG configuration at the source does not require re-initialising the CHO. However, in most cases the information if the particular change affects the prepared CHO with SCG is known at the target side.

Proposal 1-1: RAN3 acknowledges that source MN may conclude that a prepared CHO needs or does not need to be cancelled or re-initialised shall be based on the input from the target MN/SN.

The first option for a solution was proposed already in [3]: the source side knows the policy of the target side – what is decided to be kept and what is reset. All this information may be deduced from the HO Command, so the source may, if implemented so, check the HO command to see what exactly is kept at the target. This method is normally not recommended in the standard though.

The biggest problem is the list of the configurations that may or may not be kept at the target. As discussed above, examples are the measurement gap configuration or SCell setup – but there are likely more. Only RAN2 is able to define the list, but there may be not enough time to collect it before the WI is closed.

Therefore, the solution must be quite flexible, so details could be clarified also later (and so that new configurations can be added in future). The simplest option seems to be a bitmap, where each bit corresponds to a configurable option and tells the source if the target keeps the config (e.g. ‘1’) or releases it at the UE (‘0’). RAN3 may proactively include the two aforementioned features and send an LS to RAN2 to collect the complete list for Rel.16. Once collected, other positions of the list could be defined without affecting the ASN.1.

In practice, in case of a CHO with MR-DC, the bitmap can be compared in the source MN. The target MN and SN formulate it (i.e. indicate what features can be modified freely) and provide to the source MN in the HO REQ ACK message, the MN stores it. Then, when the source SN indicates that SCG is to be or has just been reconfigured, the SN provides the SCG bitmap indicating which features have been modified. The MN may then compare the two bitmaps and decide if cancellation or re-initialisation of the CHO is needed – even if the MN can’t decode the SCG config!

Proposal 1-2: RAN3 to consider a solution where a bitmap (e.g. 16 bits) is added to the CHO Request Acknowledge, where each bit corresponds to a configurable option that may or may not be kept at the target for the UE. Details of the usage of the bitmap must be consulted with RAN2.

A lighter option is to let the target to decide if given configuration update require re-preparing the CHO. In this case, the source always sends the CHO request to the target, but the target analyses the request and included configuration and responds if the CHO must be re-configured at the UE (i.e. if the new HO command needs to be provided to the UE). In this scenario, the bitmap is replaced with a simple flag and the signalling at the RAN side is not optimised. However, the most costly part, i.e. the radio signalling related to re-preparing the CHO at the UE, is avoided.

Proposal 1-3: If a complete bitmap solution with RAN2 consultation is too “heavy”, RAN3 shall consider a lighter solution, where the target MN informs the source side whether the HO command must be re-sent to the UE. The HO command may be skipped in this case altogether.

## 2.2	CHO with multiple SCGs at the target

As discussed in [1], one of the problems related to the CHO with multiple prepared SCGs is the early data forwarding: each target MN may prepare multiple target SNs. This is a new problem: in Rel.17, either multiple target MNs could prepare single target SN (CHO with DC kept at the target), or a single MN could prepare CPA in multiple target SNs (CPAC).

Observation 2-1: Early data forwarding to the target MN with multiple target SNs is a new problem.

However, at RAN3 #117-bis, it was acknowledged that RAN3 aims at enabling it, if feasible:

Early Data Forwarding optimizations with involvement of the target SCG(s) in Rel-18 will be supported.

The first issue to consider is the problem of PDU session setup: in the CHO with the Addition Preparation procedure, the target MN requests from multiple target SNs certain PDU sessions to be setup, but some of the SNs may reject some of them. Therefore, the target MN may not be able to provide the same response to the CHO request from all target SNs.

However, even if it is very likely that a target MN will request the same bearer setup in each prepared target SN, and even if all the SNs accept it, it may still happen that the prepared SNs create different set of DRBs. Therefore, the set of TEIDs for early data forwarding will depend on the pair MN-SN.

Proposal 2-1: RAN3 shall enable providing to the source SN a separate set of TEIDs for each prepared target pair MN-SN.

The TEID pair related to each target pair MN-SN should be indicated with a unique identifier to the UE at the time of handover preparation. Followingly, the unique identifier should be mapped to each CHO configuration. The source node can use the link between the TEID pair and the CHO configuration to initiate early data forwarding. For the late data forwarding, the unique identifier should be indicated in the HO success message from target MN to source MN. The source MN can determine to select the TEIDs related to the unique identifier and initiate late data forwarding accordingly while ignoring the other TEIDs.

Proposal 2-2: RAN3 shall enable an identifier in the HO Success, for example PSCell ID (PCell ID is already present) that will enable matching the selected PCell-PSCell pair with the TEIDs.

# 3	Conclusions

In this paper, we address several problems related to handling CHO with single or multiple SCGs at the target node. For the problem of unnecessary CHO cancellation or replacement, we conclude what follows:

Observation 1-1: Since there is no direct signalling between the source MN and the target SN, RAN3 must interpret the objective as avoiding unnecessary signalling between the source MN and the target node, which is the target MN.

Observation 1-2: There are scenarios where the modification of the SCG configuration at the source does not require re-initialising the CHO. However, in most cases the information if the particular change affects the prepared CHO with SCG is known at the target side.

Proposal 1-1: RAN3 acknowledges that source MN may conclude that a prepared CHO needs or does not need to be cancelled or re-initialised shall be based on the input from the target MN/SN.

Proposal 1-2: RAN3 to consider a solution where a bitmap (e.g. 16 bits) is added to the CHO Request Acknowledge, where each bit corresponds to a configurable option that may or may not be kept at the target for the UE. Details of the usage of the bitmap must be consulted with RAN2.

Proposal 1-3: If a complete bitmap solution with RAN2 consultation is too “heavy”, RAN3 shall consider a lighter solution, where the target MN informs the source side whether the HO command must be re-sent to the UE. The HO command may be skipped in this case altogether.

A drat of an LS to RAN2 is proposed in the Annex 2 below.

Finally, for the new problem of CHO with multiple SCGs at the target, we conclude that:

Observation 2-1: Early data forwarding to the target MN with multiple target SNs is a new problem.

Proposal 2-1: RAN3 shall enable providing to the source SN a separate set of TEIDs for each prepared target pair MN-SN.

Proposal 2-2: RAN3 shall enable an identifier in the HO Success, for example PSCell ID (PCell ID is already present) that will enable matching the selected PCell-PSCell pair with the TEIDs.

A TP implementing these solutions are proposed in the Annex 1 below.

# References

- R3-225391, RAN3 #117-bis

- R3-225352, RAN3 #117-bis

- R3-200213, RAN3 #107

# Annex 1: Text proposal to TS 38.423

First change, ommited text not changed

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >HO command update | O |  | ENUMERATED (ho-command-not-updated, ...) |  | – |  |
| >SCG config matrix | O |  | BIT STRING (SIZE(FFS)) |  | – |  |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |

Next change, ommited text not changed

#### 9.2.1.2	PDU Session Resources Admitted List

This IE contains PDU session resource related information to report success of the establishment of PDU session resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources Admitted List |  | 1 |  |  | – |  |
| >PDU Session Resources Admitted Item |  | 1..<maxnoofPDUSessions> |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Admitted Info | M |  |  |  | – |  |
| >>>DL NG-U TNL Information Unchanged | O |  | ENUMERATED (True, …) | Indicates the NG-U tunnels that have been kept unchanged at the target NG-RAN node | – |  |
| >>>QoS Flows Admitted List |  | 1 |  |  | – |  |
| >>>>QoS Flows Admitted Item |  | 1..<maxnoofQoSFlows> |  |  | – |  |
| >>>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>>Current QoS Parameters Set Index | O |  | 9.2.3.103 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>>QoS Flows not Admitted List | O |  | QoS Flow List with Cause 9.2.1.4 |  | – |  |
| >>>Data Forwarding Info from target NG-RAN node | O |  | 9.2.1.16 |  | – |  |
| >>>Secondary Data Forwarding Info from target NG-RAN node List | O |  | 9.2.1.31 | This IE would be present only when the target M-NG-RAN node decide to split a PDU session between MN and SN | YES | ignore |
| >>>Data Forwarding Info from Target MN-SN Pair List |  |  |  |  | YES | reject |
| >>>> Data Forwarding Info from Target MN-SN Pair Item |  | 1..<maxnoofSCGsMinusOne> |  |  | - | - |
| >>>Data Forwarding Info from target NG-RAN node | O |  | 9.2.1.16 |  | - | - |
| >>>Secondary Data Forwarding Info from target NG-RAN node List | O |  | 9.2.1.31 | This IE would be present only when the target M-NG-RAN node decide to split a PDU session between MN and SN | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofSCGsMinusOne | Maximum no. of additional SCGs (on top of the existing information) that may be prepared in a single CHO. Value is FFS. |

Remaining text not changed

# Annex 2: Draft LS to RAN2

3GPP TSG-RAN WG2 #119-bis			R3-23xxxx

Online, 17 – 26 Apr 2023

Title:	[draft] LS on creating a structure representing possible SCG reconfigurations

Response to:	—

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	Nokia, Nokia Shanghai Bell [to be RAN3]

To:	RAN2

Cc:	—

Contact Person:

#### Name:	Krzysztof Kordybach

E-mail Address:	krzysztof.kordybach@nokia.com

Attachments:	—

1. Overall Description:

RAN3 is considering enabling a solution that will enable the source MN to identify scenarios where SCG reconfiguration does not require cancelling or re-initialising of prepared CHO. In order to achieve this, RAN3 identified a solution where the target MN/SN informs the source MN about particular MCG/SCG features that may be reconfigured on the source side without impact on the prepared CHO. This requires defining a list of features that may be configured in the RRC Reconfiguration separately. The list will then be reflected in some structure that may be provided from the target node to the source node, for example, a bitmap, where each bit represents a feature.

RAN3 kindly asks RAN2 to evaluate if such structure can be defined and, if feasible, to define it.

2. Actions:

To RAN2:

ACTION:	RAN3 kindly asks RAN2 to take the above into consideration and to provide feedback on the feasibility of the considered solution.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting #120		25 – 26 May 2023				Incheon, Korea

TSG-RAN3 Meeting #121		21 – 25 Aug 2023				Toulouse, France