3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231315

Online meeting, Apr 17th - Apr 26th, 2023

Agenda item:	14.2

Source:	Qualcomm Incorporated

Title:	Signalling Support for LTM

WID/SID:	NR_Mob_enh2-Core – Release 18

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #119 [1]), there was progress towards determining the overall signalling flow for the intra-DU and inter-DU, intra-CU LTM procedures. The following agreements were made during the meeting.

RAN3 #119 agreements:

DL/UL TEID handling during LTM configuration:

For intra-DU LTM, the gNB-CU assigns a new UL GTP TEID for each DRB and provides it to the gNB-DU via UE Context Modification Request message(s). The gNB-DU assigns the new DL GTP TEIDs per DRB per candidate cell (whether it should be per candidate cell needs to be further discussed) and provides them back to the gNB-CU in UE Context Modification Response message(s).

For inter-DU LTM, the gNB-CU assigns a new UL GTP TEID for each DRB and provides it to the target gNB-DU via UE Context Setup Request message(s). The target gNB-DU assigns the new DL GTP TEIDs per DRB per candidate cell (whether it should be per candidate cell needs to be further discussed) and provides them back to the gNB-CU in UE Context Setup Response message(s).

Data transmission:

Intra-CU UP case: CU will start data transmission after LTM cells switch signaling from DU including target cell ID.

Whether new message or legacy message is FFS.

E1 aspects:

In case of CP UP separation, once CUCP receives LTM cell switch signling from (source)DU, CU CP initiates E1 bearer context modification including DL tunnel ID per DRB for target cell, and security keys corresponding to target cell (if updated) for data transmission.

The tentative flow chart to reflect above sentence is captured in section 3.6 in the Sod, to be continued based on this basis.

In this contribution, we address some of the issues that were left FFS in the previous meetings and discuss further proposals for the intra-DU and inter-DU, intra-CU LTM procedures.

# 2 Intra-DU LTM procedure

Figure 1 provides a signalling flow for the procedure.

RAN3 #117-bis-e agreements [3]:

During L1/L2 handover configuration, the gNB-CU sends the suggested candidate cell(s) to the gNB-DU in UE Context Modification Request. FFS in one message or multiple messages.

The gNB-DU may accept the target cells of L1/L2 handover and responds to the gNB-CU with the access control result in UE Context Modification Response message(s). gNB-DU may accept all or part of the target candidate cells.

From the previous meetings an FFS issue was whether it is possible to do parallel preparation of multiple candidate target cells for LTM.

We think that from the latency point of view it is better that the preparation of multiple candidate target cells takes place using a single set of request/response messages UE Context Modification Request/UE Context Modification Response. This involves changes to these messages which currently handle preparation of a single candidate target cell in legacy intra-DU handover or in Conditional Handover (CHO).

In the request message, the following information should be included:

- A list of suggested candidate cells for preparation for LTM.

  - For each candidate cell, the following information may be included:

    - List of DRBs and SRBs to be set up,

    - List of SCells to be set up.

RAN2 #119-bis-e agreements:

For L1L2 mobility will support that candidate configurations are delta configurations on top of a reference configuration. FFS if the reference configuration is a separate reference configuration or e.g. the current configuration.

In RAN2 #119-bis-e [4], it was agreed to introduce a reference configuration for LTM. The configurations of the candidate target cells and the associated SCells can be then delta configurations with respect to the reference configuration.

Upon receiving the list of suggested candidate cells from the CU, the DU can determine:

- List of accepted candidate cells for LTM configuration,

- A lower layer reference configuration (reference cell group configuration) for the accepted candidate cells,

- Lower layer configuration (cell group configuration) for each of the accepted candidate cells, that is a delta configuration with respect to the reference cell group configuration or is a full configuration.

Proposal 1. The preparation of multiple candidate target cells for LTM can be performed using a single set of request/response messages UE Context Modification Request/UE Context Modification Response for lower latency, i.e., not using parallel preparation.

Proposal 2. The UE Context Modification Request message from the CU to the DU includes a list of suggested candidate cells for preparation for LTM.

For each candidate cell, the following information may be included:

- List of DRBs and SRBs to be set up,

- List of SCells to be set up.

Observation 1. The specification of the reference configuration(s) for LTM is up to RAN2.

Proposal 3. The UE Context Modification Response message from the DU to the CU includes:

- List of accepted candidate cells for LTM configuration,

- A lower layer reference configuration (reference cell group configuration) for the cells in 1) (pending RAN2 progress),

- Lower layer configuration (cell group configuration) for each of the cells in 1), that is a delta configuration with respect to the reference cell group configuration or is a full configuration (pending RAN2 progress).

For each accepted cell, the following information may be included:

- List of accepted and failed DRBs and SRBs for setup,

- List of accepted and failed SCells for setup.

Upon receiving the reference cell group configuration for the accepted candidate cells, the CU can generate the overall reference configuration for the accepted candidate cells.

The CU provides to the UE the RRC reconfiguration message containing the LTM configurations for the candidate target cells. The CU may provide the reference configuration(s) in the message and delta configurations with respect to the reference configuration(s) for the candidate target cells.

In a previous meeting (RAN3 #118 [2]), it was agreed that the CU may cancel a configured LTM candidate target cell. It should also be possible for the DU to cancel a configured LTM candidate target cell, e.g., if it runs out of resources for the cell.

Proposal 4. DU may cancel a configured LTM candidate target cell, e.g., if it runs out of resources for the cell. DU provides an indication to the CU regarding this.

Figure 1: Intra-DU LTM procedure

In order to help with faster subsequent LTM, the prepared candidate target cells and the source cell need not be released. The CU may transmit a UE Context Modification Request to indicate to the DU to keep resources associated with a subset of the candidate target cells and the source cell (the indication may also be to keep resources associated with all the candidate target cells). This question is pending RAN2 discussion. There is UE impact since UE then also keeps the corresponding configurations of the candidate target cells and the source cell configuration.

Proposal 5. It is FFS (pending RAN2 progress) whether CU transmits a UE Context Modification Request to indicate to the DU to keep resources associated with a subset of the candidate target cells and the source cell (the indication may also be to keep resources associated with all the candidate target cells).

# 3 Intra-CU, Inter-DU LTM procedure

Figure 2 provides a signalling flow for the procedure.

As in intra-DU LTM, we think that in case of intra-CU, inter-DU LTM, the preparation of multiple candidate target cells in a target gNB-DU can take place using a single set of request/response messages UE Context Setup Request/UE Context Setup Response. This would involve changes to these request/response messages to provide the parameters corresponding to a list of cells as in the intra-DU LTM case.

The following are a set of proposals similar as those for the intra-DU LTM procedure.

Proposal 6. The preparation of multiple candidate target cells for LTM in a target DU can be performed using a single set of request/response messages UE Context Setup Request/UE Context Setup Response for lower latency, i.e., not using parallel preparation.

Proposal 7. The UE Context Setup Request message from the CU to a target DU includes a list of suggested candidate cells for preparation for LTM.

For each candidate cell, the following information may be included:

- List of DRBs and SRBs to be set up,

- List of SCells to be set up.

Proposal 8. The UE Context Setup Response message from the target DU to the CU includes:

- List of accepted candidate cells for LTM configuration,

- A lower layer reference configuration (reference cell group configuration) for the cells in 1) (pending RAN2 progress),

- Lower layer configuration (cell group configuration) for each of the cells in 1), that is a delta configuration with respect to the reference cell group configuration or is a full configuration (pending RAN2 progress).

For each accepted cell, the following information may be included:

- List of accepted and failed DRBs and SRBs for setup,

- List of accepted and failed SCells for setup.

Upon receiving the reference cell group configuration for the accepted candidate cells from a target DU, the CU can generate the overall reference configuration for the accepted candidate cells of that target DU.

The CU provides to the UE the RRC reconfiguration message containing the LTM configurations for the candidate target cells prepared by the candidate DU(s). The CU may provide the reference configuration(s) in the message and delta configurations with respect to the reference configuration(s) for the candidate target cells.

Proposal 9. A target DU or source DU may cancel a configured LTM candidate target cell, e.g., if it runs out of resources for the cell. The target DU or source DU provides an indication to the CU regarding this.

E1 aspects:

In case of CP UP separation, once CUCP receives LTM cell switch signling from (source)DU, CU CP initiates E1 bearer context modification including DL tunnel ID per DRB for target cell, and security keys corresponding to target cell (if updated) for data transmission.

The tentative flow chart to reflect above sentence is captured in section 3.6 in the Sod, to be continued based on this basis.

In the last RAN3 meeting (RAN3 #119 [1]), one E1 aspect was left FFS.

We think that since the scope of Rel-18 LTM is intra-CU, there should be no security key update at handover. CU-CP should send to the CU-UP DL TEID per DRB for the candidate target cell indicated in the notification received from the (source) DU during handover. This enables the CU-UP to begin DL data forwarding to the candidate DU corresponding to the candidate target cell.

Observation 2. Upon receiving the LTM cell switch signalling indication from the (source) DU, CU-CP does not indicate over E1-AP any security key update corresponding to the candidate target cell since in Rel-18 LTM there should be no security key update at handover.

Proposal 10. Upon receiving the LTM cell switch signalling indication from the (source) DU, CU-CP transmits to the CU-UP DL TEID per DRB for the candidate target cell included in the indication.

Proposal 11. It is FFS (pending RAN2 progress) whether gNB-CU transmits a UE Context Modification Request to indicate to the source and target gNB-DUs to keep resources associated with a subset of the candidate target cells or the source cell (the indication may also be to keep resources associated with all the candidate target cells).

Figure 2: Intra-CU, inter-DU LTM procedure

# 4 Other issues with potential RAN3 impacts

RAN1 sent an LS to RAN2, RAN3, and RAN4 [5] providing information about RAN1 agreements on measurements and early Timing Alignment (TA) determination for LTM. Early TA determination for LTM is to enable RACH-less cell switch for LTM.

RACH-less cell switch for LTM:

RACH-less cell switch has been proposed as a mechanism to reduce the latency in LTM handover.

RAN1 agreed on PDCCH ordered RACH for early TA determination of a candidate target cell. There are two options that RAN1 studied:

- Option 1: With UE configured to receive Random Access Response (RAR) on the candidate target cell in response to transmitting the RACH preamble, the RAR message containing the TA value.

- Option 2: With UE configured to not receive the RAR on the candidate target cell. In this case, the TA value of the candidate target cell is provided in the LTM cell switch command.

There are some related RAN3 impacts that can be identified:

- In inter-DU, intra-CU LTM, a candidate DU determines the RACH resources to be allocated for the candidate target cell(s), provides this information to the CU, and the CU provides the corresponding RACH configuration to the UE via the source DU, which initiates the procedure to obtain the RACH configuration in the first place. The procedure is similar for intra-DU LTM. There is F1-AP impact.

- In inter-DU, intra-CU LTM, each candidate DU determines the TA values for the candidate target cell(s) managed by it, and forwards these TA values to the source DU via the CU. All signalling takes place over F1. In intra-DU LTM, the DU determines the TA values for the candidate target cell(s) and can directly transmit the TA value in the LTM cell switch command. In intra-DU LTM, there seems to be no RAN3 impact.

The evaluation of whether Option 1 or Option 2 or both are suitable needs to be carried out in RAN2 first.

Observation 3. Regarding the approach to adopt (with or without RAR) for performing RACH for early TA determination in LTM needs to be decided by RAN2. RAN3 to wait on RAN2 progress before studying the related RAN3 impacts for both intra-DU and inter-DU, intra-CU LTM.

SSB-based L1 measurements on serving and candidate target cells for LTM:

RAN1 agreed on SSB-based L1 measurements on serving and candidate target cells for LTM.

For inter-DU, intra-CU LTM, during LTM preparation, each candidate DU needs to forward to the source DU via the CU the set of accepted candidate cells and the SSB transmission timing information for each accepted candidate cell (see Figure 3). Based on this information, the source DU can determine the configuration for L1 measurements of candidate target cells across candidate DUs (SMTC information) and reporting and provide it to the CU. The CU provides the configuration for L1 measurements and reporting to the UE via the source DU in the RRC reconfiguration message for LTM.

For intra-DU LTM, the DU determines the configuration for L1 measurements and reporting of the serving and candidate target cells for LTM and provides it to the CU. The CU provides the configuration for L1 measurements and reporting to the UE via the source DU in the RRC reconfiguration message for LTM.

Observation 4. RAN3 to wait on RAN2 progress on L1 measurement configuration for inter-DU, intra-CU and intra-DU LTM before studying the related RAN3 impacts.

Figure 3: F1-AP impact for L1 measurement configuration in inter-DU LTM

# 5 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Intra-DU LTM procedure

Proposal 1. The preparation of multiple candidate target cells for LTM can be performed using a single set of request/response messages UE Context Modification Request/UE Context Modification Response for lower latency, i.e., not using parallel preparation.

Proposal 2. The UE Context Modification Request message from the CU to the DU includes a list of suggested candidate cells for preparation for LTM.

For each candidate cell, the following information may be included:

- List of DRBs and SRBs to be set up,

- List of SCells to be set up.

Observation 1. The specification of the reference configuration(s) for LTM is up to RAN2.

Proposal 3. The UE Context Modification Response message from the DU to the CU includes:

- List of accepted candidate cells for LTM configuration,

- A lower layer reference configuration (reference cell group configuration) for the cells in 1) (pending RAN2 progress),

- Lower layer configuration (cell group configuration) for each of the cells in 1), that is a delta configuration with respect to the reference cell group configuration or is a full configuration (pending RAN2 progress).

For each accepted cell, the following information may be included:

- List of accepted and failed DRBs and SRBs for setup,

- List of accepted and failed SCells for setup.

Proposal 4. DU may cancel a configured LTM candidate target cell, e.g., if it runs out of resources for the cell. DU provides an indication to the CU regarding this.

Proposal 5. It is FFS (pending RAN2 progress) whether CU transmits a UE Context Modification Request to indicate to the DU to keep resources associated with a subset of the candidate target cells and the source cell (the indication may also be to keep resources associated with all the candidate target cells).

Intra-CU, inter-DU LTM procedure

Proposal 6. The preparation of multiple candidate target cells for LTM in a target DU can be performed using a single set of request/response messages UE Context Setup Request/UE Context Setup Response for lower latency, i.e., not using parallel preparation.

Proposal 7. The UE Context Setup Request message from the CU to a target DU includes a list of suggested candidate cells for preparation for LTM.

For each candidate cell, the following information may be included:

- List of DRBs and SRBs to be set up,

- List of SCells to be set up.

Proposal 8. The UE Context Setup Response message from the target DU to the CU includes:

- List of accepted candidate cells for LTM configuration,

- A lower layer reference configuration (reference cell group configuration) for the cells in 1) (pending RAN2 progress),

- Lower layer configuration (cell group configuration) for each of the cells in 1), that is a delta configuration with respect to the reference cell group configuration or is a full configuration (pending RAN2 progress).

For each accepted cell, the following information may be included:

- List of accepted and failed DRBs and SRBs for setup,

- List of accepted and failed SCells for setup.

Proposal 9. A target DU or source DU may cancel a configured LTM candidate target cell, e.g., if it runs out of resources for the cell. The target DU or source DU provides an indication to the CU regarding this.

Observation 2. Upon receiving the LTM cell switch signalling indication from the (source) DU, CU-CP does not indicate over E1-AP any security key update corresponding to the candidate target cell since in Rel-18 LTM there should be no security key update at handover.

Proposal 10. Upon receiving the LTM cell switch signalling indication from the (source) DU, CU-CP transmits to the CU-UP DL TEID per DRB for the candidate target cell included in the indication.

Proposal 11. It is FFS (pending RAN2 progress) whether gNB-CU transmits a UE Context Modification Request to indicate to the source and target gNB-DUs to keep resources associated with a subset of the candidate target cells or the source cell (the indication may also be to keep resources associated with all the candidate target cells).

Other issues with potential RAN3 impacts

Observation 3. Regarding the approach to adopt (with or without RAR) for performing RACH for early TA determination in LTM needs to be decided by RAN2. RAN3 to wait on RAN2 progress before studying the related RAN3 impacts for both intra-DU and inter-DU, intra-CU LTM.

Observation 4. RAN3 to wait on RAN2 progress on L1 measurement configuration for inter-DU, intra-CU and intra-DU LTM before studying the related RAN3 impacts.

# References

[1] Chair notes, RAN3 #119 meeting.

[2] Chair notes, RAN3 #118 meeting.

[3] Chair notes, RAN3 #117-bis-e meeting.

[4] Chair notes, RAN2 #119-bis-e meeting.

[5] R1-2302194	LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM.