3GPP TSG-RAN WG3 Meeting #R3-119bis	R3-231372

# Online, 17 April – 21 April, 2023

Agenda Item:	10.2.1

Source:	NEC

Title:	Discussion on related issue of inter-RAT SHR

Document for:	Discussion and agreement

1	Introduction

During the previous meeting, it has been agreed that

Inter-RAT SHR T310 and T312 related triggers are to be considered for inter-RAT SHR from NR to LTE. RAN3 thinks that at least the following parameters can be useful for optimizing inter-RAT successful handover from NR to LTE. LS RAN2 to confirm and request support. Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision. Source NR cell information Target LTE cell information Measurement results for source, target and neighbours Cause to indicate which inter-RAT SHR triggering condition was met UE location Information RAN3 thinks that all CHO related information in intra-NR SHR (e.g., time from CHO configuration to execution) is not applicable for inter-RAT SHR. Inter-RAT SHR WA: Support inter-RAT SHR from LTE to NR at least for T304 if no impact on LTE. WA: In R18, UE context retrieval in source Node is supported for inter-RAT SHR, the details are FFS. This WA does not limit to the forwarding mechanism. RAN3 #119 RAN3 should specify mechanisms on how to correlate NR SHR and LTE RLF Report in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE? The content of inter-SHR? ACCESS AND MOBILITY INDICATION procedure is adopted as SHR forwarding mechanism for inter-RAT SHR? RAN3 sees benefits to support inter-RAT SHR from LTE to NR for T304 trigger with no impact on LTE in Rel-18. Option 1: The receiving node forwards the inter-RAT SHR to source node, then source node forwards the inter-RAT SHR to target node Option 2: The receiving node forwards the inter-RAT SHR to target node, then target node forwards the inter-RAT SHR to source node Option 3: The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18. WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell . Further information from the UE is FFS depending on if and how we support correlation mechanism and UE context retrieval mechanism.

In this tdoc, we will further address those remaining issues for inter-RAT SHR

2	Discussion

Firstly, one of the remaining issues is if NR SHR container provide to the ng-eNB, then the node should be enhanced to support decode NR format report. Therefore, how to correlate NR SHR and LTE RLF report in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, should be specified.

Mainly there are three options can be considered:

Alt1- Correlation in target eNB, which is similar to the correlation mechanism as Rel-17 intra-NR SHR. However, the drawback is target eNB might not be able to correlate in case only one of the reports is retrieved since it does not know how long to hold a certain report just for correlation purpose.

Alt2- Correlation in source gNB, which is different correlation mechanism from Rel-17 intra-NR SHR so that might cause more discussion. In addition, even if UE reports source C-RNTI in SHR, it is still hard to correlate the source C-RNTI in SHR and target C-RNTI in RLF report.

Alt3- No correlation needed. Since in Rel-18, only T310/T312 is supported for the source NR node. Then the inter-RAT SHR from NR to LTE is mainly to optimize RLM/BFD configurations in the source NR node, so tight correlation is not needed between the NR node configuration with the RLF in target eNB node. Furthermore, some unnecessary enhancement for eNB node should be applied if to put the target C-RNTI inside the NR SHR container. Last but not least, unnecessary enhancement on LTE node would be applied since the target eNB should send the inter-RAT SHR report to source NR node.

Therefore, considering both alt-1 and alt-2 are not perfect solutions and further enhancement needs more effort to discuss, we do not think the correlation between NR SHR and LTE RLF report is needed.

No need to correlate NR SHR and LTE RLF report for either source gNB or target eNB.

Then for the content reported in inter-SHR from LTE to NR, we think currently at least source LTE cell and target NR cell should be included. For the source C-RNTI and target C-RNTI, no need to included since the correlation is not needed from our view.

The source LTE cell and target NR cell should be included in inter-SHR from LTE to NR.

No need to include further information from the UE since correlation mechanism is not needed.

3	Conclusion and Proposals

In this contribution, we have analysed the left issues for inter-RAT SHR, and the following proposals are given out:

Proposal1.	No need to correlate NR SHR and LTE RLF report for either source gNB or target eNB.

Proposal2.	The source LTE cell and target NR cell should be included in inter-SHR from LTE to NR.

Proposal3.	No need to include further information from the UE since correlation mechanism is not needed.

4	References