3GPP TSG-RAN WG3 #119bis-e	R3-231432

E-meeting, 17th April – 26th April 2023

Agenda Item:	12.2.1

Source:	Lenovo

Title: 	        Miscellaneous Xn interface issues

Document for:	Discussion and Approval

# 1	Introduction

In this paper, we try to discuss miscellaneous Xn interface issues left from previous RAN3 meetings.

# 2	Discussion

In the last RAN3 meeting, the following working assumption was made that procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”. To better reflect companies’ views according to the discussion and to make it crystal clear. It is suggested to further clarify that “data type agnostic” means it is upon the NG-RAN decision to use the associated data as inputs or outputs or feedback.

WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.

RAN3 confirms that procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, meaning it is upon the NG-RAN decision whether to consider the associated data as input or output or feedback.

According to the discussion so far, the agreed new Class 1 procedure AI/ML INFORMAITON REQUEST/RESPOPNSE (FFS naming) and new Class 2 procedure AI/ML INFORMATION UPDATE are used to request and collect the following information:

- Resource status prediction

- UE performance feedback

In our understanding, among which resource status prediction is kind of AI/ML specific information, while UE performance feedback can be used for non-AI/ML purpose as well. For example, a non AI/ML capable gNB can still request/collect UE performance feedback to optimize the network implementation using non AI/ML method.

Therefore, the naming of the new Class1 and Class 2 messages is better to be generic and not necessary to limit the use of it for AI/ML purpose only. The suggested naming is as following:

- AI/ML Information Reporting Initiation  Data Collection Initiation

  - AI/ML INFORMATION REQUEST  Data Collection Request

  - AI/ML INFORMATION RESPONSE  Data Collection Response

  - AI/ML INFORMATION FAILURE  Data Collection Failure

- AI/ML Information Reporting  Data Reporting

  - AI/ML INFORMATION UPDATE  Data Update

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information. The following information should be specified as a start point on the basis of TR37.817: - Predicted resource status information over Xn - UE performance (e.g, UL/DL throughput, packet delay, packet loss) Predicted Resource Status Information reported in the new procedure for AI/ML Related Information can be predicted radio resources, predicted number of active UEs, and predicted number of RRC Connections. The request in the new Class 1 procedure for initiating the reporting of AI/ML Related Information can include an ID assigned by the requesting NG-RAN node to request for reporting, which includes - the reporting parameters - list of cells to report - reporting periodicity The response in the new Class 1 procedure for initiating the reporting of AI/ML Related Information can include an ID assigned by the responding NG-RAN node which includes the confirmation on the reporting parameters requested. The message in the Class 2 procedure for Data Reporting of AI/ML Related Information can include the corresponding IDs assigned by the NG-RAN nodes, reports result. The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE performance feedback reporting. Introduce into the agreed new request message (AI/ML INFORMATION REQUEST, the name needs further discussion), an indication that UE performance feedback is provided after handover event. Whether the indication is in implicit or explicit way needs to be further discussed. Introduce a trigger indication in the HO request message to indicate that UE performance feedback is requested after HO completion. The details of indication need to be discussed. The agreed new class2 non-UE associated procedure (AI/ML INFORMATION UPDATE, which name is FFS) is used for UE performance feedback reporting.

The naming for the new Class 1 and Class 2 procedure/messages is modified to be more generic

  - AI/ML Information Reporting Initiation  Data Collection Initiation

  - AI/ML INFORMATION REQUEST  DATA COLLECTION REQUEST

  - AI/ML INFORMATION RESPONSE  DATA COLLECTION RESPONSE

  - AI/ML INFORMATION FAILURE  DATA COLLECTION FAILURE

  - AI/ML Information Reporting  Data Collection Reporting

  - AI/ML INFORMATION UPDATE  DATA COLLECTION UPDATE

Agree the TP in the Appendix

# 3	Conclusion

Based on the discussion above, we propose:

Proposal 1	RAN3 confirms that procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, meaning it is upon the NG-RAN decision whether to consider the associated data as input or output or feedback.

Proposal 2	The naming for the new Class 1 and Class 2 procedure/messages is modified to be more generic

a.	AI/ML Information Reporting Initiation  Data Collection Initiation

b.	AI/ML INFORMATION REQUEST  DATA COLLECTION REQUEST

c.	AI/ML INFORMATION RESPONSE  DATA COLLECTION RESPONSE

d.	AI/ML INFORMATION FAILURE  DATA COLLECTION FAILURE

e.	AI/ML Information Reporting  Data Collection Reporting

f.	AI/ML INFORMATION UPDATE  DATA COLLECTION UPDATE

Proposal 3	Agree the TP in the Appendix

# Appendix: TP to 38.423 BL CR

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

Editor’s note: FFS on the names of new introduced procedures and messages.

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

**************** skip unchanged part *******************

| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| --- | --- | --- | --- |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| Data Collection Reporting | DATA COLLECTION UPDATE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.AA Data Collection Reporting Initiation

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of data collection to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Collection Reporting Initiation , successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start data collection reporting and stop data collection reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested data collection reporting according to the parameters given in the request in case the Registration Request IE is set to "start"; or

-	shall stop all cells data collection reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	FFS

If the Registration Request IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics IE indicates cell specific data collection reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all or part of (exact details of if and how to support partial reporting are FFS) requested information, it shall initiate the data collection reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic data collection. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: Data Collection Reporting Initiation , unsuccessful operation

If all of (exact details of if and how to support partial reporting are FFS) the requested data collection reporting cannot be initiated, NG-RAN node2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.4.AA.4	Abnormal Conditions

FFS

### 8.4.BB	Data Collection Reporting (FFS on the name)

#### 8.4.BB.1	General

This procedure is initiated by an NG-RAN node to report data collection accepted by the NG-RAN node following a successful Data Collection Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be reported using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.BB.2	Successful Operation

Figure 8.4.11.2-1: Data Collection Reporting, successful operation

NG-RAN node2 shall report the accepted data collection in DATA COLLECTION UPDATE message. The accepted data collection is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.4.BB.3	Unsuccessful Operation

Not applicable.

#### 8.4.BB.4	Abnormal Conditions

Void

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.CC	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested data collection reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the Data collection is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Forth Bit = UE Performance FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.DD	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested data collection, for all or part of (exact details of if and how to support partial reporting are FFS)  the objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Reporting Characteristics (FFS) | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is able to report. FFS on the coding | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.EE	DATA COLLECTION FAILURE (FFS on the name)

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of (exact details of if and how to support partial reporting are FFS) the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.FF	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested data collection.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item (FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resources Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>FFS | O |  |  |  | - | - |
| UE Performance Result (FFS) | O |  | 9.2.3.Y |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>