3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231441

Online, 17th – 26th April, 2023

Agenda Item:	13.2

Source: 	Lenovo

Title: 	IAB-MT and IAB-DU migrate to different IAB-donors

Document for:	Discussion and decision

- Introduction

Based on the discussion on mobile IAB-node inter-donor migration, following agreements have been achieved in previous meetings.

RAN3 117e: As already supported in Rel17, a mobile IAB-MT and its co-located mobile IAB-DU may be served by different donor CUs. The mobile IAB donor that the co-located IAB-DU connects to may remain unchanged after the IAB-MT HO. RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT stays connected to the same donor before and after the mobile IAB-DU migration. RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT executes inter-donor migration. The mobile IAB-node may perform multiple consecutive partial migrations without inter-donor migration of its mobile IAB-DU. RAN3 117bis-e: mIAB-DU migration and mIAB-MT handover can be executed independently from each other. Details on the scenarios need to be further discussed. RAN3 118: WA: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs. This WA is subject to validation that the impact involved is affordable. RAN3 119: When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU to include the information of target logical mIAB-DU’s CU (e.g. IP address, gNB-ID). The IAB-node can inform the source logical mIAB-DU’s CU via F1AP about the successful F1 Setup with the target logical mIAB-DU’s CU, and it can include the IDs of the cells activated by the target logical mIAB-DU’s CU. The HO of UEs from the source logical mIAB-DU´s CU to the target logical mIAB-DU´s CU should happen after the completion of the F1 setup. When to trigger the HO is up to source logical mIAB-DU´s CU implementation. After all UEs have been handed over, the source logical mIAB-DU’s F1AP association can be released by the source logical mIAB-DU or by the source logical mIAB-DU’s CU.

In this contribution, general procedure for mobile IAB-node inter-donor migration is further discussed, especial for mobile IAB-MT and mobile IAB-DU migrate to different IAB-donors.

- Discussion

If mobile IAB-MT and mobile IAB-DU can be handed over/migrated to different IAB-donors, there have the following 6 basic cases based on the agreement that mobile IAB-MT handover and IAB-DU migration can be executed independently. In the discussion below, it’s assumed that Xn and IP connectivity are available between the source and target IAB-donors for mobile IAB-MT HO and mobile IAB-DU migration.

- Case 1: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor A

- Case 2: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor B

- Case 3: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor C

- Case 4: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor B

- Case 5: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor A

- Case 6: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor C

Case 1 and Case 2

Case 1 is the partial migration and Case 2 is the full revocation of partial migration triggered by non-F1-terminating IAB-donor, which have been already defined in R17 via XnAP Handover Preparation and IAB Transport Migration Management procedures.

Observation 1: Following migration cases have been already supported via the legacy XnAP Handover Preparation and IAB Transport Migration Management procedures.

- Case 1: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor A

- Case 2: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor B

Case 3

And for Case 3, since the mobile IAB-MT4 handovers from IAB-donor A to IAB-donor B as in Figure 1, then the F1 transport path for mobile IAB-DU4 needs to also switch from topology of IAB-donor A to topology of IAB-donor B. This migration case is also acknowledged during consecutive partial migrations.

Observation 2: Following migration case has been already acknowledged during consecutive partial migrations.

- Case 3: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor C

Figure 1: An example of Case 3

Case 4

In this case, before IAB-DU migration, the mobile IAB-node3 is a boundary IAB-node which terminates F1 connection to IAB-donor A and terminates RRC to IAB-donor B. During the IAB-DU migration, the second mobile IAB-DU can setup F1 connection to IAB-donor-CU B via the RRC terminating path which the same as the target path. Then, IAB-donor A initiates handover for all connected UEs from source cell(s) of first mobile IAB-DU to target cell(s) of second mobile IAB-DU and removes the source F1 connection finally. All these procedures have been supported with current agreements.

Observation 3: Following migration case is easily to be supported by legacy procedure.

- Case 4: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor B

Figure 2: An example of Case 4

Case 5

Before IAB-DU migration, both the IAB-MT and IAB-DU of the mobile IAB-node3 terminates at source IAB-donor-CU A. In order to realize the IAB-DU migration, the second mobile IAB-DU needs to setup F1 connection with target IAB-donor-CU B via the BH links of source IAB-donor A. Once F1 connection between second mobile IAB-DU and target IAB-donor-CU has been setup, following procedures (at least including UE HO and source F1 removal) are the same to the Case 4.

Figure 3: An example of Case 5

Case 6

Once F1 connection setup between second mobile IAB-DU and target IAB-donor-CU via the topology of another RRC-terminating IAB-donor has been supported, the target F1 connection in case 6 can be setup. And then source IAB-donor-CU A can perform UE HO for all connected UEs.

Figure 4: An example of Case 6

Observation 4: Following migration cases are supported if F1 connection can be setup between second mobile IAB-DU and target IAB-donor-CU via the topology of another RRC-terminating IAB-donor.

- Case 5: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor A

- Case 6: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor C

Proposal 1: In case of IAB-DU migration, to support F1 connection setup between second mobile IAB-DU and target IAB-donor-CU via the topology of another RRC-terminating IAB-donor.

As analysis above, all 6 migration cases need to be supported for the mobile IAB-node, then the WA in RAN3 118 shall be turned into agreement.

Proposal 2: Turn the following WA into agreement.

- WA: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs

# Conclusion

This contribution aims to discuss inter-donor migration for mobile IAB-node. And following observations and proposals are concluded.

Observation 1: Following migration cases have been already supported via the legacy XnAP Handover Preparation and IAB Transport Migration Management procedures.

- Case 1: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor A

- Case 2: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor B

Observation 2: Following migration case has been already acknowledged during consecutive partial migrations.

- Case 3: IAB-MT HOs from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor C

Observation 3: Following migration case is easily to be supported after trigger conditions have been defined for UE HO and source F1 removal.

- Case 4: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor B

Observation 4: Following migration cases are supported if F1 connection can be setup between second mobile IAB-DU and target IAB-donor-CU via the topology of another IAB-donor.

- Case 5: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor A

- Case 6: IAB-DU migrates from IAB-donor A to IAB-donor B and IAB-MT stays connected to IAB-donor C

Proposal 1: In case of IAB-DU migration, to support F1 connection setup between second mobile IAB-DU and target IAB-donor-CU via the topology of another IAB-donor.

Proposal 2: Turn the following WA into agreement.

- WA: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs

# Reference

- Chairman notes from RAN3 117 e-meeting to RAN3 119 meeting.