3GPP TSG-RAN WG3 #119bis-e	R3-231461

Online, 17th – 26th April 2023

Source:	CATT, CBN

Title:	Discussion on MRB PDCP count “wrap around” problem

Agenda Item:	9.2.2

Document for:	Discussion and decision

- Introduction

This discussion paper includes two parts:

The first part reiterates why the “MRB PDCP COUNT wrap around” problem—although considered possible to be handled by network implementation from RAN2’s perspective—cannot be handled either by gNB-CU-CP or by gNB-CU-UP implementation.

The second part lists three possible options to solve this problem, as well as their pros and cons.

- Discussion

  - Why it cannot be solved by implementation

This paper focuses on the issue PDCP count “wrap around”—it does not means that the PDCP count may indeed wrap around (TS 38.323 still prevents this), but what action a gNB should take in order to prevent the PDCP count of an MRB from incrementing beyond its maximum number, i.e. 2³² − 1, and halting data transmission permanently. This case should be handled since its drawback is fatal.

For unicast the solution is simple. Even if gNB-CU-CP/UP split architecture is used, the gNB-CU-CP may periodically perform KgNB update and add/release a DRB, in order to reset the PDCP count toward zero. It can be performed when the PDCP count is well below the maximum number, e.g.:

- Step 1: A DRB is established for a continuous service.

- Step 2: Now it’s in the small hours and after a lengthy delivery, the PDCP count reaches a value comparable with 2³² but still far from it, e.g. 2³²/4.

- Step 3: The gNB-CU-CP decides to “reset” its PDCP count in case (since the gNB-CU-CP doesn’t know how many packets the gNB-CU-UP has delivered). It sets up a new DRB with the PDCP count counting from zero. The result is, one packet received through NG-U is delivered with PDCP count = e.g. (2³²/4 – 1) through the old DRB, but the next packet is delivered with PDCP count = 0 through the new DRB.

This solution is fully up to implementation and thus we did not introduce any change on any spec. In fact it is also applicable for MRBs with PDCP counts not assigned according to MBS QFI SNs, e.g. MRBs for broadcast.

However, this solution is not applicable for the MRBs with PDCP counts assigned according to MBS QFI SNs—PDCP counts of such MRBs are never reset by any of these simple means. They just keep incrementing, no matter how many RRC reconfiguration procedures are performed:

- Step 1: An MRB is established for a continuous service (maybe for an everlasting public safety purpose).

- Step 2: Now it’s in the small hours and after a lengthy delivery, the PDCP count reaches a value comparable with 2³² but still far from it, e.g. 2³²/4.

- Step 3: [Useless behaviour] The gNB-CU-CP sets up a new MRB. The result is, one packet received through NG-U is delivered with PDCP count = e.g. (2³²/4 – 1) through the old MRB, and the next packet is delivered with PDCP count = 2³²/4 through the new MRB—the count isn’t reset at all!

This problem was acknowledged and ever discussed in RAN2 #118 meeting [1][2]. RAN2’s understanding was that it can be handled by proper “network implementation”:

[032] It is left to the network implementation for the prevention of the PDCP COUNT wrap-around of multicast MRB. No specification change is needed.

One method to solve this problem (as pointed out by a few companies in [1]) is to configure two MRBs to carry one QoS flow (if 1:1 mapping) or a set of flows (if N:1 mapping) simultaneously but temporarily, when the gNB finds that the PDCP count of the old MRB is near (2³² − 1). That is to say:

- Packets with PDCP counts up to (2³² − 1) are delivered (and retransmitted, if needed) through the old MRB;

- Packets with PDCP counts from zero on are delivered through the new MRB.

Since gNBs never provide flow-to-MRB mapping rules towards UEs, this method has no impact on RAN2 specs and was thus regarded as “network implementation” in RAN2. It has no impact on network interfaces either, as long as gNB-CU-CP/UP split architecture is not used.

However, this method is not feasible yet if gNB-CU-CP/UP split architecture is used. As long as the gNB-CU-CP cannot instruct the gNB-CU-UP to perform the abovementioned distinguishing between the packets with high count and low count, the “network implementation” cannot be performed.

Proposal 1: Confirm that the “PDCP “MRB PDCP COUNT wrap around” problem cannot be handled either by gNB-CU-CP or by gNB-CU-UP implementation.

  - Three possible solutions

Option 1:

The most straightforward solution is that, to enhance the E1AP spec to enable the gNB-CU-CP to instruct the gNB-CU-UP to perform the abovementioned distinguishing between the packets with high count and low count. The gNB-CU-UP should also report to the gNB-CU-CP when the PDCP count is near (2³² − 1), which is already supported in current E1AP spec.

This solution was ever criticised as it may cause out-of-sequence delivery. However we do not consider it as important, because this problem also exists for unicast and we have never tried to solve it.

The real drawback of this option, which has been pointed out as well, is the waste of one MRB ID. That is to say, NG-RAN should reserve one MRB ID in case of wrap-around handling.

Option 2:

Another solution, which was raised last meeting, aims to solve the problem by the collaboration of MB-UPF, MB-SMF gNB-CU-CP and gNB-CU-UP: the MB-SMF periodically instructs the MB-UPF and gNB to reset the MBS SN over N3mb, and then the gNB reset the PDCP COUNT over Uu accordingly. Since the MB-SMF does not know the flow-to-MRB mapping, it has to perform such operation in a relatively radical way, e.g. whenever there are near 2³² packets delivered over a given multicast session, it should reset the MBS SN over N3mb. Moreover, N4 interface should also be enhanced so that the MB-UPF can notify the MB-SMF that the number is near 2³².

The main drawback of this option is that, releasing and adding QoS flows will obviously disrupt packet delivery. Ideally it can be performed when there are temporarily no packets delivered over this session, but such opportunity may not be always available. Another drawback is that this option affects too many interfaces and requites coordination among 3GPP WGs.

Option 3:

The last solution—and the most fundamental—is to change the behaviour of UE. The trouble we encounter come from the limit in RAN2 that “the PDCP COUNT never wraps around”. This limit has two uses: one is to prevent PDCP security strings from reusing more than once (SA3 consider it risky), and the other is to simplify text description in TS 38.323.

But technically, such limit can be removed for MRBs. There is no PDCP security for MRBs now, and it not hard to adjust the text description in TS 38.323 to allow COUNT wrap around. RLC layer has SN as well. The text description for the RLC spec, i.e. TS 38.322, is also based on the SN, even though it often wrap around. Simply copying some sentence from TS 38.322 toward TS 38.323 can enable PDCP COUNT to warp around and solve the entire problem.

Nevertheless, the main drawback of this option is that, it theoretically reverses the UE behaviour.

Our opinion:

Each option is acceptable for us, but we still prefer Option 1 since its drawback is minor compared with the other two options.

Proposal 2: We propose RAN3 to adopt one solution among the three. Our preference is Option 1, i.e. by temporarily configuring one duplicating MRB.

- Conclusion

Proposal 1: Confirm that the “PDCP “MRB PDCP COUNT wrap around” problem cannot be handled either by gNB-CU-CP or by gNB-CU-UP implementation.

Proposal 2: We propose RAN3 to adopt one solution among the three. Our preference is Option 1, i.e. by temporarily configuring one duplicating MRB.

We drafted a CR for Option 1 accordingly.

- Reference

[1] R2-2206353; [AT118-e][032][MBS] PDCP (Xiaomi); Xiaomi.

[2] R2-2206587; Part 2 summary of [AT118-e][032][MBS] PDCP (Xiaomi); Xiaomi.

[3] R3-231462; Correction on MRB PDCP count “wrap around” problem; CATT, CBN.