3GPP TSG-RAN WG3 #119bis-e		                                   R3-23xxxx

Online, 17th – 26th April, 2023

Source:	CATT

Title:	Proposal on the way forward on assistant information from CN to

NG-RAN node

Agenda Item:	15.3

Document for:	Discussion and Decision

Introduction

In last RAN3 meeting, there was discussion on how to interpret the indication from CN to NG-RAN node of the MBS assistant information and no consensus was made. Before RAN3#119bis-e meeting, an offline discussion was made among interested companies. This contribution tries to summarize the offline discussion and provide the possible way forward.

Discussion

During the offline discussion, several aspects were raised and it is summarized as below.

System design principle(Function split between CN and NG-RAN)

View 1: CN should not aim at influencing RAN internal state transition and it could only provide traffic/service characteristics to NG-RAN.

The reasons listed by the proponents are as below:

System design principles, working procedures and TSG/WG responsibilities should be respected. It is up to RAN to decide whether a UE is released to RRC_INACTIVE and the 5GC only “sees” CM states. The information provided from 5GC to RAN can never aim at influencing RAN internal properties directly. It has to abstract traffic/service characteristics.

It is a clear and obvious contradiction that SA2’s decisions respect the RAN’s responsibility for determining RRC state while at the same time agree on a definition of the MBS Assistance Information, which only subject and sole content is the RRC state the UE should be configured with.

It is not proper if SA2 translates the traffic pattern directly into one specific RRC state: it is doing RAN's job and it is over-simplification. A frequent talker, based on the traffic pattern, does not mean it can not be released to RRC_INACTIVE, especially when RAN already had already defined a set of mechanisms for RRC_INACTIVE since Rel-15 e.g. lower CP/UP latency upon RRC connection resumption. (e.g., early data transmission/early measurement),- data inactivity monitoring in UP (e.g., a frequent talker won't be released in the first place).

View 2: SA2 is the right place to study the requirement from SA6/SA4 and refine them into 3GPP 5GS requirement.

The reasons listed by the proponents are as below:

The technical level decision taken by SA2 here is what is the most suitable delivery mode for a platoon leader, considering his missions, performance requirement, activity pattern, Push-to-talk enablement, during a mission critical adhoc group call. It is not the job of RAN3 to study the requirements concerning mission critical services from SA6/SA4 and refine them into 3GPP 5GS requirements, but it is the role of SA2.

Mission Critical services and Group Calls require that at least some basic requirement is commonly met by all gNBs, which is, in this case, that some users should be preferably set to RRC connected state. This is the way SA2 has defined the MBS assistance information necessary from 5GC to NG-RAN, and this is in the remit of SA2 to define mission critical performance requirement.

SA2 has actually compared the “preferential treatment” approach and the “connected state approach” and has decided to go for the second one. And RAN3 does not have the competence to revise the decision of SA2 whereby SA2 has spent several months during the study phase to analyse the SA6/SA4 requirements of mission critical services, compare the alternatives and come up with the decision.

On whether to follow the decision of SA2 or RAN3 could make our own decision

View1: SA2 has discussed for a long time and there is no objection in SA2, so RAN3 should follow the agreement of SA2

View 2: The final decision should be made by RAN3 ourselves and we should provide our feedback to SA2.

Also Chairlady remind us that in the LS R3-231030, we already said that the details of the IE need further discussion in RAN3. And the suggestion from Chairlady is that RAN3 decides the handling of this assistance Information takes full account of the actual situation of RAN node and proper behaviour (which needs to be discussed in Apr meeting), and send the LS to SA2 to tell them the agreement/consensus made by RAN3.Whether and how to update SA2 TR is up to SA2.

Proposal

To make progress, two possible options were suggested by chairlady during the offline as below:

Option 1: Describe this IE in IE semantic description like:

This IE indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception, e.g. NG-RAN node understands the UE is preferred to be kept in RRC connected when receiving the related MBS session data.

Option 2: Leave the semantic description of this IE absent, while only describe the NG-RAN node behaviour in the procedure text like:

If a XXX IE is contained in the XXX message, the NG-RAN node may use it to decide whether to send a UE to RRC Inactive state for multicast reception.

Proposal: It is proposed to take the summary of discussion and above options into account for further discussion on this issue in RAN3#119bis meeting.