3GPP TSG-RAN WG3 #119bis-e																	R3-231478

17th – 26th April 2023

Online

Agenda item:	18.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for BL CR 36.300) on data forwarding in X2-CHO with time-based trigger condition

Document for:	Discussion & decision

# 1 Introduction

Last RAN3 meeting agreed TS36.423 BL CR ([1]) to support X2-CHO with time-based trigger condition. This contribution clarifies the data forwarding in X2-CHO with time-based trigger condition.

# 2 Discussion

X2-CHO is described in TS36.300 (copied in below figure). Early data forwarding (Step 8a) is started just after the source eNB receive the RRCConnectionSetupComplete message from the UE (Step 8). Since the UE may execute the handover just after the UE send the RRCConnectionSetupComplete message, so the source eNB should start the early data forwarding as soon as source eNB receives the RRCConnectionSetupComplete message.

##### -- TS36.300 	10.1.2.1a.2	C-plane handling

Figure 10.1.2.1a-1: Intra-MME/Serving Gateway Conditional Handover

…

8.	The UE sends an RRCConnectionReconfigurationComplete message to the source eNB.

8a.	If early data forwarding is applied, the source eNB sends the EARLY STATUS TRANSFER message.

9.	The UE maintains connection with the source eNB after receiving CHO configuration, and starts evaluating the CHO execution condition(s) for the CHO candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source eNB, applies the stored corresponding configuration for that candidate cell and synchronises to that candidate cell.

---

In case X2-CHO with time-based trigger condition, the UE remains connected at least up to T1. The UE only execute the handover after T1. This means the source eNB can continue the UL/DL with the UE at least up to T1. If Early data forwarding is applied, the source eNB should only need to start the data forwarding at T1. This is different to current data forwarding in X2-CHO. We propose to capture this behaviour in Stage-2 TS36.300. For example,

--  TP for TS36.300

##### 10.1.2.1a.4	Data Forwarding

…

If early data forwarding is applied instead, the source eNB initiates data forwarding before the UE executes the handover, to a candidate target node of interest. When time-based trigger condition is used, the source NG-RAN node should start the data forwarding at the time configured to the UE. The behavior of early data forwarding for the Conditional Handover follows the same principles for DRBs configured with DAPS Handover in the intra-system handover as defined in 10.1.2.3.5.

--

Proposal 1: Capture in TS36.300 that early data forwarding should be started at T1 in X2-CHO with time-based trigger condition.

# 3	Conclusions

In this contribution, we briefly analyzed the source eNB behavior for data forwarding when time-based trigger condition is applied. Our proposals are:

Proposal 1: Capture in TS36.300 that early data forwarding should be started at T1 in X2-CHO with time-based trigger condition.

# References

- R3-230938, X2AP CR on IoT NTN Functionality (Huawei, CATT, Nokia, Nokia Shanghai Bell, Qualcomm, ZTE, CMCC, Ericsson, Samsung)

# Text Proposal for BL CR 36.300

##### 10.1.2.1a.4	Data Forwarding

If late data forwarding is applied, the source eNB initiates data forwarding once it knows which target eNB the UE has successfully accessed. In that case the behavior of the Conditional Handover data forwarding follows the same behavior as defined in 10.1.2.3.1 and 10.1.2.3.2 for the intra-system handover data forwarding for RLC-AM and RLC-UM bearers.

If early data forwarding is applied instead, the source eNB initiates data forwarding before the UE executes the handover, to a candidate target node of interest. When time-based trigger condition is used, the source NG-RAN node should start the data forwarding at the time configured to the UE. The behavior of early data forwarding for the Conditional Handover follows the same principles for DRBs configured with DAPS Handover in the intra-system handover as defined in 10.1.2.3.5.