3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231488

# Online, April 17th – 26th 2023

Agenda Item:	11.3

Source:	Ericsson

Title:	(TP for QoE BL CR for TS 38.300): QoE and RVQoE Measurements and Reporting in NR-DC Scenarios

Document for:	Agreement

# Introduction

In this paper we discuss the support for QoE and RVQoE measurements and reporting in NR-DC scenarios, based on the agreements and TBCs from the RAN3#119 and earlier meetings. A TP for TS 38.300 is provided in the Annex.

# Discussion

The main discussion areas are:

- Coordination of RRC IDs.

- QoE measurement reporting.

- RVQoE measurement reporting.

- RVQoE measurement configuration.

- The MN-SN coordination procedure.

## Coordination of RRC IDs

With respect to RRC ID coordination between the MN and the SN, the following was captured in the RAN3#119 meeting notes:

The MN is responsible for RRC ID allocation for m-based sessions configured by the MN or SN, and notifies the allocated RRC ID(s) to the SN.

FFS on whether a pool of RRC ID is splitted between MN and SN or whether it is per measurement.

Let’s assume that there is a fixed splitting of the RRC ID pool, for example, MN has a pool of RRC IDs from 1 to x and SN owns the RRC ID values from x+1 to 16. A drawback with such fixed splitting is that, if MN and SN would switch roles (there is no explicit switching, but rather PCell change followed by a PSCell change), the RRC IDs would need to be reconfigured, to remain compliant with the rule that MN has pool of RRC IDs from 1 to x and SN owns the RRC ID values from x+1 to 16.

Another problematic scenario is when the UE is reconfigured to single-connectivity and the MN may take over the QoE configurations the SN is responsible for. If the MN subsequently adds an SN, then the previous division of RRC IDs into a pool for the MN and the SN is no longer valid.

Furthermore, a rigid division into two pools of RRC IDs is less flexible in terms of how many QoE configurations each node can configure in the UE compared to when the RRC IDs are assigned on a case by case basis (i.e., per QoE measurement configuration).

Proposal 1-1: The MN and the SN can coordinate the RRC ID allocation for both s- and m-based QoE/RVQoE measurements to be configured at a UE, on a per-configuration basis.

RAN3 has previously agreed that the SN must get approval from the MN before it can configure the UE with an m-based QoE configuration. This approval is a natural opportunity for the MN to assign an RRC ID for the concerned m-based QoE configuration.

Proposal 1-2: When the MN approves that the SN configures the UE with a certain m-based QoE configuration, the MN assigns an RRC ID for this m-based QoE configuration.

## QoE measurement reporting

The following was also captured in the RAN3#119 meeting notes:

For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

- The MN shall send the configuration information to the UE, or

- The SN should send the configuration to the UE directly, or

- The SN should send the configuration information to the UE via the MN (inside a container).

With respect to the above agreement, upon measurement start, the UE should, by default, start the reporting to the node from which it received the measurement configuration. In other words, (at least) initially, the reports are delivered using the same path that was used to deliver the measurement configuration information to the UE, just in the opposite direction (e.g., in a container on the SRB4, via MN to the SN). As agreed earlier, the reporting leg can be changed during the application session.

Proposal 2-1: By default (i.e., until the reporting leg is changed), upon session start, the QoE/RVQoE reports are sent to the node that sent the QoE/RVQoE configuration to the UE, using the same path.

The following RAN3#117-bis-e agreements pertain to switching of the reporting leg:

QoE reports can be transmitted to either MN or SN and the reporting leg (MCG or SCG) can be changed during the application session.

In DC, the UE switches the reporting leg based on indication from network, FFS on implicit or explicit way.

RAN3 should discuss which node can command the UE to switch the reporting leg.

We think that explicit indication for switching of the reporting leg should be supported. One implicit leg switching indication discussed earlier is the setup/removal of an SRB. However, given that the UE may be configured to send the reports pertaining to certain configurations to the MN, and the reports pertaining to some other configurations to the SN (e.g., depending on which node carries the session to the UE), we conclude that SRB setup cannot be used as an implicit indication of leg switching. This is because the same SRB (e.g., SRB4) can be used to carry reports whose destination can be any one of the MN or the SN. We also think that we should respect the principle that each node “owns” the RRC configuration it provides to the UE, i.e., the MN cannot change the RRC configuration the SN has sent to the UE and (of course) vice versa. In line with this principle, another reason in favour of explicit indication is that, if, e.g., the SN “owns” a QoE configuration for the UE and receives the QoE reports via the MN, the MN de-configuring the SRB4 would de facto be tampering with the SN-owned configuration. De-configuring an SRB may indeed be an implicit indication of leg switching, but it does not need to be specified, since it can be realized by UE implementation.

Proposal 2-2: The network can explicitly instruct a UE in NR-DC to switch the reporting leg.

With respect to which node can command the UE to switch the reporting leg, we think that the node that currently receives the reports (directly or tunnelled in a container via the other node) should be able to request the switch from the peer node (e.g., if, for any reason, it is not able to receive the reports anymore). In addition, we think that the command can be sent to the UE by the MN, even if the reports are currently sent to the SN. This is needed, e.g., in case of SN RLF, where the MN is the only node with a working connection to the UE. The leg switch always needs to be approved by the node that is bound to start receiving the reports. Finally, we think that the leg switch should be per QoE and/or RVQoE configuration.

Proposal 2-3: The leg switching command is sent to the UE by the node that configured that specific QoE configuration.

Proposal 2-4: The leg switching always needs to be approved by the node that is bound to start receiving the reports.

The following RAN3#117-bis-e and RAN3#118 agreements are related to the scenario where, for m-based QoE measurements configured by the MN, the SN receives the measurement reports directly from the UE and forwards them to the MCE:

Turn into an agreement the WA stating that, if QoE reports are received by the SN, the SN can forward the QoE reports to MCE directly.

If a node has configured the UE with QoE measurements, and the other node is receiving the QoE reports from the UE and forwarding them directly to the MCE, then:

- The node that has configured the UE with QoE measurements should indicate the QoE reference to the node that receives the reports and forwards them directly to MCE.

When MN configures a UE with m-based QoE, it may indicate to SN: the QoE Reference, the MCE IP address. FFS for other information (e.g., RRC ID)

In our understanding, the MN needs to indicate to the SN the RRC ID, so that the SN can associate the RRC ID in the received reports with the QoE reference.

Proposal 2-5: When the MN configures a UE with an m-based QoE measurement, the MN can indicate to the SN the corresponding RRC ID (in addition to the QoE reference and the MCE IP address).

## RVQoE measurement reporting

We think that it should be possible to send the QoE reports and their corresponding RVQoE reports over different legs, for several reasons:

- The QoE and RVQoE reports can already be sent at different periodicities, i.e., in different messages.

- The RVQoE reporting is not paused at overload, as opposed to the QoE reporting.

- The node delivering the application session to the UE may change, which means that the appropriate recipient of RVQoE reports may change. Meanwhile, the recipient of QoE reports does not change, it is always the MCE.

- Being forced to send the RVQoE reports via the node that is not their ultimate recipient significantly adds to reporting latency.

Proposal 3-1: In NR-DC, the RVQoE reports can be sent over a different leg than the corresponding QoE reports (i.e., the QoE reports pertaining to the same QoE reference).

The RAN3#119 meeting notes state the following:

If a node receives an RVQoE report from a UE in NR-DC, and determines that the bearers for the application session are also or only provided by the peer node, this node can send the received RVQoE report to the peer node.

With respect to the above agreement, before forwarding any RVQoE reports to the node that is determined to be carrying the session, the node that received the RVQoE report should inquire the node that carries the session whether the node that carries the session is interested in receiving the RVQoE reports.

Proposal 3-2: The node that determined that its peer node provides the bearer(s) for a session should inquire the peer node whether the peer node is interested in receiving the RVQoE reports.

Alternatively, if all bearers for the session are carried by the SN, as explained above, it may be preferrable to send the reports to the SN directly. However, considering that the support of the SRB5 is optional, it should also be possible to tunnel the reports from the UE via the MN to the SN using an ULInformationTransferMRDC message on the SRB4.

Proposal 3-3: If the SN does not support or does not configure the SRB5, the RVQoE reports can be tunnelled on the SRB4 from the UE via the MN to the SN.

## RVQoE measurement configuration

The following was agreed at the RAN3#119 meeting:

Turn WA to agreement: SN can send an RVQoE configuration to the UE.

In case the SN does not support the SRB3, it should be possible to send the RVQoE configuration to the UE within a container via the MN using a DLInformationTransferMRDC message on the SRB1.

Proposal 4-1: The SN can send an RVQoE configuration to the UE either via tunnelling via the MN (i.e., encapsulated in SRB1) or directly to the UE (using the SRB3).

A related RAN3#119 agreement states:

To determine which node(s) provide the bearers carrying an application session, a node can configure RVQoE measurements at a UE in NR-DC:

- For the first RVQoE configuration, it is blindly configured by MN or SN.

- From the PDU session ID and QFI in the first RVQoE report this node determines which node(s) provide the bearer(s) associated to the corresponding application session.

- After the node determines which node(s) carry the session including bearer type change, the RVQoE configuration may be modified.

With respect to the above agreement, the key principle should be that the node that has initially configured a UE with an RVQoE configuration remains the owner of the RVQoE configuration until the configuration is released or until the node stops serving the UE. However, this node should modify both the RVQoE configuration and the reporting based on the preferences indicated by the node that serves the bearer(s) for the session.

Proposal 4-2: The node that has initially configured a UE with an RVQoE configuration remains the owner of the RVQoE configuration until the configuration is released or until the node stops serving the UE.

Proposal 4-3: The node owning an RVQoE configuration should modify both the RVQoE configuration and reporting based on the preferences of the node that provides the bearer(s) for the session.

Based on the above principle, the scenario when a node receiving an RVQoE report determines that the peer node provides bearer(s) for the application session, should be handled as follows:

- In line with Proposal 3-2, the node that determined that its peer node provides the bearer(s) for the session (the “old” node) should ask the node that carries the session whether the node that carries the session (the “new” node) is interested in receiving the RVQoE reports. The old node should also ask the new node to provide its preference with respect to the SRB (and tunnel, if applicable) to be used for receiving the RVQoE reports.

- With respect to “After the node determines which node(s) carry the session including bearer type change, the RVQoE configuration may be modified.”, in the same message, the old node should ask the new node to provide its preferred RVQoE configuration parameters so that the old node can update the RVQoE configuration at the UE.

  - The underlying principle is that the node(s) which provide bearers for the application session should participate in generating the RVQoE configuration.

- The new node responds, and the old node acts accordingly.

The above applies to both the initial RVQoE configuration and the bearer type change.

Proposal 4-4: When a node receiving an RVQoE report determines that the peer node provides the bearer(s) for the application session:

- The node asks the peer node whether the peer node is interested in receiving the RVQoE reports.

- The node asks the peer node which SRB (and tunnel, if applicable) should be used for delivering the RVQoE reports to the peer node.

- The node asks the peer node to provide the (updated) RVQoE configuration that the node should send to the UE.

## The MN-SN coordination procedure

The following agreement was captured in the RAN3#119 meeting notes:

If the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure.

The MN should inform the SN that a UE is configured with an m-based QoE measurement.

We think that the UE-associated XnAP MN-SN coordination procedure should be a newly defined procedure. Otherwise, RAN3 needs to enhance several existing NR-DC procedures, i.e.:

- S-NG-RAN node Addition Preparation.

- M-NG-RAN node initiated S-NG-RAN node Modification Preparation.

- S-NG-RAN node initiated S-NG-RAN node Modification.

- S-NG-RAN node initiated S-NG-RAN node Change.

- M-NG-RAN node initiated S-NG-RAN node Release.

- S-NG-RAN node initiated S-NG-RAN node Release.

Proposal 5-1: Define a new UE-associated MN-SN coordination procedure for QMC.

The following RAN3#117-bis-e agreement relates to the MN-SN coordination procedure:

The coordination between the MN and the SN should support at least the following (details to be further discussed):

- Initiation by either the MN or the SN for m-based QoE, by the MN for s-QoE.

- Coordination for configuring the UE.

- Coordination for establishing the SRB for receiving QoE/RVQoE reports.

- Indication about switching the reporting leg.

With respect to the blue TBC bullet above, the RAN3#119 agreement stating that “If the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure.” implies that the coordination procedure can be initiated by either the MN or the SN for m-based QoE, and only by the MN for s-QoE. This should be formalized by turning the above blue bullet into an agreement.

Proposal 5-2: The coordination procedure between the MN and the SN can be initiated by either the MN or the SN for m-based QoE, and by the MN for s-based QoE.

The coordination procedure needs to support additional aspects, with respect to those already agreed. For example:

- Coordination of RVQoE measurement configuration parameters.

  - This needs to be supported in case a node determines that only the peer node or both nodes provide the bearers for the application session.

- Coordination for sending the RVQoE measurement configuration to the UE.

  - This follows from the RAN3 agreements that either the MN or the SN can send the RVQoE configuration to the UE.

To facilitate tracking of agreements and writing of stage-2 and -3 text, it would be good to keep all the functionalities agreed for the MN-SN coordination procedure together. Hence, we provide a proposal that supersedes the above RAN3#117-bis-e agreement.

Proposal 5-3: The MN-SN coordination procedure should support:

- Coordination for configuring the UE.

- Coordination to determine and establish the SRB(s) used for receiving QoE/RVQoE reports.

- Coordination about switching the reporting leg.

- Coordination of RVQoE measurement configuration parameters.

- Coordination for sending the RVQoE measurement configuration to the UE.

- Coordination of the RRC ID for the measurements.

## TP for QoE BL CR for TS 38.300

A TP for QoE BL CR for TS 38.300 is provided in the Annex.

Proposal 6: Agree the TP for QoE BL CR for TS 38.300, provided in the Annex.

# Conclusion

This paper discusses the support for QoE and RVQoE measurements and reporting in NR-DC scenarios. The following is proposed:

Coordination of RRC IDs

Proposal 1-1: The MN and the SN can coordinate the RRC ID allocation for both s- and m-based QoE/RVQoE measurements to be configured at a UE, on a per-configuration basis.

Proposal 1-2: When the MN approves that the SN configures the UE with a certain m-based QoE configuration, the MN assigns an RRC ID for this m-based QoE configuration.

QoE measurement reporting

Proposal 2-1: By default (i.e., until the reporting leg is changed), upon session start, the QoE/RVQoE reports are sent to the node that sent the QoE/RVQoE configuration to the UE, using the same path.

Proposal 2-2: The network can explicitly instruct a UE in NR-DC to switch the reporting leg.

Proposal 2-3: The leg switching command is sent to the UE by the node that configured that specific QoE configuration.

Proposal 2-4: The leg switching always needs to be approved by the node that is bound to start receiving the reports.

Proposal 2-5: When the MN configures a UE with an m-based QoE measurement, the MN can indicate to the SN the corresponding RRC ID (in addition to the QoE reference and the MCE IP address).

RVQoE measurement reporting

Proposal 3-1: In NR-DC, the RVQoE reports can be sent over a different leg than the corresponding QoE reports (i.e., the QoE reports pertaining to the same QoE reference).

Proposal 3-2: The node that determined that its peer node provides the bearer(s) for a session should inquire the peer node whether the peer node is interested in receiving the RVQoE reports.

Proposal 3-3: If the SN does not support or does not configure the SRB5, the RVQoE reports can be tunnelled on the SRB4 from the UE via the MN to the SN.

RVQoE measurement configuration

Proposal 4-1: The SN can send an RVQoE configuration to the UE either via tunnelling via the MN (i.e., encapsulated in SRB1) or directly to the UE (using the SRB3).

Proposal 4-2: The node that has initially configured a UE with an RVQoE configuration remains the owner of the RVQoE configuration until the configuration is released or until the node stops serving the UE.

Proposal 4-3: The node owning an RVQoE configuration should modify both the RVQoE configuration and reporting based on the preferences of the node that provides the bearer(s) for the session.

Proposal 4-4: When a node receiving an RVQoE report determines that the peer node provides the bearer(s) for the application session:

- The node asks the peer node whether the peer node is interested in receiving the RVQoE reports.

- The node asks the peer node which SRB (and tunnel, if applicable) should be used for delivering the RVQoE reports to the peer node.

- The node asks the peer node to provide the (updated) RVQoE configuration that the node should send to the UE.

MN-SN coordination procedure

Proposal 5-1: Define a new UE-associated MN-SN coordination procedure for QMC.

Proposal 5-2: The coordination procedure between the MN and the SN can be initiated by either the MN or the SN for m-based QoE, and by the MN for s-based QoE.

Proposal 5-3: The MN-SN coordination procedure should support:

- Coordination for configuring the UE.

- Coordination to determine and establish the SRB(s) used for receiving QoE/RVQoE reports.

- Coordination about switching the reporting leg.

- Coordination of RVQoE measurement configuration parameters.

- Coordination for sending the RVQoE measurement configuration to the UE.

- Coordination of the RRC ID for the measurements.

TP for QoE BL CR for TS 38.300

Proposal 6: Agree the TP for QoE BL CR for TS 38.300, provided in the Annex.

# Annex: TP for QoE BL CR for TS 38.300

-------------------------------------------Start of changes-------------------------------------------

## 21.x	Support for QoE and RAN visible QoE measurements in NR-DC

### 21.x.1	The MN-SN coordination for QoE/RAN visible QoE measurements and reporting

For a UE in NR-DC, the MN and the SN can coordinate for configuring the UE with QoE and RAN visible QoE measurements and reporting. The coordination procedure pertains to both signalling-based and management-based QoE/RAN visible QoE measurements and reporting. The MN-SN coordination procedure supports:

- Coordination for configuring the UE with QoE and RAN visible QoE measurements.

- Coordination for determining and establishing the SRB(s) used for receiving QoE/RAN visible QoE reports.

- Coordination about switching the reporting leg.

- Coordination of RAN visible QoE measurement configuration parameters.

- Coordination for sending the RAN visible QoE measurement configuration to the UE.

- Coordination of the measConfigAppLayerId for the measurements.

If the SN is interested in configuring a UE with a management-based QoE measurement configuration, it should send the coordination request to the MN. The MN should inform the SN that a UE is configured with a management-based QoE/RAN visible QoE measurement.

In case of management-based QoE, the MN decides which node to perform the QoE measurement configuration. For a management-based QoE/RAN visible QoE configuration in case the SN is interested in configuring a UE with a management-based QoE/RAN visible QoE measurement configuration, the MN can decide and notify the SN whether:

- The MN sends the configuration information to the UE, or

- The SN should send the configuration to the UE directly, or

- The SN should send the configuration information to the UE via the MN (inside a container on SRB1).

The MN is responsible for measConfigAppLayerId allocation for management-based sessions configured by the MN or by the SN, and the MN indicates the allocated measConfigAppLayerId(s) to the SN, on a per-configuration basis.

If the MN or the SN has configured the UE with QoE measurements, and the UE is configured to send the QoE reports to the peer node (the SN or the MN), which forwards the reports directly to the MCE, then the node that has configured the UE with QoE measurements should indicate the QoE reference, the MCE IP address and the measConfigAppLayerId to the node that receives the reports and forwards them directly to the MCE.

QoE reports can be transmitted to either the MN or the SN, and the reporting leg (MCG or SCG) can be changed during the application session.

### 21.x.2	Support for RAN visible QoE measurements and reporting in NR-DC

Subject to MN-SN coordination, either the MN or the SN can generate and send a RAN visible QoE configuration to the UE and receive the RAN visible QoE reports directly from the UE. In addition, the UE can send a RAN visible QoE report to the MN or the SN, and the MN or the SN can forward it to the peer node (the SN or the MN).

The MN or the SN can configure RAN visible QoE measurements at a UE, without a priori knowledge about which node(s) provide the bearer(s) for the application session. To ensure that the RAN visible QoE reports are sent to the node(s) that provide the bearer(s) associated to the corresponding RAN visible QoE measurement result in the RAN visible QoE report, the RAN needs to determine which node(s) provides these bearer(s). From the PDU session ID and the QFI indicated in the received RAN visible QoE report the node determines which node(s) provide the bearer(s) associated to the corresponding application session. If a node receives an RAN visible QoE report from a UE in NR-DC and determines that the bearers for the application session are also or only provided by the peer node, this node can send the received RAN visible QoE report to the peer node. In this case, the RAN visible QoE configuration can be modified. The above is applied both at the beginning of RAN visible QoE measurement session, and at every bearer type change.

-------------------------------------------End of changes-------------------------------------------