3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231522

# Online, 17 April – 26 April 2023

Agenda item:	13.1

Source:	Xiaomi

Title:	Discussion on support of MBSR

Document for:	Discussion and Decision

1	Introduction

In RAN3 119 meeting, RAN3 agreed to further discuss point#6 (regarding to KI#5) in the Reply LS to SA2 [1].

For point#6 (regarding KI#5): RAN3 needs to conduct further discussions to converge on a solution. RAN3 will inform SA2 for any progress.

In this contribution, we’d like to share our understandings on the stage2 procedures in [2], and discuss the stage 3 signalling design for NRPPa/F1AP, and the potential issues based on the understanding of SA2’s conclusion.

2	Discussion

2.1	Scenario clarification

In this chapter, we’d like to share our understanding on the stage2 procedure for MT-LR procedure when a MBSR is involved in the location of a UE as agreed in SA2, which may help us to understand the purposes of NRPPa/F1AP enhancements.

In Figure 1, we added some implicit information in blue text based on the SA2’s procedure in [2] and coclusion in [3] to help RAN3 understanding.

In stpe 0, [legacy behaviour]Before UE positioning, the LMF obtains multiple TRP infomraiton from different gNBs via TRP infomraiton exchange procedure to help LMF make positioning decision for UE positioning, e.g. which positioning methods are used, which TRPs are selected for UE positioning.

[new behaviour] If the TRP is a mobile TRP (i.e. the TRP is proveided by a IAB-node), LMF needs to be awrae of it, so that LMF can take further actions to obtain the real-time lcoation of the mobile TRP, thus, we can conclude that the 1st purpose to enhance TRP informaiton exchange procedure is that LMF needs to be aware of whether the TRP is mobile or not before UE positioning.

In step 1-3, the request of UE positioning is triggered.

In step 4, [legacy behaviour] LMF decides the positioning methods and the involed TRPs based on the previous obtained infomraiton in step 0 and some history infomration, etc.

[new behanivor] LMF can learn that whether the selected TRP is mobile or not based on the infomraiton obtained in step 0. If the selected TRP for UE positioning belongs to a mobile IAB-node, the LMF can perform etiher option 1 or option 2 to obatin the actual locaiton of the TRP. Please note that serval TRPs can be selected for UE positioning, the selected TRP can be the TRP hosted by the UE’s serving cell or non-serving cell.

In step 5-7, option 1 is performed, the IAB-MT will perform MO-LR procedure to obtain the location and velocity of itself and report them to LMF via TRP information response, this indicates the 2nd purpose to enhance TRP inomration exchange proceudre is to trigger MO-LR procedure of mobile IAB-MT associated with the TRP, and obtian the location of the mobile TRP during UE positioning.

In step 8-10, option 2 is performed, the LMF needs to obatin the location of the IAB-MT via MT-LR procedure, according to the UE ID of the IAB-MT associated with the selected TRP, and this indicates that the 3rd purpose to enhance TRP inomration exchange proceudre is that LMF needs to be aware of the UE ID of IAB-MT associated with the TRP before UE positioning.

In step 11, The LMF performs legacy positioning procedures with the target UE. If Network Assisted procedure (i.e. UL related positioning) is used, as in 6.11.2, the NG-RAN may provide the mobile TRP’s updated location and velocity information to the LMF in the TRP measurement result so that the LMF can calculate the correct UE location based on the actual location of the mobile TRP when it performs the measurement, and this indicates the purpose to enhance TRP measurement result is that LMF needs to be aware of the updated locaiton of the TRP at mobile IAB-node when positioning measurement is performed.

In our understanding, option 1, option 2 and step 11 are different ways to obtain the real time location information of a mobile TRP during UE positioning, so that the LMF can derives UE location based on the actual location of the mobile TRP, which way to be used is up to LMF decision and implementation. From specification point of view, all the options that may be selected by LMF need to be supported.

Based on the above discussion, we have the following observations:

Observation 1, LMF needs to obtain the real time location of the TRP hosted by a mobile IAB-node to derive UE location.

Observation 2, several TRPs can be selected for UE positioning, which may or may not be the TRP hosted by the serving cell of the target UE.

Observation 3, LMF needs to be aware of whether the selected TRP for UE positioning is mobile or not before UE positioning.

Observation 4. LMF needs to be aware of the IAB-MT UE ID associated to a specific TRP before UE positioning.

Observation 5, LMF can obtain the real time location of the selected TRP at mobile IAB-node via the following ways:

- Trigger MO-LR procedure of the IAB-MT associated with the TRP and obtain the result via TRP information exchange procedure (i.e. option 1 in stage 2 procedure)

- Trigger MT-LR procedure of the IAB-MT associated with TRP and obtains the result by existing mechanism, but the LMF needs to be aware of the UE ID of IAB-MT in advance (i.e. option 2 in stage 2 procedure)

- Obtain the location of the mobile TRP via RAN Assisted positioning procedure if the corresponding positioning method is used. (i.e. step 11 in stage 2 procedure)

Observation 6, the following purposes are summarized in general to support UE positioning when IAB-node (i.e. MBSR) is involved

- LMF needs know whether the TRP is mobile or not via TRP information exchange procedure;

- LMF can trigger MO-LR procedure of mobile IAB-MT associated with the TRP, and obtain the result (i.e. location and velocity) via TRP information exchange procedure;

- LMF can obtains the UE ID of IAB-MT associated with the TRP via TRP information exchange procedure;

- LMF can obtains updated location of the mobile TRP (i.e. real time location and velocity information) via TRP measurement result information.

2.2	NRPPa/F1AP signalling design

Based on the above observations in chapter 2.1, we think the following information needs to be introduced in NRPPa/F1AP:

- Mobile TRP type

- Mobile IAB-MT UE ID

- Location information

- Velocity information

- Timestamp

For mobile TRP type, it can be introduced as a new TRP type in TRP information response message.

For mobile IAB-MT UE ID, GPSI as it’s already agreed by SA2, the IE type can refer to TS 29.571, it can be introduced in TRP information response message if available.

For location, velocity and time stamp information, according to the stage 2 procedure, they are related to option 1, and generated after MO-LR procedure, so we think the IE should refer to LPP spec, as it’s the measurement and calculation result by a UE (i.e. IAB-MT), according to step 5-7 in Figure 1. And the three kinds of information are for the same purpose, so a general IE e.g. mobile TRP location information can be introduced to include all the related information. This mobile TRP location information IE can be included in TRP information response message, measurement response message and measurement report message.

If we introduce a new TRP information type (i.e. mobile TRP location information), to follow the principle of NRPPa design, i.e. LMF is in control of the requested information that must be reported back from the TRPs, a new TRP information type “e.g. mbsr location info” can be introduced in TRP information request, and for UL related measurement, it would be good to introduce a new code in Measurement Characteristics Request Indicator IE, which can avoid to introduce additional new IE for the request and has no change on the procedural text.

Proposal 1, RAN3 agree to introduce MBSR as new TRP type.

Proposal 2, RAN3 agree to include Mobile IAB-MT UE ID in TRP information response message if the TRP is MBSR over NRPPa and F1AP

Proposal 3 RAN3 agree to include Mobile TRP Location Information (including Location, velocity and time stamp obtained by MO-LR) as a new TRP information type in TRP information response message over NRPPa and F1AP.

Proposal 4, RAN3 agree to include Mobile TRP Location Information in TRP Measurement Result IE over NRPPa and in Positioning Measurement Result IE over F1AP.

2.3	Potential issue in case of DU migration

We also observed one potential issue which may affect the proper execution of the whole procedure. Currently, LMF decide positioning methods and the TRPs for the selected methods based on the TRP information obtained via TRP information exchange procedure. And a specific TRP can be identified by a NG-RAN ID and a TRP ID, according to the definition as below in TS 38.455.

9.2.24	TRP ID

The TRP ID IE is used to identify a TRP uniquely within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Identifier | M |  | INTEGER (1..65535,…) | Identifies a TRP within an NG-RAN node |

The legacy TRP information exchange procedure can only work very well if the TRP is static and the TRP information may not be changed frequently. However, if the TRP is hosted by a mobile IAB-node, after inter-donor IAB-DU migration, the mobile IAB-node will migrate from the old IAB-donor-CU to a new IAB-donor-CU, the gNB ID and the TRP ID (which is assigned by the new IAB-donor-CU) to identify this mobile TRP are changed, the corresponding information should be updated to LMF, so that the LMF can make positioning decisions based on the latest information of the TRP.

In SA2’s CR [2], an OAM solution for the issue is introduced as below:

When the MBSR is integrated or fully migrated to a new gNB, the OAM triggers the LMF to perform TRP Information Exchange procedure. The LMF learns that a new integrated TRP at a gNB is mobile and its MBSR IAB UE ID (GPSI) via a TRP information exchange towards the gNB with the Cell ID of the TRP.

On the other hand, there is an Editor Note in [2], which have doubts on the OAM solution.

Editor's note: The trigger of LMF to initiate the TRP information exchange needs further discussion for the case of MBSR’s inter-CU mobility.

In our view, the OAM solution may not work in time, another way may be the UE AMF can notify the LMF if the DU migration happens, but this needs to check with SA2, whether the UE AMF can proactively to update the information is UE is not involved in location service. So, we’d like to propose RAN3 to discuss any possible solutions to address this issue.

Proposal 5, RAN3 to discuss how to let the LMF know the update mobile TRP information in time after inter-donor DU migration.

3	Conclusion

In this contribution, we discussed our understanding and potential RAN3 impacts on the mobile IAB-node is involved in the location of a UE, and we had the following observations and proposals:

Observation 1, LMF needs to obtain the real time location of the TRP hosted by a mobile IAB-node to derive UE location.

Observation 2, several TRPs can be selected for UE positioning, which may or may not be the TRP hosted by the serving cell of the target UE.

Observation 3, LMF needs to be aware of whether the selected TRP for UE positioning is mobile or not before UE positioning.

Observation 4. LMF needs to be aware of the IAB-MT UE ID associated to a specific TRP before UE positioning.

Observation 5, LMF can obtain the real time location of the selected TRP at mobile IAB-node via the following ways:

- Trigger MO-LR procedure of the IAB-MT associated with the TRP and obtain the result via TRP information exchange procedure (i.e. option 1 in stage 2 procedure)

- Trigger MT-LR procedure of the IAB-MT associated with TRP and obtains the result by existing mechanism, but the LMF needs to be aware of the UE ID of IAB-MT in advance (i.e. option 2 in stage 2 procedure)

- Obtain the location of the mobile TRP via RAN Assisted positioning procedure if the corresponding positioning method is used. (i.e. step 11 in stage 2 procedure)

Observation 6, the following purposes are summarized in general to support UE positioning when IAB-node (i.e. MBSR) is involved

- LMF needs know whether the TRP is mobile or not via TRP information exchange procedure;

- LMF can trigger MO-LR procedure of mobile IAB-MT associated with the TRP, and obtain the result (i.e. location and velocity) via TRP information exchange procedure;

- LMF can obtains the UE ID of IAB-MT associated with the TRP via TRP information exchange procedure;

- LMF can obtains updated location of the mobile TRP (i.e. real time location and velocity information) via TRP measurement result information.

Proposal 1, RAN3 agree to introduce MBSR as new TRP type.

Proposal 2, RAN3 agree to include Mobile IAB-MT UE ID in TRP information response message if the TRP is MBSR over NRPPa and F1AP

Proposal 3 RAN3 agree to include Mobile TRP Location Information (including Location, velocity and time stamp obtained by MO-LR) as a new TRP information type in TRP information response message over NRPPa and F1AP.

Proposal 4, RAN3 agree to include Mobile TRP Location Information in TRP Measurement Result IE over NRPPa and in Positioning Measurement Result IE over F1AP.

Proposal 5, RAN3 to discuss how to let the LMF know the update mobile TRP information in time after inter-donor DU migration.

4	References

[1] R3-231011, LS R3-230032 (S2-2211437) on FS_VMR solutions review

[2] S2-2301478 MT-LR procedure for when a MBSR is involved in the location of a UE

[3] TR 23.700-05-i00, Study on architecture enhancements for vehicle-mounted relays