3GPP TSG-RAN WG3 #119bis                                                                                         R3-231559

Online, 17th – 26th April, 2023

Agenda Item:	16.4

Source:	CATT

Title:	Discussion on Multi-path Support for SL relay

Document for:	Discussion

- Introduction

In previous meetings, there are some agreements have been achieved.

Add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401. FFS on other procedures.

Turn the following WAs into agreement:

WA: The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision.

WA: For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB.

WA: The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition.

In this contribution, we discuss the RAN3 impacts for multi-path for Scenario 1.

- Discussion

2.1 Scenario 1

- A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

- B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

For case A and case B, there is one left issue about how to trigger RRC_IDLE/ INACTIVE target relay UE into RRC-CONNECT state.

Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress.

In RAN2#121 [1], there are two solutions were agreed as below. If split SRB1 is configured, the RRC reconfiguration complete message can be sent via indirect path. Otherwise, a PC5-RRC triggers is sent from remote UE to target relay UE.

Agreement:

UP-based approach (excluding SL-RLC1) in Option 1 is excluded for relay UE to enter RRC_CONNECTED.

Agreements:

For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case.

TS38.401 can be update to include such agreement that reflecting how to trigger RRC_IDLE/ INACTIVE target relay UE into RRC-CONNECT state.

Proposal 1: Remove the above Editor’s note in BL CR. It is proposed to add the sentence e.g., If split SRB1 is configured, the RRC reconfiguration complete message can be sent via indirect path to trigger RRC_IDLE/ INACTIVE target relay UE into RRC-CONNECT state. Otherwise, a PC5-RRC trigger sending from remote UE to target relay UE is needed. TP can be found in Annex1.

Editor’s Note: FFS whether the remote UE sends the RRCReconfigurationComplete message to the gNB-CU via old path (direct path) or new path (indirect path).

If no spilt SRB1 configured to remote UE, the RRC reconfiguration complete message is sent via the primary path which configured with SRB1. The SRB1 is configured on primary RLC entity i.e. primary path follows the legacy principle, no specific description is needed. If split SRB1 is configured, it can send via both path.

Proposal 2: Remove the above Editor’s note in BL CR. And no further description is needed. TP can be found in Annex 1.

- G.	The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

Agreement:

Change of indirect path while keeping the direct path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

Both target relay UE and source relay UE under the same gNB-DU and the different gNB-DU should be considered. The dashed line is used to trigger RRC_IDLE/ INACTIVE target relay UE into RRC-CONNECT state in case RRC reconfiguration complete message is sent via direct path.

The procedure that remote UE changes a new relay UE for inter-DU/intra-DU indirect path is captured in Figure 1 and Figure 2 as below.

Figure 1 Remote UE Changes to a new relay UE for inter-DU indirect path

Figure 2 Remote UE Changes to a new relay UE for intra-DU indirect path

Observation 1: A dashed line sending from remote UE to target relay UE for triggering RRC_IDLE/ INACTIVE target relay UE into RRC-CONNECT state is needed in case no split SRB1 is configured.

Proposal 3: The above procedure that remote UE changes to a new relay UE for the inter-DU/intra-DU indirect path while keeping the direct path under the same gNB can be captured in TS38.401, as baseline.

For the multi-path support, the gNB-CU takes the responsibility to decide the addition/modification/release of the path.

The RAN3 waits for the RAN2 progress on whether the gNB-DU knows the path information of each configured path.

GNB-CU takes the responsibility to decide the addition/modification/release of the path. For intra-DU multi path, the path information should be indicated to DU. There are some cases listed as below:

Addition/modification case: CU1 decides to add intra-DU indirect/direct path or modify one path under the DU, it will send UE context modification request to DU. However, DU may not know whether to add a new path to support multi path or modify the current direct/indirect path to indirect/direct path.

Release case: CU1 decides to release indirect path, it will send UE context release to DU. However, when DU receives UE context release message, DU may not know which path should be release.

Observation 2: DU should know the path information of each configured path.

It can be found that the path information is needed for multi path scenario but RAN2 have not discuss this issue for a long time because they do not see any requirement to introduce path information for multi path. The efficient solution is that RAN3 makes the decision about the definition of path information.

There are two options to define path information:

- Option 1: relay UE L2ID

- Option 2: path type: direct or indirect

We slightly prefer option 1 because it would be clearer and further proof in case multi path supports two indirect paths in the further.

Proposal 4: CU indicates relay UE L2ID in UE context modification request/UE context release. TP for 38.373 is in Annex2.

- Conclusion

Proposal 1: Remove the above Editor’s note in BL CR. It is proposed to add the sentence e.g., If split SRB1 is configured, the RRC reconfiguration complete message can be sent via indirect path to trigger RRC_IDLE/ INACTIVE target relay UE into RRC-CONNECT state. Otherwise, a PC5-RRC trigger sending from remote UE to target relay UE is needed. TP can be found in Annex1.

Proposal 2: Remove the above Editor’s note in BL CR. And no further description is needed. TP can be found in Annex 1.

Observation 1: A dashed line sending from remote UE to target relay UE for triggering RRC_IDLE/ INACTIVE target relay UE into RRC-CONNECT state is needed in case no split SRB1 is configured.

Proposal 3: The above procedure that remote UE changes to a new relay UE for the inter-DU/intra-DU indirect path while keeping the direct path under the same gNB can be captured in TS38.401, as baseline.

Observation 2: DU should know the path information of each configured path.

Proposal 4: CU indicates relay UE L2ID in UE context modification request/UE context release. TP for 38.373 is in Annex2.

- Reference

[1] RAN2#112, Chair notes.

- Annex1: TP for 38.401

  - 8.xx Overall procedures for multi-path support

    - 8.xx.1 Inter-DU direct path addition on top of indirect path

The signalling flow for inter-DU direct path addition is shown in Figure 8.xx.1-1.

Figure 8.xx.1-1: Signalling procedure of inter-DU direct path addition on top of indirect path

- The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the direct path to remote UE under a different gNB-DU (i.e., gNB-DU1).

3.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU1, which contains at least the direct path configuration.

4.	The gNB-DU1 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

5.	The gNB-CU sends an RRCReconfiguration message to the relay UE to update the indirect path configuration if necessary.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for remote UE by including the RRCReconfiguration message to the gNB-DU2. The contents in the RRCReconfiguration message may include at least direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).

7.	The gNB-DU2 sends the RRCReconfiguration message to the remote UE.

8.	The gNB-DU2 sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The remote UE performs random access procedure at the gNB-DU1.

10.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU2.

11.	The gNB-DU2 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

    - 8.xx.2 Inter-DU indirect path addition on top of direct path

The signalling flow for inter-DU indirect path addition is shown in Figure 8.xx.2-1.

Figure 8.xx.2-1 Signalling procedure of inter-DU indirect path addition on top of direct path

1. The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under a different gNB-DU (i.e., gNB-DU2).

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU2 and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU2, which contains the indirect path configuration at least.

5.	The gNB-DU2 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU1. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s).

7.	gNB-DU1 sends the RRCReconfiguration message to the remote UE.

8.	The remote UE establishes PC5 connection with relay UE.

9.	The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU1.  In case target relay UE in RRC_IDLE/ INACTIVE state, if split SRB1 is configured, the RRCreconfigurationcomplete message can be sent via indirect path to trigger RRC_IDLE/ INACTIVE target relay UE into RRC-CONNECT state. Otherwise, a PC5-RRC trigger sends from remote UE to target relay UE.

10.	The gNB-DU1 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

- Annex2: TP for 38.473

8.3.4	UE Context Modification (gNB-CU initiated)

8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

<<<Unchanged part is skipped>>>

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the Relay UE L2ID IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as an indirect path addition procedure or indirect path modification procedure for SL multi-path scenario.

8.3.3	UE Context Release (gNB-CU initiated)

8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

<<<Unchanged part is skipped>>>

If the Positioning Context Reservation Indication IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall not release the positioning context including the SRS configuration for the UE.

If the Relay UE L2ID IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider it as an indirect path release procedure for multi-path scenario.

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT SRB. | YES | reject |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | YES | reject |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 |  | YES | ignore |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >>RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| >>SDT Indicator Modify | O |  | ENUMTERATED (true, false, …) | Indicates SDT DRB or not. | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Released Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| Uplink TxDirectCurrentTwoCarrierList Information | O |  | 9.3.1.283 |  | YES | ignore |
| IAB Conditional RRC Message Delivery Indication | O |  | ENUMERATED (true, …) | Indicates whether the RRC message within should be withheld. This IE is only applicable if the UE is an IAB-MT, and the gNB-DU is an IAB-DU. | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 | This IE is only applicable if the UE is an IAB-MT. | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Updated Remote UE Local ID | O |  | Remote UE Local ID 9.3.1.267 | This IE indicates the updated Remote UE Local ID for the U2N Remote UE associated with the F1AP-IDs | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | O |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to Be Released Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  |  |  |
| >>CHOICE PC5 RLC Channel QoS Information | O |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicate the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Released Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| Multicast MBS Session Remove List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has left. | YES | reject |
| UE Multicast MRB to Be Setup at Modify List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup at Modify Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| UE Multicast MRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| SL DRX Cycle List |  | 0..1 |  |  | YES | ignore |
| >SL DRX Cycle Item IEs |  | 1 .. <maxnoofSLdestinations > |  |  | EACH | ignore |
| >>RX UE ID | M |  | BIT STRING (SIZE(24)) | Indicates the destination L2 ID of RX UE associated to this UE. | - |  |
| >>CHOICE SL DRX Information | M |  |  |  | - |  |
| >>>SL DRX Cycle |  |  |  |  |  |  |
| >>>>SL DRX Cycle Length | M |  | ENUMERATED (ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...) | Indicates the desired SL DRX cycle for RX UE associated to this UE. | - |  |
| >>>No SL DRX |  |  |  |  | - |  |
| >>>>SL DRX configuration indicator | M |  | ENUMERATED(release,...) |  | - |  |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.274 |  | YES | ignore |
| SDT Bearer Configuration Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| DAPS HO status | O |  | ENUMERATED(initiation, …) | This IE is used if DAPS HO is initiated. | YES | ignore |
| Relay UE L2ID | O |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the sL-SourceIdentity IE as defined in TS 38.331 [8]. | YES | ignore |

9.2.2.5	UE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the UE-associated logical F1 connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | ignore |
| SRB ID | C- ifRRCContainer |  | 9.3.1.7 | The gNB-DU sends the RRC message on the indicated SRB. | YES | ignore |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Include it if RRCReestablishmentRequest is not accepted | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| Positioning Context Reservation Indication | O |  | ENUMERATED (True,...) |  | YES | ignore |
| CG-SDT Kept Indicator | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Relay UE L2ID | O |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the sL-SourceIdentity IE as defined in TS 38.331 [8]. | YES | ignore |