3GPP TSG-RAN WG3 #119bis                                                                                         R3-231562

Online, 17th – 26th April, 2023

Agenda Item:	21.2

Source:	CATT

Title:	Discussion on long eDRX for RRC_INACTIVE base on SA2 progress

Document for:	Discussion

- Introduction

A LS [1] is sent from SA2 about long eDRX for RRC_INACTIVE in RAN3#118.  In this paper, we further discuss the details in RAN3 and provide our proposals.

- Discussion

On the NGAP message from NG-RAN node to AMF providing the eDRX information for RRC_INACTIVE and requesting the CN to handle the MT communication (Figure 4.8.1.1a-1 steps 2 and 6), RAN3 has made an early agreement to use a class 1 procedure. The details of this class 1 procedure will be discussed by RAN3 during the WI phase next year.

In [2], whether the N2 notification to be used is a new message or an existing message can be discussed in RAN3.

2.	Either immediately following step 1a or after having delayed the request for NG-RAN based on implementation, the NG-RAN sends N2 message to AMF indicating the UE is transitioning to RRC_INACTIVE state and the CN handles MT communication. The NG-RAN also provides the eDRX cycle value for RRC_INACTIVE to AMF.

If the NG-RAN receives DL NAS message and the UE is in RRC_INACTIVE with RRC configured eDRX cycle and is considered not reachable, NG-RAN indicates to the AMF a NAS non-delivery and then initiates for the CN to handle mobile terminated (MT) communication.

Editor's note:	Whether the N2 message to be used is a new message or an existing message, e.g. N2 notification, needs coordination with RAN WGs.

6.	The AMF sends N2 response to NG-RAN acknowledging the indication and the AMF considers the UE is in CM-CONNECTED with RRC_INACTIVE state.

After the unreachability indication sending from gNB to AMF including eDRX information in step 2, AMF should response NG-RAN an acknowledge message in step 6. It should be a class 1 procedure and UE associated. We propose to introduce a new class 1 message because RRC Inactive Transition Report is a class 2 message and other signaling cannot reflect the status transition of UE.

9.2.x.1	CN BASED MT COMMUNICATION HANDLING REQUEST

This message is sent by the NG-RAN node to notify the 5GC the UE with eDRX>10.24s enters RRC_INACTIVE state and request CN based MT communication handling.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  |
| RRC State | M |  | 9.3.1.92 |  |
| eDRX cycle value | M |  | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) | TeDRX CN defined in TS 38.304 [12]. Unit: [number of hyperframes]. |

9.2.x.2	CN BASED MT COMMUNICATION HANDLING RESPONSE

This message is sent by the AMF to confirm CN based MT communication handling.

Direction: AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  |

Proposal 1: A new Class 1 procedure should be introduced in case UE is in long eDRX RRC_INACTIVE state.

- gNB sends unreachability information to AMF

- AMF sends response message to acknowledging the indication received from gNB.

In general, RAN3 will continue discussing the overall functional impacts of CN based MT communication handling on the RAN3 specifications, such as the addition of new IE in the “Core Network Assistance Information for RRC INACTIVE” IE to indicate CN capability of MT communication handling with long eDRX for RRC_INACTIVE, the XnAP and the F1AP impacts during the NR_redcap_enh WI phase next year (as agreed in RP-222675).

In [2], it confirmed by SA2 that the NG-RAN should know whether the AMF support CN based MT communication handling.

This procedure may be initiated by the serving NG-RAN node when the UE is in CM-CONNECTED with RRC_CONNECTED state and has received the "RRC Inactive Assistance Information" from the AMF as defined in clause 5.3.3.2.5 of TS 23.501 [2]. NG-RAN initiates the transition to RRC_INACTIVE state as defined in TS 38.300 [9].

RAN3 can add the " CN based MT communication handling " in “Core Network Assistance Information for RRC INACTIVE”IE in INITIAL UE CONTEXT SETUP to indicate CN capability of MT communication handling with long eDRX for RRC_INACTIVE.

9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| UE Specific DRX | O |  | Paging DRX 9.3.1.90 |  | - |  |
| Periodic Registration Update Timer | M |  | 9.3.3.24 |  | - |  |
| Skip unchanged part |  |  |  |  |  |  |
| CN based MT communication handling | O |  | ENUMERATED (supported, …) |  | - |  |

Propsoal 2: Add CN based MT communication handling in Core Network Assistance Information for RRC INACTIVE

When the NG-RAN sends an N2 Notification to the AMF indicating the UE is in RRC_CONNECTED (Figure 4.8.2.2-1 step 3b.1), RAN3 also discussed the possibility to reuse the NGAP class 2 procedure RRC INACTIVE TRANSITION REPORT message defined in TS 38.413 to indicate AMF that the UE is back to RRC_CONNECTED state, but RAN3 has not reached an agreement. RAN3 will continue the discussion during the WI phase next year.

In [2], whether the N2 notification to be used is a new message or an existing message also if it is class 1 or class 2 can be discussed in RAN3.

4b.	[Conditional] N2 Notification,

4b.1	If the accessed NG-RAN is the same as the NG-RAN that configured RRC_INACTIVE and still has the UE context, NG-RAN send an N2 Notification to the AMF indicating the UE is in RRC_CONNECTED, if an AMF requested N2 Notification (see clause 4.8.3) or Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously.

4b.5	The AMF sends the N2 response to NG-RAN.

Editor's note:	Whether the N2 message to be used is a new message or an existing message, e.g. N2 notification, needs coordination with RAN WGs considering also if it's NGAP class 1 or class 2 procedure.

In case of UE resume to RRC_CONNECTED state, NG-RAN should notify AMF. Upon reception of this notification, AMF may stop CN based MT communication handling and send DL NAS or data to NG-RAN. It seems that no feedback from AMF to NG-RAN is needed.

The existing message i.e., RRC INACTIVE TRANSITION REPORT can be reused to indicate AMF that UE back to RRC_CONNECTED. But it should be a class 1 message while RRC INACTIVE TRANSITION REPORT is a class 2 message. A class 2 signalling from AMF to NG-RAN can be introduced.

9.2.2.10	RRC INACTIVE TRANSITION REPORT

This message is sent by the NG-RAN node to notify the 5GC the UE enters or leaves RRC_INACTIVE state.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| RRC State | M |  | 9.3.1.92 |  | YES | ignore |
| User Location Information | M |  | 9.3.1.16 |  | YES | ignore |

Proposal 3: Reuse NGAP RRC INACTIVE TRANSITION REPORT message to indicate AMF that UE backs to RRC_CONNECTED. Introduce a class 2 message sending from AMF to NG-RAN to response RRC states transition.

When the AMF sends an N2 message to NG-RAN node with the request for the UE to be transitioned to RRC_CONNECTED (Figure 4.8.2.2b-1 step 2), RAN3 has not yet made a decision whether the existing NGAP CN Paging procedure should be enhanced to support CN Triggered RAN Paging with CN based MT communication handling, or if a new NGAP notification procedure should be defined to trigger RAN paging with CN based MT communication handling. RAN3 will continue the discussion during the WI phase next year.

In [2], whether the N2 notification to be used is a new message or an existing message for triggering UE into RRC_CONNECTED can be discussed in RAN3.

2.	The AMF sends an N2 message to NG-RAN with the request for the UE to be transitioned to RRC_CONNECTED.

Editor's note:	Whether the N2 message to be used is a new message or an existing message, e.g. N2 notification, needs coordination with RAN WGs.

In case of AMF considers that UE is reachable based on the long eDRX, AMF will send an N2 message to trigger RAN paging.  There are three options can be considered:

- NG PAGING

- DOWNLINK NAS TRANSPORT

Given that this message should be a UE associated signalling, including UE ID to NGAP paging is not a good solution. Furthermore, UE context is still in CN which is not a traditional CN paging. For DOWNLINK NAS TRANSPORT, it would be more suitable to send to NG-RAN which including UE ID

9.2.5.2	DOWNLINK NAS TRANSPORT

This message is sent by the AMF and is used for carrying NAS information over the NG interface.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Old AMF | O |  | AMF Name 9.3.3.21 |  | YES | reject |
| RAN Paging Priority | O |  | 9.3.3.15 |  | YES | ignore |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | O |  | 9.3.1.58 |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE Radio Capability | O |  | 9.3.1.74 |  | YES | ignore |
| UE Capability Info Request | O |  | 9.3.1.192 |  | YES | ignore |
| End Indication | O |  | 9.3.3.32 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| Target NSSAI Information | O |  | 9.3.1.229 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| RAN Paging Indication | O |  | ENUMERATED (true, …) | Indicates the NG-RAN performs RAN Paging | YES | ignore |

Proposal 4: DOWNLINK NAS TRANSPORT can be used for trigger RAN paging by adding a RAN paging indication.

- Conclusion

Proposal 1: A new Class 1 procedure should be introduced in case UE is in long eDRX RRC_INACTIVE state.

- gNB sends unreachability information to AMF

- AMF sends response message to acknowledging the indication received from gNB.

Propsoal 2: Add CN based MT communication handling in Core Network Assistance Information for RRC INACTIVE

Proposal 3: Reuse NGAP RRC INACTIVE TRANSITION REPORT message to indicate AMF that UE backs to RRC_CONNECTED. Introduce a class 2 message sending from AMF to NG-RAN to response RRC states transition.

Proposal 4: DOWNLINK NAS TRANSPORT can be used for trigger RAN paging by adding a RAN paging indication.

- Reference

[1] R3-226776, Reply LS on long eDRX support for RRC_INACTIVE, RAN3.

[2] 3GPP TS 23.502 Procedures for the 5G System (5GS), Stage 2.