3GPP TSG-RAN3 #119bis-e	R3-231572

Online, 17th – 26th April 2023

Agenda Item:	16.4

Source:	Ericsson

Title:	(TP for SL Relay BL CR to TS 38.401) Multi-path for Sidelink Relay

Document for:	Discussion, Decision

# 1	Introduction

In RAN3#119 meeting, the following progress has been made for the support of Multipath for Sidelink Relay.

Add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401. FFS on other procedures.

Turn the following WAs into agreement:

WA: The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision.

WA: For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB.

WA: The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition.

In this contribution, we would provide analysis and give the proposals.

# 2	Discussion

Figure 1 depicts CU-DU split architecture supporting multipath scenario for intra-gNB case, including both intra-DU and inter-DU.

Figure 1: CU-DU split architecture supporting the MP scenario in the same gNB, where the two paths are associated with different cells

## 2.1 Inter-DU case

RAN3 has agreed the basic signaling flows to support adding path for inter-DU case in TS 38.401. Among them, some updates are required. For example, in case of additional direct path on top of indirect path, step 1 says that the remote UE may report the measurements of one or multiple candidate relay UEs. However, current agreement for MP is that only one indirect path would be used at the same time. Thus the remote UE does not have to measure the other candidate relay UE. Similar for the case of additional indirect path on top of direct path, the remote UE does not report Uu measurements of neighboring cells since the direct path already exists.

A TP is provided in the Annex.

- Remove part of UE measurements reporting from step 1 in the stage-2 signaling flows.

- Agree the TP to TS 38.401 in the Annex.

In last meeting, the inter-DU case signaling flows have been captured in TS 38.401. One missing piece is about how the scheduling information can be sent/received to/from DU2 when the remote UE has a existing direct path with DU1. Thus, DU1 needs to be able to provide scheduling information to DU2.

RAN3 would further check whether need to transfer info over F1 to support the scheduling coordination between DU1 and DU2.

- Consider F1AP impacts to support the scheduling coordination for inter-DU case.

## 2.2 Direct/indirect path addition/release

RAN2 is discussing which cases to be supported for different scenarios, and some agreements have been made.

Change of direct path while keeping the indirect path can be done with a release-and-add in a single RRC message. This does not exclude a gNB implementation from using separate release and add procedures instead.

Change of indirect path while keeping the direct path can be done with a release-and-add in a single RRC message. This does not exclude a gNB implementation from using separate release and add procedures instead.

Taking an example, in case E, once the direct path is changed to a different cell, this can mostly be attributed to its own mobility and as a result, the remote UE will not be able to keep the indirect path via the same U2N relay UE. In addition, based on the measurement configuration and results, the gNB can configure:

- Either the change of the direct path to a different cell of the same gNB and keep the indirect path.

- Or the change of the direct path to a different cell in a different gNB and release the indirect path. As Re-18 does not support the case where the remote UE is now allowed to be connected to two cells from different gNBs.

Given that this is the first release for this item, we should specify basic operations, hence, the network behavior should be consistent when the direct path changes to a different cell of the same/different gNB. F1AP might be impacted considering the gNB-CU should inform the gNB-DU about the path addition/release.

- The gNB-CU indicates to the gNB-DU whether to add/release at least an indirect path. F1AP can be enhanced by adding an indicator and/or the channels mapping to be added/released.

# 3	Conclusion

In this paper we propose the followings. A TP to TS 38.401 is provided in the Annex.

Proposal 1	Remove part of UE measurements reporting from step 1 in the stage-2 signaling flows.

Proposal 2	Agree the TP to TS 38.401 in the Annex.

Proposal 3	Consider F1AP impacts to support the scheduling coordination for inter-DU case.

Proposal 4	The gNB-CU indicates to the gNB-DU whether to add/release at least an indirect path. F1AP can be enhanced by adding an indicator and/or the channels mapping to be added/released.

# 4	Annex

## 8.xx Overall procedures for multi-path support

### 8.xx.1 Inter-DU direct path addition on top of indirect path

The signalling flow for inter-DU direct path addition is shown in Figure 8.xx.1-1.

Figure 8.xx.1-1: Signalling procedure of inter-DU direct path addition on top of indirect path

- The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells.

2.	The gNB-CU decides to add the direct path to remote UE under a different gNB-DU (i.e., gNB-DU1).

3.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU1, which contains at least the direct path configuration.

4.	The gNB-DU1 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

5.	The gNB-CU sends an RRCReconfiguration message to the relay UE to update the indirect path configuration if necessary.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for remote UE by including the RRCReconfiguration message to the gNB-DU2. The contents in the RRCReconfiguration message may include at least direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).

7.	The gNB-DU2 sends the RRCReconfiguration message to the remote UE.8.	The gNB-DU2 sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The remote UE performs random access procedure at the gNB-DU1.

10.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU2.

Editor’s Note: FFS whether the remote UE1 sends the RRCReconfigurationComplete message to the gNB-CU via old path (indirect path) or new path (direct path).

11.	The gNB-DU2 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

### 8.xx.2 Inter-DU indirect path addition on top of direct path

The signalling flow for inter-DU indirect path addition is shown in Figure 8.xx.2-1.

Figure 8.xx.2-1 Signalling procedure of inter-DU indirect path addition on top of direct path

1. The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report PC5 measurement results of and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under a different gNB-DU (i.e., gNB-DU2).

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU2 and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU2, which contains the indirect path configuration at least.

5.	The gNB-DU2 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU1. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s).

7.	gNB-DU1 sends the RRCReconfiguration message to the remote UE.

8.	The remote UE establishes PC5 connection with relay UE.

9.	The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU1.

Editor’s Note: FFS whether the remote UE sends the RRCReconfigurationComplete message to the gNB-CU via old path (direct path) or new path (indirect path).

Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress.

10.	The gNB-DU1 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.