3GPP TSG-RAN WG3 #119bis-e								R3-231580

17th – 26th April 2023 Online

Agenda Item:	8.1

Source:	China Telecom, Huawei

Title: 	Clarification of Use Case and the Effect of the new Attribute “Only Resource Coordination”

Document for:	Discussion

Introduction

At RAN3 #119bis-e meeting, a reply LS from SA5 on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA was received [1]. This LS requires RAN3 to clarify use case and the effect of the new attribute “Only Resource Coordination”.

SA5 understands that it is beneficial to use an EN-DC X2 interface only for coordination of resources between E-UTRA cells and NR cells controlled by different RAN nodes.

In order to maintain the consistency of the attributes of the NRCellRelation IOC in TS 28.541 with the definitions of RAN3, SA5 requires more detailed information about the usage and semantics of this new attribute, i.e., use case that motivates this attribute, which network function is supposed to set this attribute (e.g. by the neighboring eNB/gNB via X2, or by human operator via OAM, in gNB or eNB), the effect of this attribute on the system once it was set.

SA5 will take the response from RAN3 into account to specify the attribute requested by RAN3 if needed.

In this paper, we would like to provide our views on the questions raised by SA5.

Discussion

- Use case that motivates this attribute

The use case aims to support the resource coordination between legacy eNBs and NR standalone gNBs to avoid cochannel interference. The straightforward solution is to support the both role of gNB and en-gNB in NR site simultaneously. However, a direct interface, i.e, X2 interface, between LTE and NR SA site was not supported in current specifications and the operators do not expect to reconfigure all the EN-DC features in NR SA site. To this end, RAN3 concluded that reusing the existing E-UTRA NR Cell Resource Coordination function specified in TS 36.423 is the most effective way and have minimum standard impacts. A new NRT attribute “Only Resource Coordination” in LTE side then is agreed for this purpose while not activating other EN-DC X2 functions. So it is need to inform SA5 that this new NRT attribute is applied in EN-DC scenario.

Proposal 1: it is need to inform SA5 that this new NRT attribute is applied in EN-DC scenario

- which network function is supposed to set this attribute

Per RAN3 agreement, stage2 CR on this feature is enough. The semantics of this new attribute is specified in the technically endorsed CR to TS 36.300 in R3-225158.

-	Resource Coordination Only: If checked, the Neighbour Cell Relation shall use the X2 interface instance only to coordinate resources between the E-UTRA cell and the NR cell.

We understand the neighbouring nodes do not provide any assistant information via X2 interface to set this attribute. Therefore, this new attribute in LTE eNB is set by human operator via OAM.

Proposal 2: this new attribute in LTE eNB is set by human operator via OAM.

- the effect of this attribute on the system once it was set

The effect on the system if it is set is that EN-DC X2 interfaces are established between neighbouring eNBs and gNBs operating on the same frequency bandwidth only for the purpose of resource coordination.

Proposal 3: The effect on the system if it is set is that EN-DC X2 interfaces are established between neighbouring eNBs and gNBs operating on the same frequency bandwidth only for the purpose of resource coordination.

Proposal 4: to approve the reply LS in [2]

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: it is need to inform SA5 that this new NRT attribute is applied in EN-DC scenario.

Proposal 2: this new attribute in LTE eNB is set by human operator via OAM.

Proposal 3: The effect on the system if it is set is that EN-DC X2 interfaces are established between neighbouring eNBs and gNBs operating on the same frequency bandwidth only for the purpose of resource coordination.

Proposal 4: to approve the reply LS in [2]

Reference

- R3-231124/S5-233142 Reply LS on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA, SA5, China Telecom

- R3-231579 Reply LS on introduction of a new attribute "Only Resource Coordination" to support source coordination between LTE and NR SA