3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231631

# E-meeting, 17 – 26 April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1027 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Network Energy Saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR contains the agreed changes required for Network Energy Saving |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3.1, 8.4.3.2, 8.4.3.3, 9.1.3.4, 9.1.3.7, 9.1.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 38.473 CR ... TS 38.401 CR … TS 38.300 CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.4.3	Cell Activation

#### 8.4.3.1	General

The purpose of the Cell Activation procedure is to enable an NG-RAN node to request a neighbouring NG-RAN node to switch on one or more cells and/or one or more beams, previously reported as inactive due to energy saving.

The procedure uses non UE-associated signalling.

#### 8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Cell Activation, successful operation

The NG-RAN node1 initiates the procedure by sending the CELL ACTIVATION REQUEST message to the peer NG-RAN node2.

Upon receipt of this message, the NG-RAN node2 should activate the cell/s or beam/s indicated in the CELL ACTIVATION REQUEST message and shall indicate in the CELL ACTIVATION RESPONSE message for which cells the request was fulfilled.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the CELL ACTIVATION REQUEST message and the CELL ACTIVATION RESPONSE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

Interactions with NG-RAN Configuration Update procedure:

The NG-RAN node2 shall not send the NG-RAN CONFIGURATION UPDATE message to the NG-RAN node1 just for the reason of the cell/s indicated in the CELL ACTIVATION REQUEST message changing cell activation state, as the receipt of the CELL ACTIVATION RESPONSE message by the NG-RAN node1 is used to update the information about the activation state of NG-RAN node2 cells in the NG-RAN node1.

#### 8.4.3.3	Unsuccessful Operation

Figure 8.4.3.3-1: Cell Activation, unsuccessful operation

If the NG-RAN node2 cannot activate any of the cells or beams indicated in the CELL ACTIVATION REQUEST message, it shall respond with the CELL ACTIVATION FAILURE message with an appropriate cause value.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the CELL ACTIVATION REQUEST message and the CELL ACTIVATION FAILURE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

#### 8.4.3.4	Abnormal Conditions

Void.

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name |  | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List |  | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType |  | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |  |
| >>Served Cells To Update NR |  | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR |  | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA |  | O |  | 9.2.2.43 |  | YES | ignore |
| >>Served Cell Specific Info Request |  | O |  | 9.2.2.102 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA |  | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR |  | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA |  | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | M |  |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | O |  |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  |  | 9.2.3.96 |  | YES | ignore |
| Coverage Modification List |  |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell to be modified. | – |  |
| >>Cell Coverage State | M |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  |  | – |  |
| >>>Replacing Cells |  |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >>SSB Coverage Modification List |  |  | 0.. 1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  |  | INTEGER (0..15, …) | Value '0' indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  |  | ENUMERATED (coverage, cell edge capacity, ..., energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| Local NG-RAN Node Identifier | O |  |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

#### 9.1.3.7	CELL ACTIVATION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request a previously switched-off cell/s or beam/s to be re-activated.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Served Cells To Activate | M |  |  |  | YES | reject |
| >NR Cells |  |  |  |  |  |  |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >E-UTRA Cells |  |  |  |  |  |  |
| >>E-UTRA Cells List |  | 1 |  |  | – |  |
| >>>E-UTRA Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>E-UTRA CGI | M |  | 9.2.2.8 |  | – |  |
| >NR Cells or Beams |  |  |  |  | YES | reject |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>>NR CGI | M |  | 9.2.2.7 |  |  |  |
| >>> CHOICE Cell or Beam |  |  |  |  |  |  |
| >>>>Full cell |  |  | NULL |  |  |  |
| >>>>Beams |  |  |  |  |  |  |
| >>>>NR Beams List |  | 1 |  |  |  |  |
| >>>>>NR Beams item |  | 1 .. <maxnoofSSBAreas> |  |  |  |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.1.3.8	CELL ACTIVATION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more cell(s) previously switched-off has (have) been activated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Activated Served Cells | M |  |  |  | YES | reject |
| >NR Cells |  |  |  |  |  |  |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells Item |  | 1 .. < maxnoffCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >E-UTRA Cells |  |  |  |  |  |  |
| >>E-UTRA Cells List |  | 1 |  |  | – |  |
| >>>E-UTRA Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>E-UTRA CGI | M |  | 9.2.2.8 |  | – |  |
| >NR Cells or Beams |  |  |  |  | YES | reject |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>>NR CGI | M |  | 9.2.2.7 |  |  |  |
| >>> CHOICE Cell or Beam |  |  |  |  |  |  |
| >>>>Full cell |  |  | NULL |  |  |  |
| >>>>Beams |  |  |  |  |  |  |
| >>>>NR Beams List |  | 1 |  |  |  |  |
| >>>>>NR Beams item |  | 1 .. <maxnoofSSBAreas> |  |  |  |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |