3GPP TSG-RAN WG3 Meeting #119bis-e  	R3-231658

E-meeting, 17 April – 26 April 2023

Agenda item:	12.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TS 38.423) Cell-based UE Trajectory Prediction

Document for:	Text Proposal

# 1	Introduction

The use and semantics of the UE trajectory prediction for AI/ML was discussed during RAN3#117-e and RAN3#117bis-e, and the following agreements were captured:

Predicted cell-granularity UE trajectory can be exchanged over Xn for AI/ML based mobility optimization.

Cell-based UE Trajectory prediction has the same structure as UE History Information IE.

Cell-based UE Trajectory prediction is provided as a list of cells into the future, each of which is indicated together with an expected time of stay into the cell.

This was further discussed during RAN3#118 and the following were agreed:

UE Trajectory Prediction is transferred to the target gNB via the Handover Request.

In RAN3 #119, two open points were captured:

The presence of the predicted time of stay of a UE in a cell is FFS.

FFS whether predicted UE Trajectory spans across multiple NG-RAN nodes or it is limited within a single target NG-RAN node.

In this paper we provide our views on the remaining open points and provide a TP for BL CR for TS 38.423.

# 2	Cell-Based UE Trajectory

Cell-based UE Trajectory prediction, provided as a list of cells into the future, each of which is indicated together with an expected time of stay into the cell, can be sent in Handover Request message for AI/ML Mobility Optimization. This information can be useful to the target node to perform:

- Resource reservation, namely, to allocate resources accordingly depending on how long a UE is expected to stay in its cells.

- Training or Retraining of a cell-based trajectory prediction AI/ML Model using neighbour information in the training input.

In addition, cell-based trajectory information at the source is useful to the NG-RAN serving the UE to select

- A handover method, e.g., to allow a source node to decide whether to trigger conditional handover versus baseline handover.

- Select the next target node

In the absence of an expected time of stay of a UE at a cell, the promised gains from sending cell-based predicted trajectory from a source to a target node diminish. Specifically, a target will not be able to know for how long to reserve resources for the incoming handover. In addition, if timing information is missing a source node may not know whether to trigger a normal handover or a conditional handover. Therefore, it becomes apparent that the timing aspect is important because different predicted times of stay in the same cell give rise to different trajectories. As can be seen in Figure 1, both UEs, UE1 and UE2, follow the same path over a set of cells in the same order. However, entering time to the next cell and effectively the duration of stay in the cell are different per UE so their trajectories are different e.g. from the point of view of prediction of required resources.

Observation 1: The same sequence of cells gives rise to different UE trajectories.

Figure 1 UE Trajectory example for two UEs, UE1 and UE2. Figure also illustrates the different times spent in a cell for the two UEs and how this defines different trajectories.

Another possible information that can be extracted from Figure 1 is the UE’s short stay in a cell. For example, UE 1 spends very limited time in Cell 4, as the UE executes a handover to Cell 6 immediately after entering Cell 4. Handover to Cell 4 may be a handover to a wrong cell that may be prevented if the predicted time of stay is indicated together with the cell information.

For the reasons above we think that the expected time of stay in a cell must be a mandatory field. It is of course possible that in certain cases the source node does not have enough data to sufficiently train a ML Model for cell-based UE trajectory prediction. In such cases, the predicted time a UE stays in a cell cannot be computed. We think that in those cases a source NG-RAN node should not provide information for this trajectory to a target NG-RAN node.

Proposal 1: Predicted Time of stay in a cell shall be a mandatory IE in the cell-based UE Trajectory prediction.

# 3  	Input Data for Training Cell-based UE Trajectory Prediction

According to the agreement taken at RAN3#117bis-e, cell-based UE Trajectory prediction is provided as a list of cells into the future, each of which is indicated together with an expected time of stay into the cell. To provide reliable predictions of a cell-based UE trajectory, suitable data must be used at the input of the ML model. Since the output of a ML model for cell-based UE Trajectory prediction is on a cell level, it is sufficient that the input data to train the model has the same granularity. Additionally, e.g. GNSS or other positioning data may not be easily mapped to cell id by the NG-RAN node.

Observation 2: Since UE Trajectory prediction is calculated as a sequence of cells into the future, input data on a cell-level are sufficient to train the ML model.

At the same time, RAN3 agreed that cell-based UE Trajectory prediction has the same structure as UE History Information IE. Thus, in order to train a model for cell-based UE Trajectory prediction, measurements reported through the network interfaces, such as UE History Information together with UE reported Mobility History Information, should be used.

However, UE History information provides a sequence of cells that the UE has visited in the past but doesn’t give any information about the cells that the UE will visit in the future. Given that the intention is to predict future UE trajectories, continuous data collection over future cells a UE may visit in the future is needed so that this information becomes available at the node training an ML Model for UE Trajectory prediction.

Proposal 2: Enabling of continuous data collection of sequences of cells that a UE will visit in the future together with the time spent in a cell is needed as an input to a node training a ML Model for cell-based UE Trajectory prediction.

An open point from last meeting was whether we need to limit predicted UE Trajectory only to the next target NG-RAN node or whether the list of cells may span across multiple NG-RAN nodes. We don’t think it is necessary to limit predicted UE trajectory only to the next target node.

Proposal 3: Predicted UE trajectory may span across cells of one or more NG-RAN nodes.

UE History Information and Mobility History Information can be used for training cell-based UE Trajectory prediction. This information potentially spans over multiple nodes. Mobility History information is available at the UE and may be reported to the network when UE connects to the network after idle mode.

Proposal 4: Use UE History Information and UE History Information from UE for Training an ML Model for cell-based UE Trajectory prediction.

A question that was discussed during last RAN3 meeting is how the source can receive UE Trajectory feedback. In our view, no feedback is needed other than input sequences of cells into the future received by the NG-RAN node where the AI/ML model training takes place. For a source node to train an ML Model for cell-based UE Trajectory prediction it needs to receive input information reflecting sequences of cells into the future and a time spent in those cells for different UEs. To continuously retrain the UE Trajectory prediction model this information needs to be provided from future cells that a UE may visit. A source node can use this information together with the calculated prediction to determine whether it has accurately predicted the trajectory. One reason behind our position originates from the purpose of sending this information in Handover Request message. A predicted cell-based trajectory at a target NG-RAN node is useful information to determine for how long a certain UE will remain at the cells of the target node. If the prediction is inaccurate, affecting the predicted time of stay in a cell or the cell itself, the target node will become immediately aware about this. Trajectory related to other cells into the future can be used for example by a target NG-RAN node to train its own model for UE Trajectory prediction using input from its neighbours. If a neighbour node sends predicted UE Trajectory information, updated as the Model gets retrained, frequently enough then even if some of the received predicted UE trajectories are inaccurate, the target node can eliminate those inputs as outliers in the training phase.

Proposal 5: A source node determines whether the UE Trajectory prediction algorithm it has trained is sufficiently accurate through continuous re-training of the model by receiving the input sequences of cells into the future and time spent in those cells for different UEs.

Finally, we propose to agree on the TP for BL CR for TS 38.423 in the Annex.

Proposal 6: Accept the TP for BL CR for TS 38.423 in the Annex.

# 4 	Conclusion

In this paper we make the following observations and proposals:

Observation 1: The same sequence of cells gives rise to different UE trajectories.

Proposal 1: Predicted Time of stay in a cell shall be a mandatory IE in the cell-based UE Trajectory prediction.

Observation 2: Since UE Trajectory prediction is calculated as a sequence of cells into the future, input data on a cell-level are sufficient to train the ML model.

Proposal 2: Enabling of continuous data collection of sequences of cells that a UE will visit in the future together with the time spent in a cell is needed as an input to a node training a ML Model for cell-based UE Trajectory prediction.

Proposal 3: Predicted UE trajectory may span across cells of one or more NG-RAN nodes.

Proposal 4: Use UE History Information and UE History Information from UE for Training an ML Model for cell-based UE Trajectory prediction.

Proposal 5: A source node determines whether the UE Trajectory prediction algorithm it has trained is sufficiently accurate through continuous re-training of the model by receiving the input sequences of cells into the future and time spent in those cells for different UEs.

Proposal 6: Accept the TP for BL CR for TS 38.423 in the Annex.

# References

[1]		R3-231042, (BLCR to 38.423) for AI/ML for NG-RAN

# Annex – TP for BL CR for TS 38.423

This TP is based on [1].

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

<< skip unchanged procedural text >>>

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Cell Based UE Trajectory Prediction IE is contained in the HANDOVER REQUEST message, the target NG-RAN node considers the content of this list as the predicted cells for UE trajectory by the source NG-RAN node, and may use it for e.g. mobility decisions.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

#### 8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: Handover Preparation, unsuccessful operation

If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

<<< skip unchanged procedural text >>>

#### 8.2.1.4	Abnormal Conditions

<<< skip unchanged procedural text >>>

If the HANDOVER REQUEST message includes information for a PLMN not serving the UE in the target NG-RAN node in the Management Based MDT PLMN List IE, the target NG-RAN node shall ignore information for that PLMN within the Management Based MDT PLMN List.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.AA AI/ML Information Reporting Initiation (FFS on the name)

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to "start"; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	FFS

If the Registration Request IE is set to "start" in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all or part of (exact details of if and how to support partial reporting are FFS) requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If all of (exact details of if and how to support partial reporting are FFS) the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

#### 8.4.AA.4	Abnormal Conditions

FFS

### 8.4.BB	AI/ML Information Reporting (FFS on the name)

#### 8.4.BB.1	General

This procedure is initiated by an NG-RAN node to report AI/ML related information accepted by the NG-RAN node following a successful AI/ML Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be reported using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.BB.2	Successful Operation

Figure 8.4.11.2-1: AI/ML Information Reporting, successful operation

NG-RAN node2 shall report the accepted AI/ML related information in AI/ML INFORMATION UPDATE message. The accepted AI/ML related information is the information that was successfully initiated during the preceding AI/ML Information Reporting Initiation procedure.

#### 8.4.BB.3	Unsuccessful Operation

Not applicable.

#### 8.4.BB.4	Abnormal Conditions

Void

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.x |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = UE Performance Fifth Bit = UE History FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.DD	AI/ML INFORMATION RESPONSE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or part of (exact details of if and how to support partial reporting are FFS)  the objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Reporting Characteristics (FFS) | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is able to report. FFS on the coding | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.EE	AI/ML INFORMATION FAILURE (FFS on the name)

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of (exact details of if and how to support partial reporting are FFS) the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item (FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resources Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>FFS | O |  |  |  | - | - |
| UE Performance Result (FFS) | O |  | 9.2.3.Y |  |  |  |
| UE History | O |  |  |  |  |  |
| >UE History Information | M |  | 9.2.3.64 |  |  |  |
| >UE History Information from UE | O |  | 9.2.3.110 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x	Cell Based UE Trajectory Prediction

Editors Note: Actual name of this IE is FFS.

The Cell Based UE Trajectory Prediction IE contains the list of NG-RAN cells where the UE is predicted to connect.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Predicted Trajectory Cell List |  | 1..<maxnoofCellsTrajectoryPredict> |  | List of cells where the UE is predicted to connect, in chronological order. The first predicted cell added to the top of this list, is the cell where the UE will move to after the serving cell. |
| >Predicted Trajectory Cell Information | M |  | 9.2.3.Z |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTrajectoryPredict | Maximum number of cells that can be predicted for UE trajectory. Value is FFS. |

#### 9.2.3.Z	Predicted Trajectory Cell Information

The Predicted Trajectory Cell Information contains the cell ID of the predicted NG-RAN cell for cell based UE trajectory prediction.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Predicted Trajectory Cell Information |  |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |
| >>Predicted Time UE Stays in Cell | M |  | INTEGER (0..4095) | The duration of time the UE is expected to stay in the cell in seconds. If the duration is more than 4095s, this IE is set to 4095. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>