3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231723

Electronic meeting, 17 Apr – 26 Apr, 2023

Title: 	Discussion on trace in CU-DU split and dual connectivity

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

1. Introduction

There is a LS from SA5 in [1] addressing the management based trace function in CU-DU function split and dual connectivity cases.

SA5 has continued to deliver enhancements to Trace/MDT.  All currently defined trace formats [1] have used existing UE identifiers including those specified by RAN3.

For Rel-18, SA5 is considering further enhancements of trace management and collection.

To support trace for Dual Connectivity specifically, there may be a need for updates to allow management-based trace data to be correlated for a session originating from the secondary node of a Dual Connectivity deployment or from the CUUP/DU in split RAN architecture deployments.

We would like RAN3 guidance on potential enhancements which may assist SA5 in progressing such work.

In this contribution, we provide our views on this issue and a draft reply LS.

2. Background

2.1 Management based trace in CU-DU function split

Section 8.13.2.2, 8.13.2.3 and 8.13.2.4 in TS 38.401 describe the management based MDT activation in CU-CP, gNB-DU and CU-UP correspondingly. As the stage 2 message flows for management based trace activation in those three cases are the same, the procedures in Section 8.13.2.2, 8.13.2.3 and 8.13.2.4 could be reused as a basic background for the further study on management based trace function.

Those procedures are cited below for reference.

#### 8.13.2.2 Management based MDT Activation in gNB-CU-CP The signalling flow for Management based MDT Activation in gNB-CU-CP is shown in Figure 8.13.2.2-1. Figure 8.13.2.2-1 Management based MDT Activation in gNB-CU-CP 1. The EM sends a Trace Session activation request to the gNB-CU-CP. This request includes the parameters for configuring UE measurements. 2. The gNB-CU-CP shall select the suitable UEs for MDT data collection. If the UE is not in the specified area or if the serving PLMN is not within the Management Based MDT PLMN List the UE shall not be selected by the gNB-CU-CP for MDT data collection as defined in TS 32.422 [20]. For each selected UE, if the gNB-CU-UP should perform MDT measurement, the gNB-CU-CP sends TRACE START message to the gNB-CU-UP, including MDT configuration parameters. 3. For each selected UE, if the gNB-DU should perform MDT measurement, the gNB-CU-CP sends TRACE START message to the gNB-DU, including MDT configuration parameters. 4. The gNB-CU-CP may send CELL TRAFFIC TRACE message to the AMF for the selected UE, including Trace ID for MDT. The AMF forwards Trace ID and other information to the TCE as specified in TS 32.422 [20]. If the UE reports an indication of measurement pollution, the gNB-CU-CP shall, if supported, include such indication as part of the measurement report to be sent to the TCE so that the TCE is able to correlate and filter the affected measurements. #### 8.13.2.3 Management based MDT Activation in gNB-DU The signalling flow for Management based MDT Activation in gNB-DU is shown in Figure 8.13.2.3-1. Figure 8.13.2.3-1 Management based MDT Activation in gNB-DU 1. The gNB-CU-CP sends UE CONTEXT SETUP REQUEST message to the gNB-DU, including Management based MDT PLMN List. The message may include the Polluted Measurement Indicator IE. If the gNB-DU has received the Polluted Measurement Indicator IE, the gNB-DU includes the information contained in such indicator as part of the measurement report to be sent to the TCE, so that the TCE is able to correlate and filter the affected measurements. 2. The gNB-DU sends UE CONTEXT SETUP RESPONSE message to the gNB-CU-CP. 3. The EM sends a Trace Session activation request to the gNB-DU. This request includes the parameters for configuring UE measurements. 4. The gNB-DU shall select the suitable UEs for MDT data collection. If the UE is not in the specified area or if the serving PLMN is not within the Management Based MDT PLMN List the UE shall not be selected by the gNB-DU for MDT data collection as defined in TS 32.422 [20]. For each selected UE, the gNB-DU may send CELL TRAFFIC TRACE message to the gNB-CU-CP in the F1 UE associated signalling, including Trace ID for MDT. 5. Upon reception of a CELL TRAFFIC TRACE message from F1, the gNB-CU-CP shall send CELL TRAFFIC TRACE message to the AMF for this UE, including Trace ID for MDT. The AMF forwards Trace ID and other information to the TCE as specified in TS 32.422 [20]. #### 8.13.2.4 Management based MDT Activation in gNB-CU-UP The signalling flow for Management based MDT Activation in gNB-CU-UP is shown in Figure 8.13.2.4-1. Figure 8.13.2.4-1 Management based MDT Activation in gNB-CU-UP 1. The gNB-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP, including Management based MDT PLMN List. The message may include Polluted Measurement Indicator. If the gNB-DU has received the Polluted Measurement Indicator IE, the gNB-DU includes the information contained in such indicator as part of the measurement report to be sent to the TCE so that the TCE is able to correlate and filter the affected measurements. 2. The gNB-CU-UP sends BEARER CONTEXT SETUP RESPONSE message to the gNB-CU-CP. 3. The EM sends a Trace Session activation request to the gNB-CU-UP. This request includes the parameters for configuring UE measurements. 4. The gNB-CU-UP shall select the suitable UEs for MDT data collection. If the serving PLMN is not within the Management Based MDT PLMN List the UE shall not be selected by the gNB-CU-UP for MDT data collection as defined in TS 32.422 [20]. For each selected UE, the gNB-CU-UP may send CELL TRAFFIC TRACE message to the gNB-CU-CP in the E1 UE associated signalling, including Trace ID for MDT 5. Upon reception of a CELL TRAFFIC TRACE message from E1, the gNB-CU-CP shall send CELL TRAFFIC TRACE message to the AMF for this UE, including Trace ID for MDT. The AMF forwards Trace ID and other information to the TCE as specified in TS 32.422 [20].

2.2 Management based trace activation in MR-DC

As specified in section 8.3.1, and section 8.3.14 TS 38.423,  the MN may initiate trace towards SN by including the Trace Activation IE in S-NG-RAN node Addition, or by sending a Trace Start message.

On the other hand, as specified in section 8.3.16, in case that the management based MDT is activated in the SN directly, the Cell Traffic Trace procedure from SN to MN is used to send the allocated Trace Recording Session Reference (TRSR) and the Trace Reference (TR) to the M-NG-RAN node. And which further triggers the MN to initiate a Cell Traffic Trace message over NGAP to signal the AMF to report the UE’s IMSI/IMEI(SV) to the TCE for trace data correlation.

3. Discussion

According to the background information in section 2.1 for management based trace in split deployment, the following observations could be made:

Observation 1:  For management-based trace initiated to CU-CP,  the CU-CP may propagate the trace task to the gNB-DU and CU-UP depending on the configured trace interfaces for selected UE.

Observation 2:  For management-based trace initiated to CU-CP,  the CU-CP may propagate the trace task to the gNB-DU and CU-UP depending on the configured trace interfaces for selected UE.

Observation 3: For management-based trace initiated to CU-CP,  the CU-CP will invoke the Cell Traffic Trace message to trigger AMF to send the IMSI/IMEI(SV) to the TCE for trace data correlation.

Observation 4: For management-based trace initiated to gNB-DU or CU-UP, the gNB-DU or CU-UP initiate the Cell Traffic Trace on F1 or E1 to trigger the gNB-CU/gNB-CU-CP to send out the Cell Traffic Trace to AMF over NG.

Based on above observations, it is proposed to reply SA5 that:

It is proposed to rely SA5 that:

  - For management based trace initiated to gNB-CU or gNB-CU-CP,  the gNB-CU or gNB-CU-CP may trigger the Cell Traffic Trace procedure on NG interface to inform AMF to send the  IMSI/IMEI(SV) to the TCE for trace data correlation.

  - For management based trace initiated to gNB-DU or gNB-CU-UP,  the gNB-DU or CU-UP may initiate the Cell Traffic Trace on F1 or E1 to trigger the gNB-CU/gNB-CU-CP to send out the Cell Traffic Trace to AMF for trace data correction.

According to the background information in section 2.2 for management based trace in MR-DC, the following observations could be made:

Observation 5:  If the serving NG-RAN node for UEs selected for management-based trace is MN, the MN may trigger the Cell Traffic Trace procedure on NG interface to inform AMF to send the  IMSI/IMEI(SV) to the TCE for trace data correlation.

Observation 6: If the serving NG-RAN node for UEs selected for management-based trace is SN, the SN may trigger the Cell Traffic Trace procedure on Xn to trigger the MN to send out the Cell Traffic Trace to AMF for trace data correlation.

Based on above observations, it is proposed to reply SA5 that:

It is proposed to rely SA5 that:

  - In MR-DC case, the MN may trigger the Cell Traffic Trace procedure on NG interface to inform AMF to send the  IMSI/IMEI(SV) to the TCE for trace data correlation.

  - In MR-DC case, the SN may trigger the Cell Traffic Trace procedure on Xn to trigger the MN to send out the Cell Traffic Trace to AMF for trace data correlation.

4. Conclusion

- IN this contribution, we discuss the trace data collection in RAN3 specifications as per the LS from SA5, and propose:

- It is proposed to rely SA5 that:

  - For management based trace initiated to gNB-CU or gNB-CU-CP,  the gNB-CU or gNB-CU-CP may trigger the Cell Traffic Trace procedure on NG interface to inform AMF to send the  IMSI/IMEI(SV) to the TCE for trace data correlation.

  - For management based trace initiated to gNB-DU or gNB-CU-UP,  the gNB-DU or CU-UP may initiate the Cell Traffic Trace on F1 or E1 to trigger the gNB-CU/gNB-CU-CP to send out the Cell Traffic Trace to AMF for trace data correction.

It is proposed to rely SA5 that:

  - In MR-DC case, the MN may trigger the Cell Traffic Trace procedure on NG interface to inform AMF to send the  IMSI/IMEI(SV) to the TCE for trace data correlation.

  - In MR-DC case, the SN may trigger the Cell Traffic Trace procedure on Xn to trigger the MN to send out the Cell Traffic Trace to AMF for trace data correlation.

- It is also proposed to agree on the reply LS in [2].

5. Reference

[1] S5-232761, LS on RAN UE Id optionality.

[2]  R3-231724	[DRAFT] Reply LS on RAN UE Id optionality