3GPP TSG-RAN WG3 Meeting #119b-e	R3-231762

Online, 17-26th April, 2023

Title: 	Further discussion on the support of R17 left-over features

Source: 	Huawei

Agenda item:	11.4

Document Type:	discussion

1. Introduction

In the last meeting, RAN3 discussed the support of R17 left-over features and had the following agreements.

RAN3 to await the further progress of SA4 and ITU-T before proceeding further on the topic of RVQoE value.

In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE.

Introduce the deactivation of RAN visible QoE information transfer via F1. No need to introduce pause/resume mechanism in Release 18.

In this paper, we further provide our views on the remaining opon issues.

2. Discussion

2.1 Event-based RVQoE reporting trigger

RAN3 has discussed event-based RVQoE reporting trigger for several meeting, we still don’t think the benefit of introducing such triggers is overwhelming, considering the additional complexity it will introduce and the small size of QoE report.

First of all, we understand one main motivation to introduce event-based RAN visible QoE reporting trigger is to alleviating Uu overload by only reporting RAN visible QoE results when certain events happen. However, as has been analyzed in R17, the signaling overhead of reporting QoE reports is not big, let alone RVQoE reports which are even smaller, so the benefit of saving Uu overload seems only marginal.  Moreover, we note that such trigger will force UE keep evaluating the trigger condition, increasing the burden of UE. Also some companies have pointed out that we can rely on post-processing in RAN and MCE for certain events. Most importantly, considering there can be various types of events, we should try to avoid introducing new reporting mechanism for many different events, where the RAN visible QoE complexity could explode.

In the last meeting, some companies proposed to first discuss the event-based triggers for mobility. While not convinced by the motivation, we would like to make it clear that the event-based triggers we are discussing should only be used for reporting, and will not affect measurement behavior.

It is suggested not to introduce event-based triggers unless great benefits have been identified.

Event-based triggers if any are only used for reporting purpose instead of measurement.

2.3 Assistance information for handling of QoE reporting upon RAN overload

We think it is beneficial to introduce the priority feature and hence have a mechanism to help the RAN to select the QoE measurement to pause/resume due to the overload of Uu. We understand this feature can be important to operators, and in the last meeting, we achieved some progress as follows:

In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE.

There are mainly two remaining controversial issues, which are first what the assistance information is, and the second who is the consumer of the QoE measurements.

Regarding the first issue, in our understanding, the assistance information is the priority for each QoE measurement (per QoE reference ID). Setting the granularity of priority as such can avoid mess and confusion when there are multiple consumers, e.g. one set the priority according to service type, another one set the priority according to slice, but offers flexibility and a consistent rule to prioritize the pause/resume of some QoE measurements in case of overload.

The assistance information is the priority for each QoE measurement.

As for the second issue, we agree there can be multiple consumers for QoE measurement, i.e, OAM is not the only consumer for QoE. However, we understand that the initial goal of QoE is for the good of network optimization, where the consumer is OAM. The other consumers are just using the QoE report for some extra purposes. Take NWDAF for instance, it simply obtains the data from OAM for AI/ML data analysis. Therefore, for other consumers except OAM, they should follow the priorities set by OAM. Moreover, we think other consumers can participate in setting the assistance information in an implicit way. Specifically, other consumers can actually check manually the current priority set by OAM. If these consumers are not satisfied with the current priority, they can simply let OAM know, ask OAM to configure a new QoE measurement with a higher priority.

OAM is not the only consumer of QoE measurement, where other consumers can implicitly affect the assistance information set by OAM.

2.4 Threshold-based trigger for RAN visible QoE

In previous meeting RAN3-118, RAN3 agreed to introduce buffer level as a threshold-based for RVQoE reporting. Recently, RAN2 has sent a new LS cc RAN3, notifying the following:

RAN3 has agreed to introduce buffer level threshold-based triggering of RVQoE reporting in Rel-18, and RAN2 has further discussed whether triggering of RVQoE reporting based on buffer level should be handled by APP layer or AS layer. From RAN2 perspective, buffer level threshold-based triggering of RVQoE reporting handled by either APP layer or AS layer is feasible, but RAN2 prefers APP layer handling. - With APP layer triggering, APP layer provides RVQoE measurements reports to AS layer when the measured buffer level satisfies a buffer level threshold, and the AS layer reports to gNB the RVQoE measurements received from the APP layer. - With AS layer triggering, APP layer provides RVQoE measurements reports to AS layer according to the configured reporting periodicity (i.e. Rel-17 RVQoE reporting mechanism is reused), and the AS layer reports to gNB the RVQoE measurements when the received buffer level measurements satisfy a buffer level threshold. RAN2 would like to ask SA4 to confirm APP layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18.

Where RAN2 states that buffer level threshold-based triggering of RVQoE reporting handled by either APP layer or AS layer is feasible, but RAN2 prefers APP layer handling. We actually has no strong view on which layer handles the checking of threshold, we can follow what is suggested by RAN2.

Threshold evaluation for buffer level is handled in APP layer.

Then RAN3 probably need to discuss how to use the threshold. In our view, we can consider two types of threshold evaluation. The first is reporting buffer level if greater than a threshold, the second is reporting buffer level if less than a threshold.

RAN3 supports two types of threshold evaluation: a) Report buffer level if greater than a threshold b) Report buffer level if less than a threshold

- Then we also notice that, the mechanism of threshold-based trigger which reports the RVQoE report only when some threshold is met intrinsically contradicts to the mechanism of RVQoE reporting periodicity which, however, reports RVQoE report in a regular and periodic way. Therefore, we should only activate one of the two features at a same time. Also, activating the RVQoE threshold-based trigger should not impact the measurement behaviour of QoE. That is, when threshold-based trigger feature is activated, the buffer level is measured every n seconds as configured in the QoE container.

RVQoE threshold-based trigger feature and RVQoE reporting periodicity feature should not be activated at the same time.

When threshold-based trigger feature is activated, the buffer level is measured every n seconds as configured in the QoE container.

3. Proposal

In this contribution, we provide the views on the R17 left-over features, and get the following observations and proposals:

- It is suggested not to introduce event-based triggers unless great benefits have been identified.

Event-based triggers if any are only used for reporting purpose instead of measurement.

The assistance information is the priority for each QoE measurement.

OAM is not the only consumer of QoE measurement, where other consumers can implicitly affect the assistance information set by OAM.

Threshold evaluation for buffer level is handled in APP layer.

RAN3 supports two types of threshold evaluation: a) Report buffer level if greater than a threshold b) Report buffer level if less than a threshold

RVQoE threshold-based trigger feature and RVQoE reporting periodicity feature should not be activated at the same time.

When threshold-based trigger feature is activated, the buffer level is measured every n seconds as configured in the QoE container.