3GPP TSG-RAN WG3 #119bis-e								R3-231811

17th – 26th April 2023

Agenda Item:	26.2

Source:	China Telecom

Title: 	Discussion on RAN3 Impact of Timing Resiliency and URLLC Enhancements

Document for:	Discussion

Introduction

In RAN#99 meeting, the new WID on NR timing resiliency and URLLC enhancements was agreed [1]. The objectives of this work item are shown as below.

1. 5GS network timing synchronization status and reporting [RAN3, RAN2]:

a.	AMF providing clock quality reporting control information per-UE to the gNB. [RAN3]

b.	gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3]

Note 1: 	Details of the 5G clock quality information will be decided by RAN3.

c.	UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2]

d.	gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

2.	Interworking with TSN network deployed in the transport network [RAN3]:

a.	RAN impacts due to 5G System integration with TSN Transport network

Note 2: This objective has dependency on the progress of SA2 and CT4.

3. Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]:

a.	RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication.

Note 3:	Reactive RAN feedback for upstream scheduling is pending RAN2 conclusion on burst arrival time (BAT) offset derivation.

In this paper, we will discuss the RAN3 impact of timing resiliency and URLLC enhancements and provide our views.

Discussion

- 5GS Network Timing Synchronization Status and Reporting

This part aims to study how RAN and 5GC learn about 5GS network timing synchronization status and inform the UE. It is assumed that the NG-RAN can detect the networking timing synchronization degradation/improvement or timing synchronization failures. Once NG-RAN discovers the timing synchronization issues, the gNB need to send the timing synchronization status to the AMF. And then AMF will provide clock quality reporting control information per-UE to the gNB. After that gNB will send the 5G clock quality information to UE. The details of timing synchronization status are decided by SA2. From the perspective of RAN3, the timing synchronization status shall be contained in NGAP message to support NG-RAN to report the timing synchronization status to AMF and support AMF to provide clock quality reporting control information to NG-RAN.

Proposal 1: The existing NGAP message shall be enhanced to support the delivery of the timing synchronization status or clock quality reporting control information between AMF and NG-RAN.

In order to support timing resiliency and URLLC enhancements in split architecture, the CU-UP as send/receive entity of user data shall support report timing synchronization status to CU-CP. Therefore, the existing E1AP message from CU-UP to CU-CP need to be enhanced to support report timing synchronization status of CU-UP.

Proposal 2: the existing E1AP message from CU-UP to CU-CP need to be enhanced to support report timing synchronization status of CU-UP.

Similarly, the gNB-DU also need to report timing synchronization status to CU-CP. Therefore, the existing E1AP message from DU to CU need to be enhanced to support report timing synchronization status of DU.

Proposal 3: the existing E1AP message from DU to CU need to be enhanced to support report timing synchronization status of DU.

- Adapting downstream and upstream scheduling

The intention of this part is to optimize the downstream and upstream scheduling to meet the low latency requirement. As for downstream direction, the NG-RAN receives the burst arrival time adaptation indication in TSCAI. Therefore, TSCAI in RAN3 extends to support burst arrival time adaptation indication. Once NG-RAN determines that the packet delay budget of the QoS profile cannot be fulfilled, it needs to calculate the relative burst arrival time offset compared to the current burst arrival time experienced, and notifies it to the 5GC for optimization. As for upstream, the UL packets are transmitted by UE. The UE can determine the relative burst arrival time offset in reference to the current burst arrival time experienced. Once the burst arrival time offset reaches the configured threshold, the UE will send the burst arrival time offset to RAN through RRC message. And then, NG-RAN will send it to the 5GC for DL scheduling optimization. In addition, the UE can also use the NAS message to send the preferred burst arrival time offset value to the 5GC for optimization. Therefore, from the perspective of RAN3, the existing NGAP messages need to support burst arrival time adaptation indication and burst arrival time offset information.

Proposal 4: The existing NGAP messages need to support burst arrival time adaptation indication and burst arrival time offset information

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: The existing NGAP message shall be enhanced to support the delivery of the timing synchronization status or clock quality reporting control information between AMF and NG-RAN.

Proposal 2: the existing E1AP message from CU-UP to CU-CP need to be enhanced to support report timing synchronization status of CU-UP.

Proposal 3: the existing E1AP message from DU to CU need to be enhanced to support report timing synchronization status of DU.

Proposal 4: The existing NGAP messages need to support burst arrival time adaptation indication and burst arrival time offset information

Reference

- RP-230754, New WID on NR Timing Resiliency and URLLC enhancements

- TR 23.700-25 v18.1.0, Study on timing resiliency and TSC and URLLC enhancements