3GPP TSG-RAN WG3 #119bis-e																	R3-231850

Online, 17th – 26th April 2023

Agenda Item:	14.4

Source:	ZTE

Title:	(TP to TS 38.423 and 38.473): Support of SCG selective activation

Document for:	Discussion and Approval

- Introduction

At the last RAN3#119 meeting, we have achieved the following progress

Enhance XnAP and F1AP signaling to support NR Selective Activation. Introduce a new indicator to the S-NODE ADDITION REQUEST message over Xn to indicate that the request is for Selective Activation. Further check the activation/deactivation of candidate PSCells for NR Selective Activation. Keep the WA for early data forwarding. Wait for RAN2’s progress on the scenarios.

Meanwhile, RAN2 #121 meeting also has achieved the following progress

- Assume to support the following scenarios of SCG selective activation: - SN initiated intra-SN SCG selective activation - MN initiated inter-SN SCG selective activation - SN initiated inter-SN SCG selective activation - It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell. - In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case). - R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions) - For inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration). - R2 understands that A target SN may include an indication in SN Addition Request Ack for each candidate target PSCell, denoting whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

This paper further discusses how to support the selective activation feature.

- Discussion

At the last meeting, we have agreed as below.

Enhance XnAP and F1AP signaling to support NR Selective Activation.

Introduce a new indicator to the S-NODE ADDITION REQUEST message over Xn to indicate that the request is for Selective Activation

Similarly, a new indicator for selective activation is needed to be introduced to the UE CONTEXT SETUP REQUEST message in F1AP.

Proposal 1: Introduce a new indicator to the UE CONTEXT SETUP REQUEST message to indicate that the request is for selective activation in F1AP.

RAN2 has go the following assumption.

- Assume to support the following scenarios of SCG selective activation:

- SN initiated intra-SN SCG selective activation

- MN initiated inter-SN SCG selective activation

- SN initiated inter-SN SCG selective activation

To support the above scenarios, a new indicator for selective activation needs to be introduced in the S-NODE CHANGE REQUIRED message in XnAP and in the UE CONTEXT MODIFICATION REQUEST message in F1AP respectively.

In case of SN initiated inter-SN CPC, we also needs to consider whether all the target SN are Rel-18 SNs, i.e. whether it is allowed that some target SNs support selective activation while some don’t.

Proposal 2: Introduce a new indicator to the S-NODE CHANGE REQUIRED message to indicate that the request is for selective activation in XnAP.

Proposal 3: Introduce a new indicator to the UE CONTEXT MODIFICATION REQUEST message to indicate that the request is for selective activation in F1AP.

Proposal 4: RAN3 shall consider whether all the target SN are Rel-18 SNs in case of SN initiated inter-SN CPC.

Meanwhile, RAN2 also agreed below.

- For inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration).

- R2 understands that A target SN may include an indication in SN Addition Request Ack for each candidate target PSCell, denoting whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

To support the second agreement, a new indicator needs to be introduced in the S-NODE ADDITION REQUEST ACKNOWLEDGE message in XnAP and in the UE CONTEXT SETUP RESPONSE message, UE CONTEXT MODIFICATION RESPONSE message in F1AP respectively to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

As for the first agreement, the MN can send an explicit IE or an inter-node RRC container to provide the reference configuration to all candidate T-SNs. It is pending on RAN2 progress to decide which way should be selected.

Proposal 5: Introduce a new indicator in the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration in XnAP.

Proposal 6: Introduce a new indicator in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration in F1AP.

Proposal 7: It is pending on RAN2 that for inter-SN CPC, whether the MN sends an explicit IE or an inter-node RRC container to a T-SN to provide the reference configuration (in order to generate the T-SN candidate configuration).

- Conclusion

Based on the above analysis, we provide the following proposals.

Proposal 1: Introduce a new indicator to the UE CONTEXT SETUP REQUEST message to indicate that the request is for selective activation in F1AP.

Proposal 2: Introduce a new indicator to the S-NODE CHANGE REQUIRED message to indicate that the request is for selective activation in XnAP.

Proposal 3: Introduce a new indicator to the UE CONTEXT MODIFICATION REQUEST message to indicate that the request is for selective activation in F1AP.

Proposal 4: RAN3 shall consider whether all the target SN are Rel-18 SNs in case of SN initiated inter-SN CPC.

Proposal 5: Introduce a new indicator in the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration in XnAP.

Proposal 6: Introduce a new indicator in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration in F1AP.

Proposal 7: It is pending on RAN2 that for inter-SN CPC, whether the MN sends an explicit IE or an inter-node RRC container to a T-SN to provide the reference configuration (in order to generate the T-SN candidate configuration).

- Text Proposal

  - TP to TS 38.423

--------------------------------------------------Start of the change---------------------------------------------------

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

-------- skip unchanged part ----------

If the SCG Activation Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use it to configure SCG resources as specified in TS 37.340 [8], and shall, if supported, include the SCG Activation Status IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message. If the SCG Activation Request IE in the S-NODE ADDITION REQUEST message is set to "Activate SCG", the S-NG-RAN node shall, if supported, activate the SCG resources and set the SCG Activation Status IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message to "SCG activated".

If the Conditional PSCell Addition Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request concerns CPAC, as described in TS 37.340 [8]. If the Selective activation Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request concerns a SCG selective activation, as described in TS 37.340 [8]. Accordingly, the S-NG-RAN node shall, if supported, include the Conditional PSCell Addition Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the Conditional PSCell Addition Information Acknowledge is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, shall, if supported, consider the indicated PSCells are selected by the target SN as candidate PSCells for CPAC. If the Selective Activation Config Indication IE is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it as specified in TS 37.340 [8].

If the CG-CandidateList is included in the S-NG-RAN node to M-NG-RAN node Container IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it for the purpose of CPAC.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

--------------------------------------------------Next change---------------------------------------------------

### 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

#### 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

-------- skip unchanged part ----------

If the S-NODE CHANGE REQUIRED message includes the Source PSCell ID IE, the M-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

If the Conditional PSCell Change Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the requirement concerns CPAC, as described in TS 37.340 [8]. If the Selective activation Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the requirement concerns a SCG selective activation, as described in TS 37.340 [8]. The S-NG-RAN node to M-NG-RAN node Container IE within the Conditional PSCell Change Information Required IE contains at least the suggested PSCell list for each candidate target S-NG-RAN node. Accordingly, the M-NG-RAN node may include the Conditional PSCell Change Information Confirm IE in the S-NODE CHANGE CONFIRM message.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Change Information Required IE included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, forward this information to the candidate target S-NG-RAN node, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

If the Multiple Target S-NG-RAN Node List IE is included in the S-NODE CHANGE REQUIRED message, if multiple Target S-NG-RAN nodes are prepared, the M-NG-RAN node may include the Additional List of PDU Session Resource Change Confirm Info – SN Terminated IE in the S-NODE CHANGE CONFIRM message to provide different data forwarding addresses for different Target S-NG-RAN nodes.

Interaction with M-NG-RAN node initiated S-NG-RAN node Release:

If the M-NG-RAN node receives the S-NODE CHANGE REQUIRED message indicating releasing target S-NG-RAN node(s) and cancelling all prepared PSCells in the target S-NG-RAN node(s), the M-NG-RAN shall, if supported, trigger the M-NG-RAN node initiated S-NG-RAN node release procedure to the target S-NG-RAN node(s) and cancel all the prepared PSCells at the target S-NG-RAN node(s).

--------------------------------------------------Next change---------------------------------------------------

9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >Selective Activation Information Request | O |  | 9.2.3.xxx |  | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

--------------------------------------------------Next change---------------------------------------------------

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>>Selective Activation Config Indication | O |  | ENUMERATED (full config, delta config, ...) | Indicates whether the associated SCG configuration is a delta with respect to the reference SCG configuration. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

--------------------------------------------------Next change---------------------------------------------------

9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name |  | Presence | Range |  | IE type and reference |  | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  | 9.2.3.1 |  |  | YES | reject |
| M-NG-RAN node UE XnAP ID |  | M |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID |  | M |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | Allocated at the S-NG-RAN node | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |  |  |  |
| Conditional PSCell Change Information Required |  | O |  |  |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  |  | 1 |  |  |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  |  | 1 .. <maxnoofTargetSNs> |  |  |  |  | – |  |
| >>>Target S-NG-RAN node ID |  | M |  |  | Global NG-RAN Node ID 9.2.2.3 |  |  | – |  |
| >>>CPC Indicator |  | M |  |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancellation, ...) |  |  | – |  |
| >>>Maximum Number of PSCells To Prepare |  | M |  |  | INTEGER (1..8, …) |  | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability |  | O |  |  | INTEGER (1..100) |  | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container |  | M |  |  | OCTET STRING |  | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |
| >Selective Activation Information Required | O |  |  | 9.2.3.xxx |  |  |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

--------------------------------------------------Next change---------------------------------------------------

#### 9.2.3.xxx	Selective Activation Information Request

This IE indicates that the request is for SCG selective activation and may include additional assistance information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Selective Activation Request Indication | M |  | ENUMERATED (true, ...) |  |
| Selective Activation Assistance Information | O |  | FFS |  |

-----------------------------------------------------End of the change-------------------------------------------------------

  - TP to TS 38.473

--------------------------------------------------Start of the change---------------------------------------------------

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. The gNB-CU shall perform RRC Reconfiguration or RRC connection resume as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]

-------- skip unchanged part ----------

If the PC5 Link Aggregate Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the TSC Traffic Characteristics IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

If the Conditional Inter-DU Mobility Information IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider that the request concerns a conditional handover or conditional PSCell addition or conditional PSCell change for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Target gNB-DU UE F1AP ID IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall replace the existing prepared conditional handover or conditional PSCell addition or conditional PSCell change identified by the Target gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Selective activation Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns a SCG selective activation and shall, if supported, include the Selective Activation Config Indication IE in the UE CONTEXT SETUP RESPONSE message.

If the Serving NID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall combine the Serving NID IE with the Serving PLMN IE to identify the serving NPN, and may take it into account for UE context establishment.

--------------------------------------------------Next change---------------------------------------------------

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

-------- skip unchanged part ----------

If the TSC Traffic Characteristics IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-initiation", the gNB-DU shall consider that the request concerns a conditional handover or conditional PSCell addition or conditional PSCell change for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-replace", the gNB-DU shall replace the existing prepared conditional mobility identified by the gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-cancel", the gNB-DU shall consider that the gNB-CU is about to remove any reference to, and release any resources previously reserved for the candidate cells associated to the UE-associated signalling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE. If the Candidate Cells To Be Cancelled List IE is also included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that only the resources reserved for the cells identified by the included NR CGIs are about to be released by the gNB-CU.

If the Selective activation Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the modification concerns a SCG selective activation and shall, if supported, include the Selective Activation Config Indication IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Transmission Stop Indicator IE is included within the DRB to Be Modified Item IE in the UE CONTEXT MODIFICATION REQUEST message and set to “true”, the gNB-DU shall, if supported, stop the data transmission for the DRB. It is up to gNB-DU implementation when to stop the UE scheduling for that DRB.

If the SCG Indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and it is set to “released”, the gNB-DU shall, if supported, deduce that an SCG is removed.

--------------------------------------------------Next change---------------------------------------------------

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - | - |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | 9.3.1.5 | Allocated at the target gNB-DU | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| >Selective Activation Information Request | O |  | 9.3.1.xxx |  | - |  |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.151 |  | YES | ignore |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |

--------------------------------------------------Next change---------------------------------------------------

9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Selective Activation Config Indication | O |  | ENUMERATED (full config, delta config, ...) | Indicates whether the associated SCG configuration is a delta with respect to the reference SCG configuration. | YES | ignore |

--------------------------------------------------Next change---------------------------------------------------

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| >Selective Activation Information Request | O |  | 9.3.1.xxx |  | - |  |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |

--------------------------------------------------Next change---------------------------------------------------

9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| UE Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>Multicast F1-U Context Reference CU | O |  | 9.3.2.13 |  | - |  |
| Selective Activation Config Indication | O |  | ENUMERATED (full config, delta config, ...) | Indicates whether the associated SCG configuration is a delta with respect to the reference SCG configuration. | YES | ignore |

--------------------------------------------------Next change---------------------------------------------------

#### 9.3.1.xxx	Selective Activation Information Request

This IE indicates that the request is for SCG selective activation and may include additional assistance information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Selective Activation Request Indication | M |  | ENUMERATED (true, ...) | Indicates the request is for SCG selective activation. |
| Selective Activation Assistance Information | O |  | FFS |  |

-----------------------------------------------------End of the change-------------------------------------------------------