3GPP TSG RAN WG3 Meeting#120		R3-232797

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	13.3

Source:	KT Corp.

Title:	Discussion on mobility enhancements for mobile-IAB

Document for:	Discussion

# Introduction

In the last RAN3 #119bis e-meeting, following was agreed and further discussion will be made in this meeting [1]:

TAC reconfiguration: - RAN3 agreed: The IAB-DU’s TAC can have the same or a different value than the TAC of the mIAB-MT’s serving cell. Discuss how mIAB-DU’s new TAC is configured in case it is the same as that of the mIAB-MT’s serving cell and in case it is different, and how the IAB-node knows which of these two configurations are applied. Discuss how RANAC is managed. Explicit mIAB-indication/authorization in Xn HO - Discuss whether explicit mIAB-node indication and/or explicit mIAB-node-authorized indication needs to be included in the HO request for the mIAB-MT, e.g., so that the target CU can perform admission control.

In this contribution, above open issues from the last RAN3 meeting are discussed.

# Discussion

From the agreement from the previous meeting, we can assume that for the vehicle mounted with m-IAB traveling only on the dedicated route such as shuttle bus, trams, etc. is the case when m-IAB-DU’s TAC is pre-configured via OAM and has the same value of the m-IAB-MT’s TAC. We have also let the m-IAB-DU’s TAC to have the different value with m-IAB-MT’s TAC when vehicle is moving through random route without any pre-configuration depending on the traffic situation. For this case, we may assume that DU parameters are configured through CU.

Observation 1: For the vehicles moving only on the dedicated route, OAM configuration can be applied and other cases CU configuration can be applied

Proposal 1: Mechanism for both OAM-based and CU-based configuration is specified

For the case when CU-based configuration is used, we need to make sure that the target CU supports m-IAB node in advance. In this way, source CU will decide whether to send HO request to the target donor CU. For the benefit of operator, KT sees strong benefit of explicit indication of “m-IAB node” so that the target CU performs necessary admission control.

Observation 2: It is beneficial for operators to know the support of “m-IAB node” in advance to explicitly indicate “m-IAB node”

Proposal 2: Explicitly indicate “m-IAB node” and/or explicit “m-IAB-node-authorized indication” needs to be included in the HO request for the m-IAB-MT

# Conclusion

Based on the discussion in the above section, the following proposal is suggested.

Proposal 1: Mechanism for both OAM-based and CU-based configuration is specified

Proposal 2: Explicitly indicate “m-IAB node” and/or explicit “m-IAB-node-authorized indication” needs to be included in the HO request for the m-IAB-MT

# Reference

- R3-232764, Workplan for Rel-18 mobile IAB. 3GPP RAN3 #120