3GPP TSG-RAN3 Meeting #120	R3-232977

# Incheon, Korea, May 22nd – 26th 2023

Agenda item:	11.4

Source: 	Xiaomi

Title: 	Discussion on RVQoE information (TP to BL CR TS 38.473 Enhancement on NR QoE)

Document for:	Discussion & decision

Introduction

In this contribution, we’d like to discuss the following topics based on the previous discussion and agreements:

- Threshold triggers for RVQoE

- RVQoE information over F1

- Inter-RAT QoE

Discussion

Threshold triggers for RVQoE

In last RAN3 meeting, we had the following agreements on threshold-based RVQoE reporting:

If a UE is configured with periodic RVQoE reporting that automatically starts at the beginning of the application session or immediately upon reception of RVQoE configuration, it cannot be configured with a threshold-based trigger at the same time.

Discuss whether threshold-based buffer level reporting starts: i) when buffer level is greater than a threshold or ii) when buffer level is below a threshold or iii) when buffer level is between two thresholds.

RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options:

Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports)

Option 2: Periodically based on a gNB configured reporting periodicity

Option 3: A certain number of times based on gNB configured report amount

Further discuss whether to introduce TTT (time to trigger) for threshold-based triggers.

When we talk about thresholds triggered reporting, it’s easy to think of the AS layer measurement reporting, like A2 event, in which there are threshold, timeToTrigger, reportAmount, etc. but in our view, the application measurement e.g. buffer level is different from the AS measurement e.g. RSRP, before we define it, we should know how it works, how the network will use it.

According the SA4, the buffer level is defined in Annex D.4.5 in ISO/IEC 23009-1, as below. According to the definition, the buffer level is used to indicate the playout duration for which media data of all active media components is available starting from the current playout time. Firstly, we should discuss how the buffer level information will be used for the gNB. In most cases, the low buffer level may stand for bad UE experience, high buffer level stands for good UE experience.

Figure 1 screen shot in Annex D.4.5 in ISO/IEC 23009-1[1]

In our view, both bad UE experience and good experience can be useful for gNB, e.g. if UE has bad experience, the gNB can prioritize the scheduling priority for this UE, if UE has good experience, the gNB may assign resources for other UEs for a while, as gNB knows that this action will not affect the good experience UE to some extent. So, in our view, a threshold value for high buffer level and a threshold value for low buffer level can be considered, similar to A1 and A2 events for RSRP measurement.

Proposal 1, the thresholds-based RVQoE reporting can be used to indicate the good and/or bad UE experiences for NG-RAN node.

Proposal 2, the RVQoE reporting can start when the buffer level is greater than a threshold and/or when the buffer level is below a threshold, i.e. introduce two threshold values for different purposes.

Regarding whether to introduce time-to-trigger, we think this may not be needed, the reason to introduce time-to-trigger for AS measurement is that the radio signal fluctuates, but we don’t think the buffer level measurement has that fluctuation characteristic, the change of the buffer level depends on the new received segments and playout rate, if we want to define the value of the time-to-trigger for buffer level, we need to analyse how the buffer level is changed and this needs to be checked with SA4.

Observation 1, introduction of time-to-trigger for AS measurement is due to the fluctuation characteristic of the radio signal, which may not be the characteristic for buffer level in application layer.

Proposal 2, check with SA4 how the buffer level is changed if RAN3 agrees to introduce time-to-trigger for buffer level reporting.

After the threshold is met, we don’t think only one RVQoE report is enough, we prefer Option 2: Periodically based on a gNB configured reporting periodicity, the RVQoE report can stop until the threshold is not met.

Proposal 3, UE should send the RVQoE reports periodically based on a gNB configured reporting periodicity after the threshold is met, and stop send the RVQoE reports when the threshold is not met.

In addition, as we mentioned above, the application layer measurement is different from AS measurement, if the buffer level is relative lower (e.g. lower than a threshold), it may be the following reasons:

- Network issue (including sever issue)

- UE issue

- Near the end of the session (i.e. no more segments need to be buffered)

For the third reason, the lower buffer level is normal and it doesn’t mean bad UE experience, we need to discuss how to avoid RVQoE reporting in such case, and discuss whether we need to check with SA4 about our understanding.

Observation 2, the buffer level may be lower than a threshold near the end of session, which doesn’t stand for bad UE experience.

Proposal 4, RAN3 to discuss how to avoid the exception case and decide whether to check with SA4.

RVQoE information over F1

In last RAN3 meeting, we had the following agreements on DU participation:

WA: A class-2 procedure is used for DU to deactivate the RVQoE reporting over F1AP.

Further discuss the details of the procedure used for RVQoE deactivation over F1, e.g., legacy or new procedure, UE associated or non-UE associated signaling.

Whether the deactivation of RVQoE reporting over F1 is performed per RVQoE configuration or not?

Clarify whether the DU triggered deactivation of RVQoE reporting over F1 pertains only to the present application session.

Further discuss and clarify the necessity of DU participation in assembling RVQoE configuration.

UE associated or non-UE associated signaling

Regarding whether UE associated or non-UE associated signalling is used, we think UE associated signalling may be more flexible, but we need to discuss whether per UE level RVQoE deactivation is useful or needed, is it possible the scheduler only cares about only one UE’s experience. If the RVQoE is used for the scheduling optimization for all the UEs in the cell, the non-UE associated signalling should be used.

Observation 3, the scenario that needs per UE level RVQoE deactivation over F1AP is not clear.

Observation 4, if the RVQoE is used for the scheduling optimization for the cell, the non-UE associated signalling can be used.

Proposal 5, non-UE associated signalling can be used to deactivate the RVQoE information transfer over F1AP for cell level scheduling.

Proposal 6, per UE level RVQoE deactivation over F1AP needs further study.

Whether the deactivation of RVQoE reporting over F1 is performed per RVQoE configuration or not

Similar to the previous open issue, whether per RVQoE configuration deactivation is needed or not depends on how the DU (i.e. scheduler) use the RVQoE, normally, Scheduling is a holistic thing, one-sided information may not be useful, we didn’t see the necessity of per RVQoE configuration deactivation.

Observation 5, the scenario for per RVQoE configuration deactivation of RVQoE reporting over F1 is not clear.

Clarify whether the DU triggered deactivation of RVQoE reporting over F1 pertains only to the present application session.

Actually, we think the reason for DU to trigger the deactivation of RVQoE transfer over F1AP doesn’t depend on application session, it should depend on the need of DU for scheduling, e.g. when the resources are limited, the number of UEs or DRBs are quite high, the DU may need more information to balance the UE experience and resource allocation efficiency, if there’re only few UEs or enough resources for UE resources allocation, there’s no need for DU to know the RVQoE, because there will be no resource allocation issue even UE have bad experience.

Observation 6, the reason for DU to trigger the deactivation of RVQoE transfer over F1AP should depend on DU’s needs on scheduling, instead of application session.

Proposal 7, RAN3 agree the TP in Appendix.

Inter-RAT QoE

In last RAN3 meeting, we had the following agreements on Support QoE during Intra-5GC Inter-RAT::

WA: For HO from LTE/5GC to NR, there is no impacts to RAN3.

For HO from NR to LTE/5GC case, identify the impact on RAN3.

In our understanding, besides the configuration content and structure, the main difference between LTE QoE and NR QoE are below:

- LTE QMC only support streaming and MTSI, NR QoE support more service types than LTE QoE.

- LTE QMC only support one QMC per UE, NR QoE support multiple QMC per UE.

If the UE hands over from LTE to NR, and if the UE is configured LTE QMC, the LTE QMC can be continued, but how UE continue the QMC after handover should be discussed in RAN2 scope, NR may assign new RRC ID for the LTE QMC. After UE switches to NR network, UE may perform handover among NR gNBs, it may also move back to LTE network, the Ng-eNB needs to select one of the QoE configuration to continue the measurement. Of course, Ng-eNB can only select the QoE configuration for the streaming service and MTSI service, but there may be multiple QoE configurations for the same service type, how the Ng-eNB select the one QMC to continue needs to be discussed. In our view, if there is one QMC configured in LTE network, after the UE handed over from LTE to NR, an indictor can be added to that QMC, to help QMC selection in the further handover from NR to LTE.

Proposal 8, RAN3 discuss how the Ng-eNB select the one QMC among multiple QMCs to continue the measurement in LTE, when UE performs handover from LTE to NR.

Proposal 9, an indictor can be added to the QMC that configured in LTE network to help QMC selection in the further handover from NR to LTE.

Conclusion

The following are the observations and proposals:

Observation 1, introduction of time-to-trigger for AS measurement is due to the fluctuation characteristic of the radio signal, which may not be the characteristic for buffer level in application layer.

Proposal 2, check with SA4 how the buffer level is changed if RAN3 agrees to introduce time-to-trigger for buffer level reporting.

Proposal 3, UE should send the RVQoE reports periodically based on a gNB configured reporting periodicity after the threshold is met, and stop send the RVQoE reports when the threshold is not met.

Observation 2, the buffer level may be lower than a threshold near the end of session, which doesn’t stand for bad UE experience.

Proposal 4, RAN3 to discuss how to avoid the exception case and decide whether to check with SA4.

Observation 3, the scenario that needs per UE level RVQoE deactivation over F1AP is not clear.

Observation 4, if the RVQoE is used for the scheduling optimization for the cell, the non-UE associated signalling can be used.

Observation 5, the scenario for per RVQoE configuration deactivation of RVQoE reporting over F1 is not clear.

Proposal 5, non-UE associated signalling can be used to deactivate the RVQoE information transfer over F1AP for cell level scheduling.

Proposal 6, per UE level RVQoE deactivation over F1AP needs further study.

Observation 6, the reason for DU to trigger the deactivation of RVQoE transfer over F1AP should depend on DU’s needs on scheduling, instead of application session.

Proposal 7, RAN3 agree the TP in Appendix.

Proposal 8, RAN3 discuss how the Ng-eNB select the one QMC among multiple QMCs to continue the measurement in LTE, when UE performs handover from LTE to NR.

Proposal 9, an indictor can be added to the QMC that configured in LTE network to help QMC selection in the further handover from NR to LTE.

Reference

[1] ISO/IEC 23009-1, Information technology — Dynamic adaptive streaming over HTTP (DASH)

[2] R3-230389, LS/r on RAN visible QoE value (reply to 3GPP-LS8), ITU-T SG12

Aappendix

(TP to BL CR TS 38.473) Enhancement on NR QoE

Changes Start

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup |  | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update |  | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update |  | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup |  | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) |  | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) |  | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) |  | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning |  | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel |  | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination |  | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal |  | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  | BAP MAPPING CONFIGURATION FAILURE |  |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  | GNB-DU RESOURCE CONFIGURATION FAILURE |  |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  | IAB TNL ADDRESS FAILURE |  |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |  |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |  |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |  |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |  |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |  |
| Broadcast Context Setup |  | BROADCAST CONTEXT SETUP REQUEST |  | BROADCAST CONTEXT SETUP RESPONSE |  | BROADCAST CONTEXT SETUP FAILURE |  |
| Broadcast Context Release |  | BROADCAST CONTEXT RELEASE COMMAND |  | BROADCAST CONTEXT RELEASE COMPLETE |  |  |  |
| Broadcast Context Modification |  | BROADCAST CONTEXT MODIFICATION REQUEST |  | BROADCAST CONTEXT MODIFICATION RESPONSE |  | BROADCAST CONTEXT MODIFICATION FAILURE |  |
| Multicast Context Setup |  | MULTICAST CONTEXT SETUP REQUEST |  | MULTICAST CONTEXT SETUP RESPONSE |  | MULTICAST CONTEXT SETUP FAILURE |  |
| Multicast Context Release |  | MULTICAST CONTEXT RELEASE COMMAND |  | MULTICAST CONTEXT RELEASE COMPLETE |  |  |  |
| Multicast Context Modification |  | MULTICAST CONTEXT MODIFICATION REQUEST |  | MULTICAST CONTEXT MODIFICATION RESPONSE |  | MULTICAST CONTEXT MODIFICATION FAILURE |  |
| Multicast Distribution Setup |  | MULTICAST DISTRIBUTION SETUP REQUEST |  | MULTICAST DISTRIBUTION SETUP RESPONSE |  | MULTICAST DISTRIBUTION SETUP FAILURE |  |
| Multicast Distribution Release |  | MULTICAST DISTRIBUTION RELEASE COMMAND |  | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |  |  |
| PDC Measurement Initiation |  | PDC MEASUREMENT INITIATION REQUEST |  | PDC MEASUREMENT INITIATION RESPONSE |  | PDC MEASUREMENT INITIATION FAILURE |  |
| PRS Configuration Exchange |  | PRS CONFIGURATION REQUEST |  | PRS CONFIGURATION RESPONSE |  | PRS CONFIGURATION FAILURE |  |
| Measurement Preconfiguration |  | MEASUREMENT PRECONFIGURATION REQUIRED |  | MEASUREMENT PRECONFIGURATION CONFIRM |  | MEASUREMENT PRECONFIGURATION REFUSE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Multicast Group Paging |  | MULTICAST GROUP PAGING |  |
| Broadcast Context Release Request |  | BROADCAST CONTEXT RELEASE REQUEST |  |
| Multicast Context Release Request |  | MULTICAST CONTEXT RELEASE REQUEST |  |
| PDC Measurement Report |  | PDC MEASUREMENT REPORT |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| PDC Measurement Failure Indication |  | PDC MEASUREMENT FAILURE INDICATION |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| Measurement Activation |  | MEASUREMENT ACTIVATION |  |
| QoE Information Transfer |  | QOE INFORMATION TRANSFER |  |
| Positioning System Information Delivery |  | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |  |
| QoE Information Transfer Deactivation |  | QOE INFORMATION TRANSFER DEACTIVATION |  |

Next Change

### 8.16.x	QoE Information Transfer Deactivation

#### 8. 16.x.1	General

The QoE Information Transfer Deactivation procedure is initiated by the gNB-DU to indicate to deactivate the RVQoE information transfer from gNB-CU. The procedure uses non UE-associated signalling.

#### 8.16.x.2	Successful Operation

Figure 8.16.x.2-1: QoE Information Transfer Deactivation procedure, successful operation

The gNB-DU initiates the procedure by sending a QOE INFORMATION TRANSFER DEACTIVATION message to the gNB-CU, including the deactivation information of QoE information transfer in the gNB-DU. The gNB-CU should stop sending QoE information to gNB-DU.

#### 8.16.x.3	Unsuccessful Operation

Not Applicable.

#### 8.16.x.4	Abnormal Conditions

Void.

Next Change

#### 9.2.16.x	QOE INFORMATION TRANSFER DEACTIVATION

This message is sent by a gNB-DU to a gNB-CU, to indicate the deactivation of QoE information transfer.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Deactivation Indication | M |  | ENUMERATED (true, …) |  | YES | ignore |

End of Change