3GPP TSG-RAN Meeting #120	R3-233277

# Incheon, Korea, May 22-26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Security Issues for MO-SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2023-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the agreements in RAN2#112 meeting, UL data packets transmission use the legacy procedure to perform security protection. the stage 2 description for SDT in TS38.300 does not clear address whether the security protection for UL data is needed. All other related specifications, i.e., TS38.331, TS33.501, only specify the DRB security protection behaviour for legacy non-SDT mechanism. Therefore, in order to reflect the RAN2 agreements, it need to add stage 2 description on security protection for UL data packets transmission in TS38.300. For the case that the UP policy received from the last serving gNB is different from that received in Path Switch Response message, the current specification does not applicable for SDT as the received UP security policy should be verified in Path Switch Request procedure only after UE state transits to RRC_CONNECTED. It need to specify how to handling the case that the UP policy received from the last serving gNB is different from that received in Path Switch Response message. Moreover, since the UL and DL data packets may not pass the the UL data packets need to active UP encryption and/or UP integrity protection based on the stored security context provided by the last serving gNB while the UP security policy received from the last serving gNB may be tampered with. the UL and DL data packets may not pass the integrity/confidentiality verification in gNB/UE PDCP layer, to handle these packets in gNB PDCP layer after Retrieve UE Context Response message may result in packet loss. Therefore, to deliver UL and DL data packets to core network and UE which mentioned in step3-6 should not a mandatory requirement for MO-SDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add one Note on how to handling the case that the UP policy received from the last serving gNB is different from that received in Path Switch Response message in clauses 18.2 - to add one Note “whether and when to deliver UL/DL data packets to UPF/UE in Step 3-6 is up to implementation” in clauses 18.2 Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has an impact under protocol & functional point of view. The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CU-UP could not configure the correct operator defined dictionary to UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.0,18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

////////////////////////////////////////////////////////////////////////start of change ////////////////////////////////////////////////////////////////////////

18	Small Data Transmission

18.0	General

The initial PUSCH transmission during the SDT procedure includes at least the CCCH message. When using CG resources for initial SDT transmission, the UE can perform autonomous retransmission of the initial transmission if the UE does not receive confirmation from the network (dynamic UL grant or DL assignment) before a configured timer expires as specified in clause 5.4.1 of TS 38.321 [6]. After the initial PUSCH transmission, subsequent transmissions are handled differently depending on the type of resource used to initiate the SDT procedure:

-	When using CG resources, the network can schedule subsequent UL transmissions using dynamic grants or they can take place on the following CG resource occasions. The DL transmissions are scheduled using dynamic assignments. The UE can initiate subsequent UL transmission only after reception of confirmation (dynamic UL grant or DL assignment) for the initial PUSCH transmission from the network. For subsequent UL transmission, the UE cannot initiate re-transmission over a CG resource.

-	When using RACH resources, the network can schedule subsequent UL and DL transmissions using dynamic UL grants and DL assignments, respectively, after the completion of the RA procedure.

The security protection of UL SDT data packets is based on the stored security context received in the last serving gNB.

////////////////////////////////////////////////////////////////////////skip unchanged part////////////////////////////////////////////////////////////////////////

18.2	SDT with UE context relocation

The overall procedure for SDT procedure over RACH with UE context relocation is illustrated in the figure 18.2-1.

Figure 18.2-1. RA-based SDT with UE context relocation

1.	The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.	The receiving gNB identifies the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE request is for an SDT and may also provide SDT assistance information (e.g., single packet, multiple packets).

3.	The last serving gNB decides to relocate UE context and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered from the receiving gNB to the UPF.

4-6.	The receiving gNB decides to keep UE in RRC_INACTIVE state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses via the Xn-U ADDRESS INDICATION message. The receiving gNB also initiates NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF. After the Path Switch procedure, the buffered UL NAS PDU, if any, is delivered from the receiving gNB to the AMF. And then, the subsequent UL/DL SDT data and/or signalling are transferred between UE and core network via the receiving gNB.

NOTE 1: whether and when to deliver UL/DL data packets to UPF/UE in Step 3-6 is up to implementation

NOTE 2: If the UP policy received from the last serving gNB is different from that received in Path Switch Response message, the receiving gNB could either send UE back to IDLE, or move the UE to RRC_Connected by sending RRCResume and update the security configuration via RRCReconfiguration.

7.	After the SDT transmission is completed, the receiving gNB generates and sends the RRCRelease message including the suspend indication to the UE to complete the SDT procedure and continue in RRC_INACTIVE state.

NOTE 3:	In case DL non-SDT data or DL non-SDT signalling arrives, or the UE assistance information (i.e. UL non-SDT data arrival indication) is received from the UE, the receiving gNB may decide to directly send the UE to RRC_CONNECTED state by sending the RRCResume message.

8.	The receiving gNB indicates to the last serving gNB to remove the UE context by sending the XnAP UE CONTEXT RELEASE message. The XnAP UE CONTEXT RELEASE message can be sent after step 6.

////////////////////////////////////////////////////////////////////////end of change////////////////////////////////////////////////////////////////////////