# 3GPP TSG-RAN WG3 Meeting #120	R3-233280

# 22nd -  26th May 2023

# Incheon, Korea

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1187 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on system information request over F1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, CMCC, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | On-demand SI request from idle/inactive UE is on the basis of SI messages, where one SI message could include different SIB types. The mapping of SIB types to SI messages is configured in schedulingInfoList and schedulingInfoList2. For on-demand SI request from idle/inactive UE, it has been agreed in R3-185233 at RAN3#101 that the granularity should be SI message. However, In F1AP, the SI messages requested by UE is mentioned as ‘SI type’, which is ambiguous and has resulted in confusion with SIB type in RAN3 group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change SIType list into SI list. - Remove the semantic description that ‘the SI Type value of other SI starts from 2’ in F1AP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SIType list in F1AP is ambiguous and there is misunderstanding between SI list and SIB type. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.62 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/CR ...CR... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Changes Start

9.3.1.62	SI List

This IE is used by gNB-CU to provide SI list of other SI for gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SI list item IEs |  | 1.. <maxnoofSITypes> |  |  |
| >SI message | M |  | INTEGER (1..32, ...) | Indicates a certain SI message required to be broadcasted by the gNB-DU. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSITypes | Maximum no. of SI messages , the maximum value is 32. |

End of Changes