3GPP TSG-RAN WG3 Meeting #120	R3-233557

Incheon, Korea, May 22th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1010 | rev | 3 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR to 38.423) Introduction on MT-SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, Intel Corporation, Nokia, Nokia Shanghai Bell, China Telecom, Huawei, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MT_SDT-Core |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of NR Mobile Terminated – Small Data Transmission (NR MT-SDT). |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3-119 (Athens) - Implement TP R3-230913 - Captured the abbreviation of MT-SDT; - MT-SDT Information IE is introduced in the RAN PAGING message, with details FFS. RAN3-119bis (Online) - Implement TP R3-232087 - Addition of MT-SDT Data Size IE in MT-SDT Information IE with FFS RAN3-120 (Incheon) - Implement TP R3-233405 - Addition of MT-SDT Indicator IE and resolution of Editor’s Notes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 NR MT SDT is not supported in TS 38.423. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.2.5.2, 9.1.1.7, 9.2.3.xxx(new), 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR0284 TS 38.473 CR1140 TS 37.483 CR0054 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.300 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS 38.420 CR 0034 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: resubmission to RAN3#119bis-e Rev#2: Implementation of R3-232087 post RAN3#119bis-e Rev#3: Implementation of R3-233405 post RAN3#120 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

BH	Backhaul

CAG	Closed Access Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

E-RAB	E-UTRAN Radio Access Bearer

GUAMI	Globally Unique AMF Identifier

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

MT-SDT	Mobile Terminated Small Data Transmission

M-NG-RAN node	Master NG-RAN node

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe 	Proximity Services

RANAC	RAN Area Code

RedCap	Reduced Capability

RSN	Redundancy Sequence Number

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SNPN	Stand-alone Non-Public Network

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

SDT	Small Data Transmission

TAC	Tracking Area Code

TAI	Tracking Area Identity

UL	Uplink

UPF	User Plane Function

V2X	Vehicle-to-Everything

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34], and for eDRX or the UE_ID based subgrouping according to TS 38.304 [33]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards the NG-RAN node2.

When available, the NG-RAN node1 shall include the E-UTRA Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the E-UTRA Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the NR Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the NR Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the NR Paging eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.2.3.161 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.2.3.162 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |
| MT-SDT Information | O |  | 9.2.3.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.xxx	MT-SDT Information

This IE provides the assistance information for MT-SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true, …) |  |
| MT-SDT Data Size | M |  | INTEGER (1…96000, …) | Indicates the total data size for DL NAS signalling and user plane data for all SDT bearers. Unit: byte. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

>>>>>>NEXT CHANGE<<<<<<<

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

BitRate,

Cause,

CellAndCapacityAssistanceInfo-EUTRA,

CellAndCapacityAssistanceInfo-NR,

CellAssistanceInfo-EUTRA,

*** skip unchanged ***

SDTSupportRequest,

SDT-Termination-Request,

SDTPartialUEContextInfo,

SDTDataForwardingDRBList,

PEIPSassistanceInformation,

UESliceMaximumBitRateList,

PagingCause,

MDTPLMNModificationList,

F1-terminatingIAB-donorIndicator,

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

MT-SDT-Information

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

*** skip unchanged ***

id-SDTSupportRequest,

id-SDT-SRB-between-NewNode-OldNode,

id-SDT-Termination-Request,

id-SDTPartialUEContextInfo,

id-SDTDataForwardingDRBList,

id-PEIPSassistanceInformation,

id-UESliceMaximumBitRateList,

id-S-NG-RANnodeUE-Slice-MBR,

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-MT-SDT-Information,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

FROM XnAP-Constants;

*** skip unchanged ***

-- **************************************************************

--

#### -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue					CRITICALITY reject		TYPE UEIdentityIndexValue						PRESENCE mandatory}|

{ ID id-UERANPagingIdentity						CRITICALITY ignore		TYPE UERANPagingIdentity							PRESENCE mandatory}|

{ ID id-PagingDRX								CRITICALITY ignore		TYPE PagingDRX										PRESENCE mandatory}|

{ ID id-RANPagingArea							CRITICALITY reject		TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-PagingPriority							CRITICALITY ignore		TYPE PagingPriority									PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging				CRITICALITY ignore		TYPE AssistanceDataForRANPaging				PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging				CRITICALITY ignore		TYPE UERadioCapabilityForPaging				PRESENCE optional }|

{ ID id-ExtendedUEIdentityIndexValue			CRITICALITY ignore		TYPE ExtendedUEIdentityIndexValue				PRESENCE optional }|

{ ID id-EUTRAPagingeDRXInformation				CRITICALITY ignore		TYPE EUTRAPagingeDRXInformation				PRESENCE optional }|

{ ID id-UESpecificDRX							CRITICALITY ignore		TYPE UESpecificDRX									PRESENCE optional }|

{ ID id-NRPagingeDRXInformation					CRITICALITY ignore		TYPE NRPagingeDRXInformation					PRESENCE optional }|

{ ID id-NRPagingeDRXInformationforRRCINACTIVE	CRITICALITY ignore		TYPE NRPagingeDRXInformationforRRCINACTIVE	PRESENCE optional }|

{ ID id-PagingCause								CRITICALITY ignore		TYPE PagingCause										PRESENCE optional }|

{ ID id-PEIPSassistanceInformation				CRITICALITY ignore		TYPE PEIPSassistanceInformation				PRESENCE optional }|

{ ID id-MT-SDT-Information						CRITICALITY	ignore		TYPE MT-SDT-Information							PRESENCE optional },

...

}

>>>>>>NEXT CHANGE<<<<<<<

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

*** skip unchanged ***

#### -- M

*** skip unchanged ***

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extension				ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs}}	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

MT-SDT-Information ::= SEQUENCE {

mT-SDT-Indicator			MT-SDT-Indicator,

mT-SDT-DataSize 			MT-SDT-DataSize,

iE-Extensions				ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs} } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-DataSize	::= INTEGER (1..96000, ...)

MT-SDT-Indicator ::= ENUMERATED {true, ...}

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL,

...

}

MultiplexingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- N

>>>>>>NEXT CHANGE<<<<<<<

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

*** skip unchanged ***

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																		ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-MT-SDT-Information																				ProtocalIE-ID ::= xxx

>>>>>>END OF CHANGEs<<<<<<<