3GPP TSG-RAN3 Meeting #120	R3-232510

Incheon, Korea, 22th May - 26th May 2023

3GPP TSG-SA WG2 Meeting #156-e	S2-2306254

# E-Meeting, 17th – 21st April 2023

Title:	Reply LS on partially allowed/rejected S-NSSAI

Response to:	R3-230923

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	SA2

To:	RAN3, RAN2

Cc:	None

Contact Person:

Name:	alessio casati

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

SA2, thanks RAN3 for their Reply LS in R3-230923. SA2 has reviewed the LS and would like to provide the following answers to RAN3 questions:

- Whether the Target NSSAI and Partially Allowed NSSAI could be signalled simultaneously to the NG-RAN node? And in such case, what is the expected behaviour at the NG-RAN node (also considering the Allowed NSSAI)?

SA2 reply:

The Target NSSAI could be signalled simultaneously with the Partially Allowed NSSAI to the NG-RAN node. If received the NG-RAN would use the Target NSSAI to steer the UE to a cell supporting the S-NSSAIs in the Target NSSAI also taking any associated RFSP into account.

- Whether and how the Partially Allowed NSSAI would work together with the slice-based cell reselection?

SA2 reply:

It is expected that the Partially Allowed NSSAI is indeed given as input into the cell reselection algorithm. The current text in TS 23.501 reads (the red text is the new text added by SA#99 approved CRs):

“The UE NAS provides to the UE AS the NSAG Information as received from the AMF and the S-NSSAIs in the Allowed NSSAI and any Partially Allowed NSSAI as input to cell reselection, except when the UE intends to register with a new (including any S-NSSAIs rejected partially in the RA) set of S-NSSAIs with a Requested NSSAI different from the current Allowed NSSAI, and any Partially Allowed NSSAI, in which case the UE NAS provides to the UE AS layer the NSAG Information as received from the AMF and the S-NSSAIs in the Requested NSSAI, and this may trigger a cell reselection, before sending the Registration Request including the new Requested NSSAI.”

- Whether the Partially Allowed NSSAI would be associated with its own RFSP and if yes, how would this relate to existing information signalled today?  
How would the RFSP(s) related to other information, e.g. Allowed NSSAI signaled today, relate to the Partially Allowed NSSAI?

SA2 reply:

It is expected that the AMF provides the RAN with a single RFSP index, taking the Allowed and the Partially Allowed NSSAI into account. The RFSP is handled like any RFSP signalled to the NG-RAN from AMF. It logically relates to both the Allowed NSSAI and Partially allowed NSSAI as the current text in TS 23.501 reads as follows (the red text is the new text added by SA#99 approved CRs):

“In order to enable UE idle mode mobility control and priority-based reselection mechanism considering availability of Network Slices at the network and the Network Slices allowed for a UE, an RFSP is derived as described in clause 5.3.4.3, considering also the Allowed NSSAI, and any Partially Allowed NSSAI for the UE”.

- How does the Partially Allowed NSSAI work with respect to UE-Slice-MBR?

SA2 reply:

SA2 expects to have UE-Slice-MBR information stored in the UE context in the RAN also for the S-NSSAIs of the Partially Allowed NSSAI. UE-Slice-MBR is applied for applicable S-NSSAIs when there are active PDU sessions for the S-NSSAI and it works in the same way as for S-NSSAIs in the Allowed NSSAI.

2. Actions:

To RAN2 and RAN3 group.

ACTION: 	SA2 kindly asks RAN3 and RAN2 to take the above into account and provide SA2 with feedback as soon as possible.

3. Date of Next SA2 Meetings:

Next SA2 Meetings dates