3GPP TSG-RAN WG3 Meeting #121bis	R3-235090

Xiamen, China, 09-13 Oct, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0260 | rev | 13 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR to 38.401) for L1L2Mob |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is to introduce the L1/L2 inter-cell mobility function into the stage 2 specfication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #117bis: The message flow for intra-DU L1/L2 inter-cell mobility procedure is introduced. #118: Added the inter-DU LTM procedure in TP R3-226913. #119: Added the TPs in R3-231026 and R3-231027. #119b-e: Added the TP in R3-232090. #120: Added the TPs in R3-233522, R3-233495. #121 Added the TPs in R3-234758, R3-234757. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L1/L2 inter-cell mobility is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.2.1.X, 8.2.1.Y, 8.2.1.Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.473 CR 1037 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1, update according to comments in #117bis-e meeting. Rev#2, further update according to comments in #117bis-e meeting. Rev#3, further update according to comments in #117bis-e meeting. Rev#4, resubmission to #118 meeting. Rev#5, R3-226930, merge the TP in R3-226913 from #118 meeting. Rev#6, resubmission to #119 meeting. Rev#7, R3-231048, merge TPs in #119 meeting. Rev#8, R3-231137, resubmit to #119b-e based on 17.4.0. Rev#9, R3-232531, merge TPs in #119b-e meeting. Rev#10, R3-233548, merge TPs in #120 meeting. Rev#11, R3-233743, resubmit to #121 meeting based on v17.5.0 spec. Rev#12, R3-234800, merge TPs in #121 meeting. Rev#13, R3-235090, resubmit to #121bis meeting based on v17.6.0 spec. |  |  |  |  |  |  |  |  |

--------------------------------------------------------- the start of changes -------------------------------------------------------------

3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

BH	Backhaul

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

EM	Element Manager

EN-DC	E-UTRA-NR Dual Connectivity

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

FTEID	Fully Qualified TEID

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

L2					Layer-2

LTM				L1/L2 Triggered Mobility

MBS	Multicast Broadcast Service

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MN	Master Node

MgNB	Master gNB

MRB	MBS Radio Bearer

MRDC	Multi-Radio Dual Connectivity

NAS	Non-Access Stratum

NID	Network identifier

NPN	Non-Public Network

NSA	Non Standalone

OAM	Operation, Administration and Maintenance

PNI-NPN	Public Network Integrated Non-Public Network

PTP	Point to Point

PTM	Point to Multipoint

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS	Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SA	Standalone

SAP	Service Access Point

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SgNB	Secondary gNB

SM	Session Management

SMF	Session Management Function

SN	Secondary Node

SNPN	Stand-alone Non-Public Network

SRAP	Sidelink Relay Adaptation Protocol

TCE	Trace Collection Entity

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

U2N	UE-to-Network

---------------------------------------------------------next change -------------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU LTM

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID. The gNB-CU requests PRACH resources from the gNB-DU.

- If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations  (e.g., TCI state configuration and RACH configuration) and the RS configuration (FFS) for the accepted target candidate cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

FFS: whether it is DL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION REQUEST message.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS: whether it is UL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION RESPONSE message.

- Early synchronization is performed as specified in TS 38.300 [x].

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the selected beam (FFS).

- FFS: How the target gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The gNB-CU may send the UE CONTEXT MODIFICATION message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU LTM

1. The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2. The gNB-CU determines to initiate LTM configuration.

3. The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID. The gNB-CU indicates the source gNB-DU ID, and requests PRACH resources from the Candidate gNB-DU.

4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration (e.g., TCI state configuration and RACH configuration) and the RS configuration (FFS) for the accepted target candidate cell.

5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the collected RS configuration, TCI state configuration and RACH configuration for the accepted target candidate cell(s) in other gNB-DUs.

6. The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which may include the RS configuration of the source cell (FFS), prepared candidate cells (FFS) and the generated CSI resource configuration (FFS).

7. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing  the cell ID(s) of the prepared candidate cell(s) and associated RS configuration for each candidate cell in other candidate gNB-DU(s).

Note: The candidate cell may be the same cell as source cell.

8. The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated CSI resource configuration (FFS).

9. The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10. The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11. The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12. The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. Early synchronization is performed as specified in TS 38.300 [x].

14. The candidate gNB-DU sends the TA and the associated CFRA resource information (FFS) to the source gNB-DU via the gNB-CU.

FFS on the message name.

15. The UE sends the lower layer measurement result to the source gNB-DU.

16. The source gNB-DU decides to execute LTM to a candidate target cell.

FFS: Notifying the LTM triggering decision to the other nodes as well.

17. The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

18. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the selected beam information.

19. The gNB-CU sends the target cell ID and the selected beam information to the target gNB-DU.

FFS on the message to be used and details of the selected beam information.

20. FFS:  how the target gNB-DU detects the UE access.

21. The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

22. The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

23. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

---------------------------------------------------------next change -------------------------------------------------------------

#### 8.2.1.Z	LTM with gNB-CU-UP change

Figure 8.2.1.z-1 shows the procedure used for LTM with the change of gNB-CU-UP within a gNB.

Figure 8.2.1.z LTM with the change of gNB-CU-UP

0. The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1. The gNB-CU-CP decides to initiate LTM configuration.

2. The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4 - 5. F1 UE context setup procedure is performed to setup one or more bearers in the gNB-DU.

6 - 7.The gNB-CU-CP sends the RRC Reconfiguration message to the UE. Details are FFS.

8.  The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a candidate target cell.

9. WA: The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

10-11. The gNB-CU-CP performs the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers.

12-13.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UP/DL status to the target gNB-CU-UP.

14.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

15.  The target gNB-DU detects the UE in the target cell.

16. The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

17 - 19. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

20-21.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-DU.

--------------------------------------------------------- the end of changes -------------------------------------------------------------