3GPP TSG-RAN WG3 Meeting #121bis	          R3-235129

Xiamen, China, Oct 9 - Oct 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0148 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.425) Support for NR XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Nokia, Nokia Shanghai Bell, Ericsson, ZTE, Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the support for NR XR features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3#121: - Add the definitions and abbreviations for NR XR features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NR XR features will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.483 CR 0078 TS 38.413 CR 1025 TS 38.415 CR 0037 TS 38.423 CR 1091 TS 38.473 CR 1219 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-0: Capture the agreed changes from R3-233657 in RAN3#121. Rev:1: Resubmission to RAN3 #121bis. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 3	Definitions and abbreviations

# [Editor’s Note]: The XR User Plane structure is not decided and subject to change.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Corresponding node: a node interacting with a node hosting PDCP for flow control. In an IAB network, this is the IAB-DU serving the UE for the corresponding data radio bearer.

eNB-CP: as defined in TS 36.401 [9].

eNB-UP: as defined in TS 36.401 [9].

Master node: as defined in TS 37.340 [3].

Secondary node: as defined in TS 37.340 [3].

Data Burst: as defined in TS 23.501 [8].

PDU Set: as defined in TS 23.501 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

MBS	Multicast/Broadcast Service

MR-DC	Multi-RAT Dual Connectivity

MRB	MBS Radio Bearer

PTM	Point To Multipoint

EPDU	End PDU of the PDU Set

EDB	End of Data Burst

PSI	PDU Set Importance

PSSN	PDU Set Sequence Number

PSN	PDU Sequence Number within a PDU Set

PSSize	PDU Set Size

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>