3GPP TSG-RAN WG3 #121-bis	R3-235137

Xiamen, China, 9th – 13th October 2023

Source:	CATT

Title:	(TP to 38.300) Adding the description of UE trajectory feedback

Agenda Item:	12.2.1

Document for:	Discussion and decision

- Introduction

This document proposes adding the description of UE trajectory feedback.

- Annex: TP for TS 38.300 on R3-235137

******************************************Start of 2nd Change****************************************

X.X	AI/ML for NG-RAN

X.X.1	General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2	Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

Signalling procedures used for the exchange of AI/ML related information are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

The predicted Resource Status Information as well as the UE performance feedback can be configured to be reported once or periodically. This information can be used for all use cases. The reporting is configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The cell-based UE trajectory prediction, which can be used e.g. for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Request message to provide information for e.g. subsequent mobility decisions. The cell-based UE trajectory feedback, which can be used e.g. for training UE-trajectory prediction AI/ML models or for monitoring their performance, is configured via the Handover Request message by including the measurement ID pair referring to the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The metric of Energy Cost (EC), which can be used e.g. for the Network Energy Saving use case, is introduced and can be exchanged with the neighbouring NG-RAN nodes upon request.

******************************************End of Change********************************************