3GPP TSG-RAN WG3 Meeting #121bis	R3-235188

Xiamen, China, 09-13 Oct, 2023

Title: 	(TP to MT-SDT TS 38.423 BL CR) Correction on the MT-SDT Data Size

Source: 	Huawei, Nokia, Nokia Shanghai Bell, China Telecom, ZTE, Qualcomm Incorporated, China Unicom

Agenda item:	31.1

Document Type:	other

# 1. Introduction

At RAN3#121 meeting, RAN3 agreed to add below sentence in the MT-SDT information of E1 message to clarify the user data size is SDAP SDU size:

Agree to add “Corresponding to the SDAP SDU size of the received DL data” in Semantics Description of 9.3.1.xxx MT-SDT Information.

From our view, as Data Size is also transmitted over XnAP for MT-SDT, we also need to add the similar clarification. For E1AP, only user plane data is considered, but over XnAP, we need to consider both DL NAS signalling and user plane data.

For the user plane data, SDAP SDU size applies, but considering that the DL NAS signalling is not delivered to SDAP, the data size of DL NAS signalling should rather correspond to the PDCP SDU size instead.

Proposal: Clarify in XnAP, the MT-SDT Data Size IE in the MT-SDT Information IE corresponds to the PDCP SDU size of the DL NAS signalling and the SDAP SDU size of the received DL user plane data.

# 2. TP to TS 38.423 BL CR

================================== Start of change ==================================

#### 9.2.3.xxx	MT-SDT Information

This IE provides the assistance information for MT-SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true, …) |  |
| MT-SDT Data Size | M |  | INTEGER (1…96000, …) | Indicates the total data size for DL NAS signalling and user plane data for all SDT bearers. Unit: byte. Corresponds to the PDCP SDU size for the DL NAS signalling and the SDAP SDU size for the received DL user plane data. |

================================== End of change ==================================