3GPP TSG-RAN WG3 #121bis	R3-235208

Xiamen, China, 9 – 13 Oct 2023

Title: 	(TP for BLCR TS 38.473) Discussion on MBS reception by inactive state UE

Source: 	Samsung

Agenda item:	15.3

Document Type:	Discussion

1. Introduction

According to the approved Rel-18 NR MBS enhancement WID [1], the following objectives have impact to RAN3.

This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include: - Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3] - Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3] Note: collaboration with SA2 is expected in due course for the above objectives.

2. Discussion

We only list the open issues on RRC inactive reception as showed in below:

- FFS on the “inactive reception” mode decision,

- Which option should be adopted to provide the PTM configuration from the gNB-DU to enable UEs being configured with the PTM configuration before moving the UE to RRC_INACTIVE?

- Whether an indication should be introduced in NGAP Group Paging message to enable gNB to be aware of the cause of group paging?

- Other open issues

2.1 Inactive reception mode decision

The base station determines which users are allowed to enter RRC_INACTIVE mode. If the UE currently only has MBS services without unicast services and no assistance information for this UE is received from the CN, the base station can decide to put this UE into RRC_INACTIVE mode.

The base station can decide to further narrow down the range of UE entering RRC_INACTIVE mode. For example, if it feels unnecessary to have all UEs fulfilling the above mentioned two conditions entering RRC_INACTIVE mode, the base station can make its own decision. The base station can make decisions based on the information currently available on it. E.g. UE measurement report. Very dynamic quality information, which should not be very helpful because UE may be constantly moving, from the area currently with good quality moving to areas with poor quality. Or vice versa.

As for the decision of the base station, whether it is per cell or per DU, this is an implementation issue. Anyway, MCCH is configured per cell, at least this decision is per cell. Whether it is per DU has no impact on the protocol.

- It is un-necessary for the gNB-DU to report more timely information to the gNB-CU for the “inactive reception” mode decision. The decision is at least per cell.

RAN2 and RAN3 had agreed if there is currently no data transmission, the base station may decide to move all the UE into RRC_INACTIVE mode. When the data coming, the base station sends an enhanced paging to the UEs. The UE in RRC_INACTIVE mode can stay in RRC_INACTIVE mode to receive the MBS multicast data. Some special UE should be move into RRC_CONNECTED mode. But the assistance information is sent from the CN to the base stations and stored in the UE context. If the base station found the special UE is not moved into RRC_CONNECTED mode, the base station should send UE dedicated paging.

So the most efficient way is when the gNB moves UE into RRC_INACTIVE state, the gNB includes the UE assistance information in the RRC Connection Release message. The UE stores the information. When receiving the group paging with the new indication, the UE still need to move into RRC CONNECTED mode.

- The gNB includes the UE assistance information in the RRC RELEASE REQUEST message. Based on the UE assistance information, the UE can move into RRC_CONNECTED for MBS reception receiving the per session level indicator in Uu group paging message.

2.2 How to provide latest PTM configuration from DU to CU

When the UE moves into the RRC_INACTIVE state, the MRB configuration can be included in the RRC RELEASE REQUREST message. That means the MRB configurations need to be received by the gNB-CU before sending out RRC Release message. There are three options for the gNB-CU to get the MRB configurations:

Option 1: CU retrieves the PTM configuration from DU via CU initiated Multicast context modification procedure.

Option 2: During active MBS multicast sessions, the DU always ensures that the CU is provided with the latest PTM configuration via a new DU initiated Multicast context modification procedure.

Option 3: CU retrieves the PTM configuration from DU via CU initiated UE context modification procedure.

For the procedure of RRC connected to RRC inactive, the gNB-CU decides UE state and sends the UE CONTEXT RELEASE COMMAND to the UE. The MRB configuration is included in the UE CONTEXT RELEASE COMMAND. Therefore the gNB-CU needs to receive the latest PTM configuration before sending out this message.

For option 3, considering there is special PTM configuration for RRC_CONNECTED UE, the MRB received in UE dedicate message can not be re-used for RRC_INACTIVE UEs. It is better to include MRB configuration in the MC session specific message, i.e. Option 2 o option 1. We believe that the MRB configuration has not changed much, and now MRB configuration can be transmitted in MCCH. The UE has two ways to get the MRB configuration. Therefore, we think that option 1 is sufficient and it is not worth defining a new message.

- Include a DU to CU RRC information IE into MC CONTEXT SETUP/MODIFICATION RESPONSE message for latest PTM configuration transmission.

2.3 Group paging

For the group paging in the NG interface, some companies assume if the trigger for grouping paging is session activation, the UE can keep in RRC_INACTIVE. If the trigger is for session release, the UE shall move into RRC_CONNECTED. Therefore a new indication is needed in NGAP, based on this new indicator, the NG-RAN sends the enhanced group paging in Uu.

According to SA2 TS, for the active MBS session, to release radio resources as early as possible, the MB-SMF may trigger Multicast Session Deactivation towards the NG-RAN prior to or in parallel with triggering MBS Session Release to the SMF. Also SA2 said in LS in R3-233728 “SA2 would also like to emphasize that the existing procedures in TS 23.247 assume that the UEs involved in an active MBS Session are not moved to RRC_IDLE state unless failure happens, and there is no CN procedures to trigger a paging of IDLE UEs for an active MBS session”. i.e. the group paging only happens when there is no MBS context in the NG-RAN.

According to RAN3 previous agreements, the NG-RAN decides if moving the UE into RRC Inactive mode after the MBS context is established in the NG-RAN, i.e.

The gNB decides whether a UE is configured to receive multicast data in RRC_INACTIVE. The gNB may take at least the following information into account based: 5QI, PER, ARP, and expected UE Activity Behaviour, information locally available at the gNB and other.

When the NG-RAN receives a group paging without saving the MBS context, the behaviour of the NG-RAN is to forward the group paging in the Uu. Whether the group paging is for session activation or for session release, the NG-RAN behaviour is same. At this stage, the NG-RAN can not active RRC_INACTIVE reception without receiving MBS context information. Therefore, we think this is no need to include a new indicator in the NGAP group paging message.

- No need to include the session activation indication or new indicator in the NG group paging message.

2.4 Other open issues

Whether/how to exchange ongoing multicast service between neighbour nodes

We discussed whether to exchange ongoing broadcast service in release 17 for BC and no agreement. According to RAN2 agreement, the multicast MCCH is similar as the broadcast MCCH. So the similar question is also applicable for multicast service.

The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline.

To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline.

Taking the broadcast MCCH structure as the baseline, there are two lists included

- mbs-NeighbourCellList: List of neighbour cells providing MBS broadcast services via broadcast MRB.

- mtch-NeighbourCell-r17: Indicates neighbour cells which provide this service on MTCH, it is a bitmap for mbs-NeighbourCellList and it is per TMGI.

These two lists provide assistance information for the UE, but the UE can not determine if the service is provided in a neighbouring cell based on the presence or absence of these two lists. In most cases, the MBS neighbouring cell list generated from MBS Service Area information are accurate, unless the neighbour cell rejects the session, then the list will contain inaccurate information. Even it is useful to exchange the ongoing multicast service between nodes, but we think the benefit is insufficient to compensate the overhead introduced by exchanging signalling.

- Not to exchange ongoing multicast services between neighbour nodes.

Mobility in RAN areas

The first UE in a new NG-RAN node is always important, determining whether the new base station can receive MBS data. According to legacy Rel-17 behaviour, the last serving NG-RAN may decide not to relocate the UE context and to keep the UE in RRC INACTIVE. In this way, if the UE is the first MBS user, there will be problem. There are two solutions:

- The last serving NG-RAN shall always transfer the UE context to the new NG-RAN if the UE context has the MBS information.

- The new NG-RAN node notify the last serving NG-RAN node the UE is the first MBS user, then the last serving  NG-RAN node shall forward UE context to the new NG-RAN node. For other UEs, the old NG-RAN just behaviour as normal.

- The old NG-RAN shall transfer the UE context to the new NG-RAN if the Retrieve UE context procedure is triggered for the first MBS user in the new NG-RAN.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- It is un-necessary for the gNB-DU to report more timely information to the gNB-CU for the “inactive reception” mode decision. The decision is at least per cell.

- The gNB includes the UE assistance information in the RRC RELEASE REQUEST message. Based on the UE assistance information, the UE can move into RRC_CONNECTED for MBS reception receiving the per session level indicator in Uu group paging message.

- Include a DU to CU RRC information IE into MC CONTEXT SETUP/MODIFICATION RESPONSE message for latest PTM configuration transmission.

- No need to include the session activation indication or new indicator in the NG group paging message.

- Not to exchange ongoing multicast services between neighbour nodes.

- The old NG-RAN shall transfer the UE context to the new NG-RAN if the Retrieve UE context procedure is triggered for the first MBS user in the new NG-RAN.

- Text Proposal to TS 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

9.2.14.3	MULTICAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | Reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Multicast DU to CU RRC Information | O |  | FFS |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.14.8	MULTICAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Multicast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Multicast DU to CU RRC Information | O |  | FFS |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>