3GPP TSG-RAN WG3 #121bis								R3-235213

Xiamen, China, 9th – 13th Oct 2023

Agenda Item:	21.2

Source:	China Telecom

Title: 	Support of MT-SDT in NR RedCap Enhancement

Document for:	Discussion

Introduction

In this meeting, RAN3 has received the reply LS [1] from SA2 on “INACTIVE eDRX above 10.24sec and SDT”. SA2 believes it is technically feasible for UPF/SMF to provide the DL data size of the first downlink packet to NG-RAN via AMF to support MT-SDT and eDRX. In addition, the signalled 5QI provided in DL DATA NOTIFICATION message is the same as the one stored in RAN UE context. This paper will discuss the remaining issues on INACTIVE eDRX above 10.24sec with MT-SDT.

Discussion

SDT information signalled to CN

Per the mechanism as defined in clause 4.8.1.1a of TS 23.502, the NG-RAN does not provide any SDT information in MT Communication Handling Request message to AMF. Without this information, AMF is not clear whether MT-SDT is configured and which QoS flows are SDT type. On SDT information provided to AMF, there are two alternatives:

- Alt 1: SDT type QoS flow List. AMF provides buffered DL volume for SDT QoS flows and NAS signalling in DL DATA NOTIFICATION message.

- Alt2: One MT-SDT indicator. Since AMF has no idea of SDT type QoS flow List, it needs to provide buffered DL data volume per QoS flow and size for NAS signalling in DL DATA NOTIFICATION message.

In principle, both alternatives are workable. In our understanding, Alt1 is slightly simpler than Alt2 as there is no need to calculate the data volume per QoS flow in case of non-SDT data arrived in SMF/UPF. In addition, NG-RAN determines whether to initiate MT-SDT based on total volume of SDT data rather than data volume per QoS flow. Therefore, it is proposed to introduce SDT type QoS flow List IE in MT Communication Handling Request Message.

Proposal 1: to introduce SDT type QoS flow List IE in MT Communication Handling Request.

In previous meetings, RAN3 had discussed the need to signal the DL data size in the DL DATA NOTIFICATION message to help NG-RAN making decision. Furthermore, SA2 has agreed to provide the DL data size of the first downlink packet to NG-RAN via AMF. However, SA2 had not reached consensus whether more accurate data size is needed. In Alt1, AMF only needs to provide DL data volume in case of all QoS flows is SDT type. Upon one of QoS flows buffered in CN is non-SDT type, AMF does not include DL data size in DL DATA NOTIFICATION message.

Proposal 2: to include total DL data volume in DL DATA NOTIFICATION message in case of all QoS flows buffered in CN are SDT type.

Proposal 3: AMF does not include DL data size in DL DATA NOTIFICATION message in case one of QoS flows buffered in CN is non-SDT type.

Another issue is the range of MT-SDT Data Size which Indicates the total data size for DL NAS signalling and all SDT data packets. This issue was discussed in MT-SDT WI, and the range of MT-SDT data size over Xn is [0, 96000byte]. To align with the range and definition of MT-SDT data size defined in TS38.423, the range of MT-SDT Data Size specified in NGAP is [0, 96000byte].

Proposal 4: the range of MT-SDT Data Size, indicates the total data size for DL NAS signalling and all SDT data packets, in NGAP is [0, 96000]. The unit is byte.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: to introduce SDT type QoS flow List IE in MT Communication Handling Request.

Proposal 2: to include total DL data volume in DL DATA NOTIFICATION message in case of all QoS flows buffered in CN are SDT type.

Proposal 3: AMF does not include DL data size in DL DATA NOTIFICATION message in case one of QoS flows buffered in CN is non-SDT type.

Proposal 4: the range of MT-SDT Data Size, indicates the total data size for DL NAS signalling and all SDT data packets, in NGAP is [0, 96000]. The unit is byte.

Reference

- S2-2309757/R3-235020,Reply LS on INACTIVE eDRX above 10.24sec and SDT,Source: SA2,To: RAN3, CT4,cc: RAN2